- Table 1  Description of the data.

| Data type | Description | Source |
| --- | --- | --- |
| Topography | ASTER Digital Elevation Model (DEM) Products (30 m Resolution), Dataset of grade length with 30 m resolution on the Loess Plateau (2014) | https://earthdata.nasa.gov/ http://loess.geodata.cn |
| Satellite image | Landsat - 5 TM (1987.8.28, 1997.7.22, 2007.9.20 ), Landsat - 8 OLI (2017.6.27) (path: 128; row: 35) | https://glovis.usgs.gov/ |
| Climate | Monthly rainfall dataset of the Loess Plateau for 1km resolution (1978.1-2014.12) | http://loess.geodata.cn |
| Soil | Soil map of Qingyang in Gansu Province at 1:1380000 scale. | Soil inventory office in Qingyang, 1989 |
| Engineering measures | Silt dams data of 2017 in DZY | Soil and Water Conservation Monitoring Center (http://www.cnscm.org/), Google Earth and field visits |
| Statistics | Socio-economic data set of Loess Plateau (1980-2010) | http://loess.geodata.cn |

- Table 2  The mechanical composition and the soil erodibility factor K of different soil types of DZY

| Soil types | Sa (%) | Si (%) | Cl (%) | C (%) | K |
| --- | --- | --- | --- | --- | --- |
| Alluvial soil | 16.3 | 58.5 | 25.2 | 0.8 | 0.389 |
| Red clay | 13.2 | 50.3 | 36.5 | 0.85 | 0.366 |
| Dehumidified soil | 12.6 | 58.4 | 29 | 1.05 | 0.373 |
| Loessal soil | 11.6 | 62.9 | 25.5 | 0.77 | 0.407 |
| Dark loessial soil | 15 | 60 | 25 | 1.06 | 0.374 |

- Table 3  Silt dams data of sub-watersheds in DZY (N, S, Sd, Q and E are the number, shape area, silt dam quantity, silt dam control area and engineering measure factor of the sub-watersheds, respectively).

| N | S (ha) | Sd (ha) | Q | E | N | S (ha) | Sd (ha) | Q | E | N | S (ha) | Sd (ha) | Q | E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 46805 | 7366 | 10 | 0.843 | 12 | 8996 | 5125 | 13 | 0.430 | 23 | 3349 | 0 | 0 | 1.000 |
| 2 | 38893 | 22903 | 13 | 0.411 | 13 | 7405 | 0 | 0 | 1.000 | 24 | 3090 | 0 | 0 | 1.000 |
| 3 | 19586 | 2802 | 7 | 0.857 | 14 | 7363 | 295 | 1 | 0.960 | 25 | 2977 | 0 | 0 | 1.000 |
| 4 | 15043 | 0 | 0 | 1.000 | 15 | 7050 | 315 | 1 | 0.955 | 26 | 2576 | 0 | 0 | 1.000 |
| 5 | 14345 | 5023 | 11 | 0.650 | 16 | 6016 | 5180 | 15 | 0.139 | 27 | 1595 | 758 | 2 | 0.525 |
| 6 | 12462 | 850 | 5 | 0.932 | 17 | 5829 | 0 | 0 | 1.000 | 28 | 1240 | 1238 | 1 | 0.001 |
| 7 | 11372 | 581 | 1 | 0.949 | 18 | 4872 | 0 | 0 | 1.000 | 29 | 1189 | 0 | 0 | 1.000 |
| 8 | 11046 | 0 | 0 | 1.000 | 19 | 4816 | 0 | 0 | 1.000 | 30 | 1068 | 0 | 0 | 1.000 |
| 9 | 11026 | 733 | 1 | 0.934 | 20 | 4641 | 0 | 0 | 1.000 | 31 | 460 | 0 | 0 | 1.000 |
| 10 | 9174 | 2809 | 4 | 0.694 | 21 | 3997 | 2403 | 2 | 0.399 | 32 | 381 | 0 | 0 | 1.000 |
| 11 | 9077 | 0 | 0 | 1.000 | 22 | 3594 | 3544 | 6 | 0.014 |  |  |  |  |  |
| Total |  |  |  |  |  |  |  |  |  | / | 281333 | 61925 | 93 | / |

- Table 4  Grading standards of soil erosion intensity in DZY

| Grade of soil erosion intensity | Soil erosion modulus(t·ha-1·yr -1) |
| --- | --- |
| No erosion | 0 |
| Slight erosion | 0~10 |
| Mild erosion | 10~25 |
| Moderate erosion | 25~50 |
| Intensity erosion | 50~80 |
| Extreme erosion | 80~150 |
| Severe erosion | >150 |

- Table 5  Land use transition matrix in DZY during 1987-2017 (km2) (1-6 represent the six types of land use: Farmland, Forest, Grassland, Water, Urban and Others, respectively)

|  | 1987-1997 |  |  |  |  | 1997-2007 |  |  |  |  | 2007-2017 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Land use types | Farmland | Forest | Grassland | Water | Urban | Farmland | Forest | Grassland | Water | Urban | Farmland | Forest | Grassland | Water | Urban |
| Farmland | — | 77.39 | 378.6 | 4.3 | 2.79 | — | 11.25 | 270.3 | 1.19 | 14.62 | — | 14.19 | 167.89 | 1.36 | 53.46 |
| Forest | 9.13 | — | 115.98 | 0.17 | 0.13 | 66.22 | — | 155.4 | 0.82 | 1.29 | 15.23 | — | 56.34 | 0.47 | 1.68 |
| Grassland | 206.23 | 135.1 | — | 0.97 | 1.18 | 275.19 | 109.1 | — | 3.36 | 9.32 | 263.56 | 208.6 | — | 1.83 | 19.22 |
| Water | 0.97 | 0.08 | 0.69 | — | 0.03 | 1.76 | 0.1 | 1.11 | — | 0.07 | 1.67 | 0.42 | 3.33 | — | 1.32 |
| Urban | 0.11 | 0.1 | 2.35 | 0 | — | 0.27 | 0.07 | 0.5 | 0.07 | — | 5.33 | 0.1 | 1.92 | 0.1 | — |