Relationship between land use and soil erosion in a typical area in the tableland and gully region of the Loess Plateau, China

- Jianxiang Zhanga,b, , Naiang Wanga, Duoyong Zhangc, Xianbao Sua

a College of Earth and Environment Science, Lanzhou University, Lanzhou, 730000, Gansu, China

b Provincial Key Laboratory of Loess Engineering Properties and Applications, Key Disciplines of Civil Engineering, College of Civil Engineering , Longdong University, Qingyang,745000, Gansu, China

c College of Ethnology, North Minzu University, Yinchuan, 750021, Ningxia, China

Correspondence

- N. A. Wang, College of Earth and Environment Science, Lanzhou University, Lanzhou, 730000, Gansu, China

Email: wangna@lzu.edu.cn

Funding information

National Natural Science Foundation, China; National Key Research and Development Program, China

Abstract

Soil erosion is an important environmental problem that threatens human ecological security and socioeconomic development. Unreasonable land use is one of the main causes of soil erosion. By collecting the topography, climate, soil, vegetation and satellite image data of Dongzhiyuan (DZY) in the tableland and gully region (TGR) of the Loess Plateau (LP), the Revised Universal Soil Loss Equation (RUSLE) and the Chinese Soil Loss Equation (CSLE) models were used to simulate the soil erosion intensity in different periods (1987, 1997, 2007 and 2017). We found that under high rainfall erosivity factor (R), the soil erosion usually be overestimated in the RUSLE model. However, the result has been changed when we use the annual average R.Besides, the model of CSLE based on the RUSLE takes engineering measure factor (E) into account, making the results more reliable. Also, land use change will affect soil erosion. The conversion of urban to farmland and grassland will increase soil erosion, and vice versa. The mutual transfer between farmland and grassland, forest and grassland will cause soil erosion to change in different directions, and the greater of land use change, the more serious soil erosion will be. Nevertheless, we shouldn’t increase the urban land by mountain excavation and gully filling blindly. Otherwise, geological disasters such as landslides and surface subsidence may occur in the near future. This work may enhance our understanding of soil erosion mechanisms in DZY, and provide references for land use planning and soil and water conservation in China's Loess Plateau.

- KEYWORDS

- Land use; Soil erosion; RUSLE; CSLE; Loess Plateau (LP); Dongzhiyuan (DZY)

- 1 | Introduction

Soil erosion is a major environmental problem (Mabit et al., 2014), which has caused more than 80% of global land degradation (Oldeman, 1994; Zhang et al., 2019). It is divided into natural erosion and artificial accelerated erosion. The artificial accelerated erosion refers to the rapid development of erosion based on natural erosion and affected by human activities (Hudson, 1995; Chen, 2019). It is an important environmental problem in the world today and also the main object for human to control erosion (Chen et al., 1988; Alkharabsheh et al., 2013). Many reports show that land use change is an important factor for soil erosion (Fu, 1999), and the unreasonable land use will accelerate soil erosion(Gao et al., 2013; Thomas et al., 2017; Feng et al., 2019). Therefore, it is particularly important to explore the relationship between land use and soil erosion.

The Dongzhiyuan (DZY), a typical area in the tableland and gully region (TGR) of the Loess Plateau (LP) in China, has the largest remaining loess tableland in China and the thickest loess deposits with the two major geomorphic units of loess tableland and loess gully (Figure 1) (Wang et al., 2015). According to the statistics, 373 first-level ditches of five surrounding rivers have been extending into the hinterland of the DZY, which divided the face of DZY into pieces (Zhang et al., 2014) and seriously affected the safety of urban land use. Despite the fact that some areas have been compensated by valley filling, it is likely to cause ground subsidence in the near future (Figure 2).

The universal soil loss equation (USLE) proposed by Wischmeier is a common method for predicting soil erosion, but the model is too provincial and poor portability. In view of this, Renard revised the USLE model and established the Revised Universal Soil Loss Equation (RUSLE) in 1992. The RUSLE model has a simple structure, fewer parameters, and easy access to the required data (Wang et al, 2013; Feng et al., 2016), making it the most widely used universal model in the world (Devatha et al., 2015; Tamene et al., 2017; Jiang et al., 2019). Mohammad et al. (2017) simulated the impacts of future land use change on soil erosion in the Kasilian watershed using the RUSLE and Markov Cellular Automata. The results indicate that the most significant factor of erosion induced by land use change is the change from forest to settlements. Zhang et al. (2011) analyzed the relationship between changes in landscape pattern and soil erosion in Malian River Basin, where is a small watershed in TGR of LP. The results show that reduction of soil erosion was related to expansion of urban land during 2001 to 2007. In order to make soil erosion simulation results more in line with Chinese reality, the Chinese soil loss equation (CSLE) based on the successful experience of USLE/RUSLE has been established in 2001 (Liu et al., 2002). It can be simply understood as adding the factor of soil and water conservation engineering measures based on the RUSLE model, which has good applicability in LP of China (Soil Conservation Monitoring Center, 2005). Xie (2008) evaluated soil erosion of a small watershed in LP based on the CSLE model, and found that the results have a good correlation with sediment transport.

However, there are few reports about land use and soil erosion on DZY. Zhang et al. (2014) analyzed the temporal variation characteristics of rainfall erosivity in the core area of DZY. Although the rainfall erosivity did not increase significantly in 61 years, the variability mutation will cause more serious erosion damage to the soil during the peak year. In order to explore the laws of soil loss and analyze the benefits of soil and water conservation in TGR of LP. The experimental observation station of soil erosion in TGR of Xifeng was established in 1951. Wang (2016) revealed the basic laws of soil loss in TGR of LP by the observation data for more than 60 years, and proposed the comprehensive management model of the soil and water loss in TGR of LP. It has made important contributions to control soil erosion in LP and even the whole country. However, the previous studies ignored the relationship between land use and soil erosion. Guo et al. (2019) studied the sediment load change of a small watershed in TGR and found that Sediment load change was mainly influenced by human induced vegetation cover changes. Yan (2018) proved that the land use types of DZY in diverse soil erosion severity (low to high) are: forest, shrub, farmland, urban, grassland and bare land. On the contrary, Liang et al. (2018) analyzed the effect of Robinia pseudoacacia forest on soil moisture, and believed that large-scale afforestation in LP may reduce soil moisture and cause dry soil layers, which has accelerated soil erosion. It means the Grain for Green Program may increase soil erosion to some degree. In fact, it has been corroborated by multiple studies (Wang et al., 2015, 2016; Guo et al., 2018). Then, the relationship between land use change and soil erosion remains divergent.

Therefore, the objectives were to (a) explore the key soil erosion factors in DZY based on the two models of RUSLE and CSLE; (b) estimate the temporal-spatial changes of land use and soil erosion during 1987-2017; and (C) discuss the relationship between land use and soil erosion in DZY, and then provide references for rational land use planning and reduction of soil erosion in LP.

2 | Description of study area and data

- 2.1 | Study site

The Dongzhiyuan (DZY) (107°27′ - 107°57′E, 35°15′ - 36°4′N) is situated in the south-central Chinese Loess Plateau, north of the Wei River, west of the Malian River, east of the Pu River and south of the ditch of Caijiamiao, with an altitude of 1200 to 1500 m and an area of 2813.33 km2 (Figure 3). It is one of the distribution centers of loess accumulated since the Quaternary of China. The DZY is a typical area in TGR of LP in China, which has the largest remaining loess tableland in China with a maximum surface area about 960 km2 (slope <10°) and the thickest loess deposits (100 m - 200 m) (Wang et al., 2015). DZY is mainly composed of two major geomorphic units: Loess Tableland and Loess Gully. According to the statistics, 373 first-level ditches of five surrounding rivers have been extending into the hinterland of the DZY, which divided the face of DZY into pieces. (Zhang et al., 2014). There have been obvious temperate continental monsoon climate characteristics, with an average annual temperature of 8.6 °C and average annual precipitation of 525 mm (Yang et al., 2014; Wang et al., 2015). The soil type is mainly dark loessial soil (Heilu soil) and the loessal soil (Soil inventory office in Qingyang, 1989). The dark loessial soil is mainly distributed on the loess tableland with the good water storage and strong fertility. The loess soil is mainly distributed on slopes and ditches, with a loose texture, which is easily washed away by rainwater and causes soil erosion. It is the main source of river sediment (Ren et al., 2017). The DZY is dominated by perennial herbaceous vegetation, with poor vegetation coverage and fragile ecological environment.

- 2.2 | Data collection and processing

- Multi-source datasets were gathered and used to simulate land use change and soil erosion in DZY (Table 1). The topography data of DEM and grade length were utilized to calculate the slope length and slope. We use Landsat TM/OLI images to extract NDVI (Normalized vegetation index) according to different bands. At the same time, those Landsat images can be used for the classification of land use by supervised classification. In order to account for the contribution of precipitation to soil erosion, we use the monthly rainfall dataset for 1km (Yang, 2015). Because there is only one national weather station in the study area (Dongzhi), 1km raster data can compensate for the spatial homogeneity of precipitation due to the scarcity of meteorological stations. We are required to digitize the soil map of Qingyang City with a ratio of 1:138,000 to obtain the vector data of the soil type map of the study area. Besides, we collected silt dams data through the Soil and Water Conservation Monitoring Center of the Ministry of Water Resources, and combined with Google Earth and field visits. The data mainly includes the number of silt dams and their control area. It can estimate the erosion resistance of soil and water conservation engineering measures in 2017 (Figure 1, Table 2).

- 3 | Methods

- 3.1 | Estimation of soil erosion based on RUSLE

We utilize the RUSLE model to estimate the soil erosion in DZY of the loess plateau. The expression is as follows (Wischmeier, 1978; Lin et al., 2017; Wang et al., 2018):

ARUSLE = R× K × LS × C × P.                                  (1)

Where ARUSLE is the modulus of soil erosion (t·ha-1·yr-1) based on the RUSLE model, R is the rainfall erosivity factor (MJ·mm·ha-1·h -1·yr-1), K is the soil erodibility factor(t·ha·h·MJ-1·mm-1·ha-1). LS, C and P are the dimensionless factors, which represent the topographic factor, crop management factor and the conservation support practice factor, respectively.

The rainfall erosivity factor (R) reflects the erosion ability of rainfall to the soil. It is determined from rainfall and rainfall intensity. However, there are few rainfall stations in the study area and lack of sufficient rainfall intensity data; we just use the monthly rainfall data and the yearly rainfall data to set up the simple model of R (Quansah, 1981; Lin et al., 2017). Soil erodibility factor (K) is related to the mechanical composition of different soil types. Because DZY is located in Qingyang of Gansu Province. We obtained the vector data of soil type from the map of Qingyang soil at 1:1380000 scale. Then, sand, powder, clay and organic matter content of different soil types were collected (Soil inventory office in Qingyang, 1989). Then, soil erodibility factor K could be calculated according to Men et al. (2004). Topographic factor (LS) is determined by the slope length and slope, which both can be extracted from DEM. The classic international algorithm for the topographic factor is not suitable for the gully region of the Loess Plateau. Here, we use the modified LS algorithm proposed by Wang et al. (2007). Crop management factor (C) refers to the ratio of the amount of soil loss of a particular crop or vegetation to the amount of soil loss after continuous cultivation under the same soil, slope and precipitation conditions. It is a dimensionless number whose value varies between 0 and 1. Crop management factor (C) can be calculated by the formula proposed by Qin et al. (2006). The conservation support practice factor (P) refers to the ratio of the amount of soil loss after special measures to the amount of soil loss when farming downhill. At present, trimming terraces is the most basic soil and water conservation in China. In order to quantitatively evaluate the intensity of soil erosion, we found that P is related to land use type and slope (Wang et al., 2000; Zhang et al., 2011). The calculation formulas of the factors used in this paper are described in detail in the relevant references.

- 3.2 | Chinese soil loss equation (CSLE) based on RUSLE

The revised universal soil loss equation (RUSLE) which proposed by Wischmeier is the most widely used method to predict soil erosion. However, considering the differences in physical and geographical conditions, Liu et al. combined with Chinese reality and proposed the Chinese Soil Loss Equation (CSLE) based on the RUSLE model (Liu et al., 2002; Liu, 2006). It can be expressed as:

ACSLE = R× K × LS × B × E×T.                                  (2)

Where ACSLE is the modulus of soil erosion (t·ha-1·yr-1) based on the CSLE model. R, K and LS are the same as formula (1). B is vegetation cover and biological measure factor and T is farming measure factor, which are the same as C and P in formula (1), respectively. E is an engineering measure factor which refers to the ratio of soil erosion in runoff plots with engineering measures to soil erosion in runoff plots without engineering measures under the same conditions. Then, the modulus of soil erosion ACSLE can be expressed as:

ACSLE = ARUSLE× E.                                           (3)

Measures for soil and water conservation in the Loess Plateau include slope and ditch management. Slope control projects, such as horizontal terraces or equal height farming, have been considered in the P factor; trench control projects rely on silt dams in general, which are the silt dams never was deposited. The silt dams can completely block the incoming sand in the accumulation control area. Therefore, this study simplified the formula for calculating E proposed by Xie (2008). Then, the engineering measure factor (E) can be calculated as follows:

E=1−Sd/S.                                           (4)

Where Sd is controlled area of silt dams, S is the sub-watershed area.

- 4 | Results and discussions

- 4.1 | Analysis of soil erosion factors

The spatial and temporal characteristics of the rainfall erosivity factor (R) are very obvious. In general, the R was higher in the south and lowered in the north of the region. However, the distribution pattern of R has changed in 1997 and 2007. The high-value zone of R has extended from south to north, while its low-value zone has moved to the edge of the region. Besides, R showed a clear increasing trend, which has increased by 10-20 units from1997 to 2017. Because of the change of R on the time scale is very significant, it must be under a decisive effect on the results of soil erosion simulation. To stress the impact of other factors on soil erosion, we calculated the annual average value of R to simulate the changes of other factors under the same R for soil erosion changes (Figure 4a-e).

The crop management factor (C) showed a trend of increasing first and then decreasing in the four periods. It was mostly determined by the change of vegetation coverage. We can see the C factor was highest in 1997 because of the dwindling vegetation coverage. It was most likely due to over and unreasonable land use. However, the continuous restoration of vegetation after the 1990s was mainly due to the implementation of policies such as the returning farmland to forest program and grazing bans. Urbanization has also caused the C factor to become higher in some areas. The hydrothermal conditions in the south area were better than those in the north under the spatial scale (yan, 2018), so the C factor was lower (Figure 4g-k).

The conservation support practice factor (P) was directly involved in land use, especially the contour farming under different slope (Liu et al., 2019). The spatial distribution pattern of the P factor was established by the two major landform units of the loess tableland and the loess gully of DZY. In general, the P value is lower in the Loess Tableland and higher in the Loess Gully region. Besides, conservation support practice factor P had a decreasing trend on the Loess Tableland after 1997. On the one hand, it may be due to the urbanization of the area; on the other hand, it may be caused by the conversion of sloping farmland to forest and grassland.

The soil erodibility factor (K) was established by the mechanical composition of different soil types. The soil was barren and the content of soil organic matter was low in DZY. Dark loessial soil and the loessial soil were the main soil types, which distributed in the loess tableland and the loess gully of DZY. In addition, there is also some alluvial soil in the river valley of the edge of DZY, and a small amount of red clay and dehumidified soil in the north and the east of the region, respectively. For the mechanical composition of soil, the content of silt was exceeding 50%, which was the highest one among all soil types. The loessal soil content of silt was arrived at 62.9%, while the content of organic matter was as low as 0.77, which had resulted in the highest K value (0.407) with the weak erosion resistance. Alluvial soil was similar to the loessal soil for the mechanical composition of the soil. So the factor K was higher too. Dark loessial soil and the dehumidified soil were coarse-grained, but their organic matter content was higher. Therefore, those two K values were lower relative. Red clay has the highest clay content (36%), which was the lowest K value (0.366) because of the strongest soil erosion resistance (Table 2, Figure 4f).

The topographic factor (LS) was established by the slope length and slope. It was lower in the loess tableland and higher in the loess gully of DZY. The topographic factor would not change significantly in a short period of 30 years, just like the K factor (Figure 4l).

In order to compare the simulation results of RUSLE and CSLE, we need to consider the engineering measure factor (E). In fact, the simulation results of CSLE only superimposed the E factor on the basis of the simulation results of the RUSLE model (Formula 3). The construction of silt dams can not only effectively intercept sediment and flood control security, but also can build silt land, increase production and income, and facilitate transportation. It is an important ecological construction measure that benefits the country and the people. Therefore, it is a good soil and water conservation engineering measure. We divided the study area into 32 sub-watersheds and numbered each sub-watershed according to the size of the area. There were 93 silt dams and the control area of soil erosion by more than 22% of the region in 2017. According to the statistics,  sub-watershed 28 has the lowest E value (0.001). Because the largest reservoir (Bajiazui reservoir) of the DZY located right here, which controlled almost the entire sub-watershed 28. Besides, the E value of sub-watershed 22 was as low as 0.014. The sub-watershed 22 named Nanxiaohegou, which controlled more than 90% of the sub-watershed. Xifeng Water Conservation Station of the Yellow River Water Conservancy Commission had carried out large-scale comprehensive treatment in the Nanxiaohegou watershed (Wang & Zhao, 2011; Wang & Zhang, 2016; Wang, 2016). It was the initial implementation area for soil and water conservation projects and had a beneficial effect for soil erosion control. The watershed 16 named Laohugou with up to 15 silt dams, which is the most intensive area for DZY to implement soil and water conservation projects. The soil erosion control area was large, and the E value was low. The second largest sub-watershed Yanwachuan located in the hinterland of DZY. It was a typical loess tableland and gully region with large-scale soil and water conservation projects (Chang et al., 2015). There were 13 silt dams controlled nearly 60% of the Yanwachuan watershed. Sub-watershed 12 named Xiaohegou also had 13 silt dams, but it is mainly concentrated upstream of the watershed. Therefore, soil and water conservation engineering measures in the lower reaches of the watershed were relatively insubstantial. Also, the sub-watershed 1, 3, 5, 6, 10 distributed about 5-10 silt dams, and the watershed control area was small with the higher E factor. The rest of sub-watersheds with only 1-2 silt dams and even some areas had no silt dams, so their soil and water conservation engineering measures were the weakest, and the E factor were close to 1.000 (Table 3, Figure 4r).

4.2 | Temporal-spatial changes of soil erosion

We can be clear from Figure 4a-e that R has particularly obvious characteristics of spatiotemporal changes. Then, the R factor may have a great impact on the simulation of the soil erosion model. When we use R in different periods to simulate the soil erosion, the average soil erosion modulus was 9.86 t·ha-1·yr-1, 19.79 t·ha-1·yr-1, 14.87 t·ha-1·yr-1 and 21.53 t ha-1 yr-1 in 1987, 1997, 2007and 2017, respectively. It may weaken the influence of other factors on the results. However, if only the annual average R was used for simulation, the average soil erosion modulus would become 9.86 t·ha-1·yr-1, 19.79 t·ha-1·yr-1, 14.87 t·ha-1·yr-1 and 21.53 t ha-1 yr-1. And then, the changes in soil erosion caused by the changes in rainfall erosivity would be ignored. Therefore, we combined the above two situations to analyze the spatiotemporal characteristics of soil erosion under different models (RUSLE/CSLE).

- 4.2.1 | Overall changes of soil erosion in DZY

In general, soil erosion intensity is weak on the tableland, while strong in the gully region. Due to the high gully density and the low vegetation coverage, the soil erosion was more serious in the north of DZY (Figure 5). The soil erosion increased more than 1,100 km2, while reduced less than 50 km2 from 1987-1997. Conversely, the increased area was lower than 100 km2, while reduced more than 500 km2 from 1997-2007. Especially, the erosion reduction area was more close to 1000 km2 under the same R during 1997-2007. Besides, increased rainfall erosion caused soil erosion to increase more and to decrease less during 2007-2017. However, soil erosion would have increased less and decrease more under the same R. It means the increase in R is much larger than the decrease in other factors, which make the simulation results of soil erosion larger (Figure 6).

According to the Classification and Grade standards of soil erosion (SL190-2007) issued by the Ministry of Water Resources of the People's Republic of China, we developed a classification standard of soil erosion in DZY (table 4). Specifically, no erosion areas are gradually increasing by the rapid development of urbanization. The slight, mild and moderate erosion are no significant change, which are the most important erosion types accounting for more than 80% of the DZY. The intensity, extreme and severe erosion area were greatly affected by R, and changed in varying degrees from 1987-2017. Those three erosion types accounted for the highest proportion in 2017 and lower in 1997 under the different R, while the situation is opposite under the same R (Figure 7). It shows that the erosion intensity will increase more significantly with increasing of R. However, the crop management factor (C) would play a more important role under the same R. The simulation of CSLE model considering the E factor reduced the soil erosion in some degree compared with the RUSLE model. Therefore, the largest reduction regions were mainly sub-watershed Nos. 16, 22 and 28, followed by sub-watershed Nos. 1, 2, 5, 10 and 12, while the other regions were not significantly decreased (Figure 4r, Figure 5e, j).

- 4.2.2 | Mean value and range of soil erosion based on sub-watersheds

We divided DZY into 32 sub-watersheds, and the average soil erosion modulus of each sub-watershed was calculated on the basis of the simulation results of RUSLE and CSLE. After the statistical analysis, the average and range of soil erosion in DZY under the different R and the same R can be showing from the box plots (Figure 8). It shows an increasing-decreasing-increasing trend under the different R from 1987 to 2017 (Figure 5a, Figure 6a). And the average value of soil erosion in 1987 was lower than 10 t·ha-1·yr -1 with a slight erosion. There was fluctuated within the range of mild erosion between 10-25 t·ha-1·yr -1 after 1987, of which the average erosion modulus was higher in 1997 and 2017. The amplitude of erosion change in RUSLE model from 1987 to 2017 increased slightly. Nevertheless, the CSLE model showed the largest amplitude of change from non-erosion to moderate erosion (Figure 8a). However, under the same R, the average value of soil erosion in 1997 is the largest, up to more than 20 t·ha-1·yr -1, and its variation is also the largest. The average values in other periods are relatively stable, all staying at 10-15 t·ha-1·yr -1(Figure 8b). In comparison, the average soil erosion increased in 1987 and 1997 compared with the former of the different R, and almost unchanged in 2007, while decreased significantly in 2017. Besides, the CSLE model taking the E factor into account, and the average soil erosion reduced more (Figure 8). In fact, soil erosion under the same R may be more in line with the actual situation according to existing research combined with field research (yan, 2018). It shows that soil erosion has improved significantly due to the restoration of vegetation and the implementation of soil and water conservation measures, although the R continues to increase.

4.3 | Analysis of Land use changes

The principle land use types are farmland, forest, grassland, water area and construction land in DZY. Combining supervised and unsupervised classification methods to classify the remote sensing images (Landsat TM/OLI). Then, we get the spatial distribution map of land use in 1987, 1997, 2007 and 2017. The total classification accuracy was over 88%, and the Kappa coefficient was greater than 0.85. It showed that farmland was mainly distributed on the loess table land, while the grassland was mainly distributed on the loess gully region. Those were the two principle land use types for DZY. Besides, the forest mainly distributed in the gully region of the central and southern areas. Urban was concentrated on the loess platform in the core area of DZY. The area of water was the smallest. It mainly distributed in the low-lying areas at the edge of the region. In the past 30 years, farmland has shown a tendency to decrease first and increase last, while grassland was the opposite. Forest land and urban have continued to increase(Figure 9). From 1987 to 1997, the main transfer types were the mutual transfer between farmland and grassland, grassland and forest, and the transfer of farmland to forest. Among them, the largest converted area was farmland to grassland (378.6km2). From 1997 to 2007, the main transfer were the mutual transfer between farmland and grassland, grassland and forest, followed by the transfer of forest to farmland(66.22km2). From 2007 to 2017, the direction of land use transfer was relatively single. It was mainly the transfer of grassland to farmland and forest, followed by the transfer of farmland to grassland. In addition, there is also a large area of farmland converted to urban (53.46km2) (Table 5).

- 4.4 | Response of land use change to soil erosion

We analyzed several major land use changes for the increased soil erosion and reduced soil erosion, respectively. Through the previous analysis, we found that increased soil erosion was mainly during 1987-1997, while the decreased one was mainly during 1997-2007. What’s more, both of the above conditions exist during 2007-2017. There were 10 main types of land use change processes corresponding to the soil erosion changes. They were farmland to grassland, farmland to urban, forest to grassland, grassland to farmland, grassland to forest, grassland to water, grassland to urban, urban to farmland, urban to grassland and others, respectively. In general, the conversion of urban to farmland and grassland would increase soil erosion (Figure 10a), while the conversion of farmland and grassland to urban, and grassland to water would reduce soil erosion (Figure 10b). The results are in agreement with the findings from Zhang et al. (2011). Although the urban expansion is conducive to reducing soil erosion, increasing urban land through mountain excavation and gully filling is likely to produce landslides and ground subsidence, which will generate a large and expensive heritage of geological disasters for future generations (Juang et al., 2019). Mutual changes between farmland and grassland, forest and grassland will cause soil erosion to change in different directions, and the larger the change in land use, the more severe the soil erosion. In addition, the land use change for the two periods of increased soil erosion was similar, while the decreased one was different. There was not any large-scale transfer of farmland and forest to grassland in the decreased period of soil erosion during 2007-2017.

- 5 | Conclusion

On the basis of the simulation of the RUSLE model, we combined the CSLE model in 2017, and discussed the spatial and temporal change of soil erosion in two cases of the R factor. The soil erosion showed a trend of increasing- decreasing-increasing under the different periods of R, while it showed an increasing-decreasing-decreasing trend under the annual average of R. In fact, the last situation was found to be more in line with the actual situation through the combination of existing research and field surveys, indicating that the large changes in the R factor weakened the impact of other factors on soil erosion, which caused the simulation of the RUSLE model to be overestimated to some extent. To this end, we set up the CSLE model based on the RUSLE model with the data of silt dams in 2017. The results prove that soil erosion has improved considerably due to the restoration of vegetation and the implementation of soil and water, even with increased rainfall erosion. Of course, we only think the silt dams data as the soil and water conservation engineering factor (E), which will further overestimate the simulation value. It means that soil erosion is more optimistic than the simulation in DZY.

During 1987 to 2017, the land use changed dramatically and had a certain effect on soil erosion in DZY. There were 10 types of land use change related to the change of soil erosion: farmland to grassland, farmland to urban, forest to grassland, grassland to farmland, grassland to forest, grassland to water, grassland to urban, urban to farmland, urban to grassland and others, respectively. The conversion of urban to farmland and grassland will increase soil erosion, while conversion of farmland and grassland to urban will reduce the erosion. The reciprocal transfer between farmland and grassland, forest and grassland will cause soil erosion to change in different directions. The greater land use change, the more serious soil erosion will be.

- Acknowledgements

This work was supported by the National Natural Science Foundation of China (41530745) and the National Key Research and Development Program of China (Grant No. 2019YFC0507402). Acknowledgement for the data support from "Loess plateau science data center, National Earth System Science Data Sharing Infrastructure, National Science & Technology Infrastructure of China. (http://loess.geodata.cn)".

- References

Alkharabsheh, M., Alexandridis, T.K., Bilas, G., Misopolinos, N., & Silleos, N. (2013). Impact of land cover change on soil erosion hazard in northern Jordan using remote sensing and GIS. Procedia Environmental Sciences, 19, 912-921. https://doi.org/10.1016/j.proenv.2013.06.101

Chang, W.Z., Wang, Z.X., Hu, W.F., & Min, H.J. (2015). Experience and reflections on the construction of the Yanwachuan project of Yellow River Ecological Engineering. China Soil and Water Conservation, (5), 62-63.  https://doi.org/10.14123/j.cnki.swcc.2015.0136

Chen, H. (2019). Spatial and temporal changes of soil erosion and its driving factors before and after the Grain for Green project in the Loess Plateau. Master Dissertation. Yangling: Northwest A&F University.

Chen, Y.Z., Jing, K., & Cai, G.Q. (1988). Modern Erosion and Control in the Loess Plateau. Beijing: Science Press.

Devatha, C.P., Deshpande Vaibhav, & Renukaprasad M. S. (2015). Estimation of Soil loss using USLE model for Kulhan Watershed, Chattisgarh-A case study. Aquatic Procedia, (04), 1429-1436. https://doi.org/10.1016/j.aqpro.2015.02.185

Feng, T., Chen, H.S., Polyakov, V.O., Wang, K.L., Zhang, X.B., & Zhang, W. (2016). Soil erosion rates in two karst peak- cluster depression basins of northwest Guangxi, China: Comparison of the RUSLE model with 137 Cs measurements. Geomorphology, 253, 217-224. https://doi.org/10.1016/j.geomorph.2015.10.013

Feng, J.J., Shi, M.C., & Jiang, Q.O. (2019). Influence of land use / cover change on soil erosion in Chaobai river Basin. Science of Soil and Water Conservation, 17(03), 121-132. https://doi.org/10.16843/j.sswc.2019.03.016

Fu, B.J., Chen, L.D., & Ma, K.M. (1999). The effect of land use change on the regional environment in the Yangjuangou catchment in the Loess Plateau of China. Acta Geographica Sinica, 54(3), 241-246. https://doi.org/10.3321/j.issn:0375-5444.1999.03.006

Gao, G.Y., Fu, B.J., Lü, Y.H., Liu, Y., Wang, S., & Zhou, J. (2013). The effect of land cover pattern on hillslope soil and water loss in the arid and semi-arid region. Acta Ecologica Sinica, 33(1), 12-22. https://doi.org/CNKI:SUN:STXB.0.2013-01-001

Guo, N., Wang, A., Degen, A.A., Deng, B., Shang, Z., Ding, L., & Long, R. (2018). Grazing exclusion increases soil CO2 emission during the growing season in alpine meadows on the Tibetan Plateau. Atmospheric Environment, 174, 92-98. https://doi.org/10.1016/j.atmosenv.2017.11.053

Guo, Q.K., Ding, Z.W., Qin, W., Cao, W.H., Liu, W., Xu, X.M., & Yin, Z. (2019). Changes in sediment load in a typical watershed in the tableland and gully region of the Loess Plateau, China. Catena, 182, 1-14. https://doi.org/10.1016/j.catena.2019.104132

Hudson, N.W. (1995). Soil Conservation. Iowa: Iowa State University Press.

Jiang, C., Zhang, H.Y., Wang, X.C., Feng, Y.Q., & Labzovskii, L. (2019). Challenging the land degradation in China’s Loess Plateau: Benefits, limitations, sustainability, and adaptive strategies of soil and water conservation. Ecological Engineering, 127, 135-150. https://doi.org/10.1016/j.ecoleng.2018.11.018

Juang, C.H., Dijkstra, T., Wasowski, J., & Meng, X.M. (2019). Loess geohazards research in china: advances and challenges for mega engineering projects. Engineering Geology, 251, 1-10. https://doi.org/10.1016/j.enggeo.2019.01.019

Liang, H.B., Xue, Y.R., Li, Z.S., Wang, S., Wu, X., Gao, G.Y., … Fu, B.J. (2018). Soil moisture decline following the plantation of Robinia pseudoacacia forests: Evidence from the Loess Plateau. Forest Ecology and Management, 412, 62-69. https://doi.org/10.1016/j.foreco.2018.01.041

Lin, H.L., Zheng, S.T., & Wang, X.L. (2017). Soil erosion assessment based on the RUSLE model in the Three-Rivers Headwaters area, Qinghai-Tibetan Plateau, China. Acta Prataculturae Sinica, 26(7), 11-22. https://doi.org/10.11686/cyxb2016371

Liu, B.Y., Zhang, K.L., & Xie, Y. (2002). An empirical soil loss equation. Proceedings Process of Soil Erosion and Its Environment Effect (vol II), 12th International Soil Conservation Organization Conference. Beijing: Tsinghua Press, 21-25.

Liu, B.Y. (2006). Report on the Research of the Soil Erosion Forecasting Model Development Project in the Northwest of Loess Plateau. Beijing: Water and Soil Conservation Monitoring Center of the Ministry of Water Resources.

Liu, W.C., Liu, J.Y., & Kuang, W.H. (2019). Spatiotemporal patterns of soil protection effect of the Grain for Green Project in northern Shaanxi. Acta Geographic Sinica, 74(9), 1835-1852. 	doi.org/10.11821/dlxb201909010

Mabit, L., Zapata, F., Dercon, G., Benmansour, M., Bernard, C., & Walling, D. E. (2014). Assessment of Soil Erosion and Sedimentation: The Role of Fallout Radionuclides. Iaea Tecdoc Series, 3: 181-201. https://doi.org/10.1007/s11442-018-1553-2

Men, M.X., Zhao, T.K., Peng Z.P., & Yu Z.R. (2004). Study on the Soil Erodibility Based on the Soil Particle-size Distribution in Hebei Province. Scientia Agricultura Sinica, 37(11), 1647-1653. https://doi.org/10.3969/j.issn.1000-6850.2007.07.131

Mohammad, Z., Thomas, P., & Luis, L. (2017) Simulating the impacts of future land use change on soil erosion in the Kasilian watershed, Iran. Land Use Policy, 67, 558–572. http://dx.doi.org/10.1016/j.landusepol.2017.06.028

Oldeman, L.R. (1994). The global extent of soil degradation. In: Greenland DJ, Szabolcs I (eds) Soil resilience and sustainable land use. CAB International, Wallingford, pp 99-118.

Peng, S.Z., Gang, C.C., Cao, Y., & Chen, Y.M. (2018). Assessment of climate change trends over the Loess Plateau in China from 1901 to 2100. International Journal of Climatology, 38, 2250-2264. https://doi.org/10.1002/joc.5331

Qin, W., Zhu, Q.K., Zhang, X.X., Li, W.H., & Fang, B. (2006). Review of vegetation covering and its measuring and calculating method. Journal of Northwest Agriculture and Forestry University (Natural Science Edition), 34(9), 163-170. https://doi.org/10.3321/j.issn:1671-9387.2006.09.031

Quansah, C. (1981). The effect of soil type, slope, rain intensity and their interactions on splash detachment and transport. Journal of Soil Science, 32(2), 215-224. https://doi.org/10.1111/j.1365-2389.1981.tb01701.x

Ren, P., Wang, L., & Yan, J.P. (2017). Research on soil and water loss prevention model and inclusion system in Dongzhiyuan of Qingyang City. Water Resources Development Research, 17(03), 41-45. https://doi.org/CNKI:SUN:SLFZ.0.2017-03-012

Soil Conservation Monitoring Center, (2005). Soil and water loss equation for the Loess Plateau-user’s Guide. Beijing: Ministry of Water Resources and Beijing Normal University.

Soil inventory office in Qingyang (1989). Soils in Qingyang, Gansu Province. Gansu Science & Technology Press, Lanzhou (in Chinese).

Tamene, L., Adimassu, Z., Aynekulu, E., & Yaekob, T. (2017). Estimating landscape susceptibility to soil erosion using a GIS-based approach in Northern Ethiopia. International Soil and Water Conservation Research, (05), 221-230. https://doi.org/10.1016/j.iswcr.2017.05.002

Thomas, J., Joseph, S., & Thrivikramji, K. P. (2017). Assessment of soil erosion in a tropical mountain river basin of the southern Western Ghats, India using RUSLE and GIS. Geoscience Frontiers, (06), 1-14. https://doi.org/CNKI:SUN:GSFT.0.2018-03-012

Wang, H.B. (2016). Experimental Observation of Soil and Water Loss Regularity in the loess table and gully region of Xifeng Soil and Water Conservation Station. China Soil and Water Conservation, (10), 27-28. https://doi.org/10.3969/j.issn.1000-0941.2016.10.010

Wang, H.S., Liu, G.B., & Wang, Q.N. (2000). Effect of vegetation as whole in watershed on protecting soil from erosion and dissection of its landscape structure. Journal of Soil and Water Conservation, 14(5), 73-77. https://doi.org/10.3321/j.issn:1009-2242.2000.z1.014

Wang, J., Gao, J.B, & Hou, W.J. (2018). Quantitative attribution analysis of soil erosion in different morphological types of geomorphology in karst areas: Based on the geographical detector method. Acta Geographica Sinica, 73(9), 1674-1686. https://doi.org/DOI: 10.11821/dlxb201809005

Wang, B.W., Yang, Q.K., Liu, Z.H., & Zhao, Z.C. (2007). Extraction of RUSLE-LS factors using DEM data and GIS techniques. Science of Soil and Water Conservation, 5(2), 18-23. https://doi.org/10.3969/j.issn.1672-3007.2007.02.004

Wang, D., Wu, G.L., Liu, Y., Yang, Z., & Hao, H.M. (2015). Effects of grazing exclusion on CO2 fluxes in a steppe grassland on the Loess Plateau China. Ecological Engineering, 83, 169-175. https://doi.org/10.1016/j.ecoleng.2015.06.017

Wang, Z., Ji, L., Hou, X., & Schellenberg, M.P. (2016). Soil respiration in semiarid temperate grasslands under various land management. PLoS One 11, 0147987. https://doi.org/10.1371/journal.pone.0147987

Wang, Z.X., & Zhao, A.C. (2011). Practice of Science and Technology Demonstration Park of Soil and water conservation in Nanxiaohegou. China Soil and Water Conservation, (1), 37-54. https://doi.org/10.14123/j.cnki.swcc.2011.01.015

Wang, Z.X., & Zhang, G. (2016). Science and Technology Demonstration Park of Soil and water conservation in Nanxiaohegou. China Soil and Water Conservation, (9), 1. https://doi.org/10.14123/j.cnki.swcc.2016.0255

Wang, S.B., Li, C.B., Yang, L.S., Zhang, X.L., Yang, W.J., & Wang, X.S. (2015). Reasonable utilization of water resources based on WEAP simulations in the Dongzhi Loess Tableland. Agricultural Research in the Arid Areas, 33(4), 238-245. https://doi.org/10.7606/j.issn.1000-7601.2015.04.36

Wang, Y., Cai,Y.L., & Pan, M. (2013). Analysis on the relationship between soil erosion and land use in Wujiang River Basin in Guizhou Province. Research of Soil and Water Conservation, 20(3), 11-18.

Wischmeier, W.H., & Smith, D.D. (1978). Predicting Rainfall Erosion Losses: a Guide to Conservation Planning. Agriculture Handbook No. 537. USDA, Washington, DC.

Wu, L., Liu, X., & Ma, X.Y. (2016). Tracking soil erosion changes in an easily-eroded watershed of the Chinese Loess Plateau. Polish Journal of Environmental Studies, 25(1), 351-363.

Xie, H.X. (2008). Study on the Spatio-temporal change of soil loss and on the assessment of impacts on environment pf soil and water conservation in Yanhe Basin. Master Dissertation. xi’an: Shaanxi normal university.

Xie, L.X. (2008). Study on the spatio-temporal change of soil loss and on the assessment of impacts on environment of soil and water conservation in Yanhe basin. Doctor Dissertation. Xi’an: Shaanxi University.

Yan, Y. (2018). Relationship between landuse change and soil erosion in Dongzhiyuan. Master Dissertation. Chendu: Chendu University of Technology.

Yang, Q.K. (2015). Dataset of grade length with 30 m resolution on the Loess Plateau (2014). National Data Sharing Infrastructure of Earth System Science_Data Sharing Infrastructure of Loess Plateau.

Yang, L.S., Li, C.B., Wang, S.B., & Yang, W.J. (2014). Water balance within soil water reservoir in the loess tableland by means of combination of SWAT model and remote sensing: A case study in the Dongzhi Loess Tableland of eastern Gansu. Journal of Glaciology and Geocryology, 36(3), 691-698. https://doi.org/10.7522/j.issn.1000-0240.2014.0083

Zhang, J.X., Zhang, B., Zhang, H., Zhang, D.Y., Dai, S.P., & Ma, Z.H. (2011). Landscape Pattern Change and Soil Erosion Research - Take the Example for Malian River Basin. Journal of Natural Resources, 26(9), 1513-1525. https://doi.org/10.1007/s12583-011-0162-0

Zhang, J.X. , Zhang, D.Y., Liu, W.F., Hu, A.P., Sun, B., & Zhang, Y.L. (2019). Progress in research on land use and soil erosion in the Loess Plateau of China, IOP Conf. Series: Earth and Environmental Science 242, 052021. https://doi.org/10.1088/1755-1315/242/5/052021

Zhang, X., Li, Q.Q., Guo, J., Liu, J., Yue, T.X., Feng, Q., ... Zheng, Y.F. (2014). Temporal Distribution Characteristics and Variation Trend of Rainfall Erosivity in the Core Area of Dongzhi Tableland. Arid Zone Research, 31(6), 178-185. https://doi.org/10.13866/j.azr.2014.06.15