The bioaccumulative potential of heavy metals in five forest species living in mining environments in the Ecuadorian Amazon region

Yudel García-Quintana1*, Luis Ramón Bravo-Sánchez1, Dixon Andi Grefa1, Jorge de Jesús Guerrón-Torres1, Jenny Elizabeth Rosero-Sanchez1, Yasiel Arteaga-Crespo1

1Universidad Estatal Amazónica. Km. 2 ½ vía Puyo a Tena (Paso Lateral), Puyo, Pastaza, Ecuador

* Corresponding author: ygarcia@uea.edu.ec

Abstract

Pollution caused by heavy metals in soils and ecosystems is an environmental problematic that requires urgent attention due to the ecological problems that it generates. Forest species can be used to mitigate contamination because of their potential to bioaccumulate contaminating metals. Hence, the aim of this research was to identify tree species with good heavy metal bioaccumulating capacities that can contribute to mitigate pollution. The bioconcentration factor for five forest species, such as: Spanish cedar (Cedrela odorata L.), cutanga (Parkia multijuga Benth.), guaba or ice cream bean (Inga edulis Mart.), guarumo (Cecropia ficifolia Warb. ex Snethl.) and Amazon tree grape (Pourouma cecropiifolia Mart.), commonly found in the Ecuadorian Amazon was analysed, based on the relationship between the leaves and soil concentration of the heavy metals. For heavy metal analysis in leaves and soil samples of each plant species, atomic absorption spectrometry was used. The results showed that P. cecropiifolia had the highest bioconcentration factor for lead, C. odorata for cadmium and nickel, and I. edulis had the highest potential for iron and aluminium absorption. Any kind of correlation between the concentration of each element in soil and leaves was found, which shows that the bioaccumulation capacity of the species studied does not determine the concentration of metals in the soil.

Keywords: bioconcentration factor, Ecuadorian Amazon, heavy metals, identification of potential speciesfigure

- INTRODUCTION

Heavy metals are considered toxic elements and one of the world’s largest ecological problems, affecting human health both directly and indirectly (Kulkarni et al., 2018; Latif et al., 2018; Lazo et al., 2019; Palansooriya et al., 2020; Tóth et al., 2016). They are usual nature constituents and it is possible to find them in various environmental components (Garrett, 2000). Among these pollutants, chromium (Cr), copper (Cu), zinc (Zn), nickel (Ni), cadmium (Cd), lead (Pb), and mercury (Hg) are of special concern due to their persistence in the environment, such as its toxicity and possible risk of bioaccumulation (Kołodyńska et al., 2017; Li et al., 2018; Szymanowska et al., 1999). These elements can be together with suspended environmental powder and this way could be carried throw several kilometres by airflow, to settle again as dry or wet deposition (Janhäll, 2015; Lazo et al., 2019).

Recently, heavy metals have been analysed in various natural ecosystems (Tyler, 2005) in both slightly and highly polluted areas, such as mining (Serbula et al., 2013), agricultural (Latif et al., 2018; Ratul et al., 2018) or urban, where heavy metal content originates mainly from various anthropic events (Hu et al., 2014; Ugolini et al., 2013). These human activities could elevate metal content above their normal levels and origin toxic damages in living organisms (Neumann, 2000). Although some heavy metals such as zinc, copper, selenium, are also essential for the normal plant grow (Wuana & Okieimen, 2011); nevertheless, they could be extremely toxic, depending on their specific chemical form or concentration level (Latif et al., 2018; Rehman et al., 2018). They have a tendency to be bioaccumulated in several organisms, rendering them extremely hazardous (Nagajyoti et al., 2010).

Plants are extremely important because they act as an air filter and they release oxygen, regulating that way temperature of the air, thus accumulating some toxic chemical species (Sawidis et al., 2012). Additionally, some authors have been stated that roots and leaves could gather important amounts of  heavy metals (Liang et al., 2017; Ugolini et al., 2013), so, they can be environmental pollution markers. Trees can absorb both essential and non-essential metals, and the uptake of heavy metals by both roots and leaves increases when the concentration of heavy metals increases in the external environment (Ugolini et al., 2013), i.e. soil or air. The gathering of metals in plants may lead to conclude that such elements could be present in surrounded soil or air, but in many of then the heavy metals amounts can be several times higher in leaves than in soil (Sawidis et al., 2012). The toxicity of metallic elements in plants hang on several factors, such as: the nature of plant, sort of metal or physical or chemical properties of specific metal chemical specie, concentration level or edaphic characteristics (Nagajyoti et al., 2010; Pajević et al., 2016; Wuana & Okieimen, 2011). Therefore, the leaves of specific tree species have been used as biomonitors of heavy metal pollution in several studies (Çelik et al., 2005; Hu et al., 2014; Monfared et al., 2013; Ŝućur et al., 2010; Suzuki et al., 2009; Tomašević et al., 2004; Tóth et al., 2016). However, metal concentrations in tree organs are not a sufficient indicator of tree contamination, as uptake depends on the plant species and its bioconcentration factors, i.e. the ability of a plant to accumulate heavy metals from soil and air (Alahabadi et al., 2017).

In the Ecuadorian Amazon, given the proximity of volcanoes, soils may have high levels of heavy metals (Mainville et al., 2006). In addition, mining operations are a major cause of environmental contamination, due to inadequate mining waste treatment. Heavy metals are effortlessly released into the environment, posing a potential risk to human health. Small-scale gold mining is one the main causes of metallic elements releasing into the ecosystems, leading to severe pollution (Nakazawa et al., 2016; Xiao et al., 2017). This has been widely reported in developing countries, where inefficient or non-existent environmental regulations exacerbate the problem (Tarras-Wahlber et al., 2000). This fact is particularly worrying in Ecuador as illegal and uncontrolled mining has produced serious environmental pollution, mainly in terms of discharges of potentially toxic elements (Carling et al., 2013; Peña-Carpio & Menéndez-Aguado, 2016). These are not the only causes of heavy metal pollution; the population growth is also to blame in that it has led to the diversification of economic activity (Pimm et al., 2014), the intensive use of toxic agrochemicals (Panday et al., 2015) and so on. In view of the above, this research aimed to identify tree species with good bioaccumulate capacities of heavy metals that can mitigate pollution.

- MATERIALS AND METHODS

2.1 Study Area

This study was carried out in a mining area near the banks of the Jatunyacu River in the Yutsupino community, Puerto Napo parish, Napo province, Ecuador (Figure 1), where illegal gold mining activities are carried out.

Figure.1 Sampling area for soil and plant material (tree leaves).

2.2 Sampling of Plant Material and Soil

For the analysis of heavy metals present in the plant material (leaves), five tree species living in an area close to the mining site were selected: Spanish cedar (Cedrela odorata L.), cutanga (Parkia multijuga Benth.), guava (Inga edulis Mart.), guarumo (Cecropia ficifolia and Amazon tree grape (Pourouma cecropiifolia Mart.) (http://www.theplantlist.org/). The plant material was collected from different sides of the trees, whereby the branches were cut to avoid the leaves coming into contact with the metal scissors. Leaves without mechanical damage or apparent disease were selected.

Soil samples were taken at three different points at a depth of 0-30cm within 30cm around the selected trees; leaf litter and roots were removed. The samples were labelled and taken to the laboratory for further processing to search for evidence of lead (Pb), cadmium (Cd), nickel (Ni), iron (Fe) and aluminium (Al) presence.

2.3 Analysis of Soil and Leaf Samples

The collected soil and plant material samples were air-dried at room temperature for seven days before being taken to the forced circulation oven at a temperature of 40°C for 48 hours. The dried samples were placed in a mortar and pestle to grind them to a completely fine material with particle sizes of less than 2mm (Greksa et al., 2019).

A pre-labelled vial to process the digested soil and plant material samples was used. A mass of 0.5g of each sample was weighed, and then, 7mL of 65% nitric acid (HNO3), was added, followed by 1mL of 30% hydrogen peroxide (H2O2). The vial was closed and placed in a microwave-assisted digestion apparatus (ETHOS ONE) for 50 minutes. Once the digestion process was completed, the sample was allowed to stand at room temperature (Wang et al., 2019).

Once digested, the samples were transferred to a previously labelled volumetric flask, to which distilled water was added for a final volume of 25mL. The samples were filtered prior to analysis and lastly, heavy metal determinations were performed on an Aurora Intruments LTD atomic absorption spectrometer with a data processer software Trace 1200, under the following conditions: flame type for the elements Pb, Cd, Ni and Fe, air-acetylene and nitrous oxide-acetylene for Al, with an airflow of 1.5Lmin-1 (air pressure 50psi), acetylene flow of 3.5Lmin-1 (acetylene pressure 50psi) and for hollow cathode lamp conditions, 20mA current, slit width 0.2nm and wavelengths of 217nm, 228.8nm, 232nm, 248.3nm and 309.2nm for Pb, Cd, Ni, Fe and Al, respectively.

2.4 Data Analysis

An analysis of variance (ANOVA) and Tukey’s test were performed, in order to determine the significance of the differences in heavy metals (Pb, Cd, Zn, Fe and Al) concentrations in the species analysed (C. odorata, P. multijuga, I. edulis, C. ficifolia and P. cecropiifolia) as well as the concentrations of metals mentioned above in the soil around the place where they grow. A cluster analysis was performed to define the similarity between the heavy metal uptake capacity of the species, and Pearson’s correlation coefficient was used to determine the correlations in the concentrations of the heavy metals analysed in the plants and the soil. A Principal Component Analysis (PCA) to establish the separation of the analysed plants according to the concentrations of heavy metals in soil and trees was done. Origin 2021 software was used for the statistical analyses.

The bioconcentration factor (BCF) was employed to measure each plant’s ability to absorb heavy metals from the soil (Alahabadi et al., 2017). BCF was calculated using Equation (1):

| BCF=CleavesCsoil | (1) |
| --- | --- |

where Cleaves and Csoil represent the concentration of the element in the plant and in the soil, respectively.

- RESULTS

3.1 Concentrations of Heavy Metals in Leaves and Soil

Figure 2a shows the results of the concentration of the heavy metals Pb, Cd, Ni, Fe and Al present in the leaves of C. odorata, P. multijuga, I. edulis, C. ficifolia and P. cecropiifolia, whilst Figure 2b shows the concentrations in the soils where those species grow. Foliar Pb concentrations ranged from 0.25mg kg-1 in C. odorata to 7.05mg kg-1 for P. cecropiifolia. Foliar Cd concentrations ranged from 0.08mg kg-1 for P. cecropiifolia to 0.98mg kg-1 in C. odorata. As for Ni, the lowest value was found in P. multijuga with a concentration of 1.80mg kg-1 and the highest in C. ficifolia with 6.93mg kg-1. Fe values were 69.03mg kg-1 in C. ficifolia and 99.02mg kg-1 for I. edulis. Al was 109.4mg kg-1 in C. ficifolia and 149.6mg kg-1 for C. odorata.

Figure 2. Significance of differences in heavy metal concentrations (Pb, Cd, Ni, Fe and Al) between analysed (A) tree leaves and (B) their concentration in soil (The values within the same heavy metal followed by the same letter do not differ significantly according to Tukey’s test).

Pb concentrations in soil ranged from 1.18mg kg-1 for C. ficifolia to 1.89mg kg-1 for P. multijuga. Cd values ranged from 1.49mg kg-1 for C. odorata and P. cecropiifolia to 1.89mg kg-1 in P. multijuga. Ni concentrations ranged from 1.98mg kg-1 around the place where C. odorata was located, to 3.60mg kg-1 where I. edulis was grown. Fe in the soil had a minimum concentration of 198mg kg-1 and a maximum concentration of 390.8mg kg-1, corresponding to C. ficifolia and C. odorata respectively. Al values ranged from 322.4mg kg-1 in the soil around I. edulis to 542.2mg kg-1 for P. cecropiifolia.

3.2 Statistical Analysis

The Analysis of variance (ANOVA) and Tukey’s test showed statistical differences in the concentration of the metallic elements Pb, Cd, Ni, Fe and Al in the leaves of the species studied, as well as in the soil where the species were grown (Figures 2A and 2B). For instance, C. odorata leaves had a significantly lower Pb concentration than P. cecropiifolia leaves. However, C. odorata had the highest Cd concentration, while P. cecropiifolia had the lowest Cd content. As for Ni, P. multijuga had the lowest concentration with significant differences to C. ficifolia, which was the species with the highest value. Yet C. ficifolia showed the lowest Fe and Al concentrations, whilst I. edulis and C. odorata had the highest Fe and Al values respectively (Fig. 2A).

In general, the soil where the species grow showed significant differences in the concentration of heavy metals (Fig. 2B). The lowest Pb concentration was in the areas of I. edulis and C. ficifolia, whereas the highest concentration was in the soil around P. multijuga and C. odorata. Cd concentration was similar in the locations of C. odorata and P. cecropiifolia. Indeed, the lowest and highest values of Cd were similar for the rest of the soil where the other species were found. Moreover, the presence of Ni in the soil had significant differences with lower values in soil surrounding P. cecropiifolia than in soil around I. edulis. As for the presence of Fe, where C. odorata is present, it had a higher content with significant differences to C. ficifolia. Al was higher in the soil around P. cecropiifolia and lower in I. edulis areas.

According to Pearson’s correlation coefficient (Fig. 3), when analysing the concentration of metals in the leaves of the species studied, only a positive correlation (higher than 50%) was found between Al and Fe (r = 0.54). Meanwhile, other elements showed negative correlations: Cd and Pb (r = -0.79), Fe and Cd (r = -0.71) and Al and Pb (r = -0.69). Positive correlations higher than 50% between soil element concentrations were between Ni and Cd (r = 0.70), Fe and Pb (r = 0.62) and Al and Pb (r = 0.80), whereas Ni presented negative correlations with negative Pb, Al and Fe elements (r = -0.76, r = -0.77, r = -0.87), respectively.

Figure 3. Correlation between the concentration of heavy metals in the leaves of the analysed species and the soil (The numerical value corresponds to Pearson’s correlation coefficient).

The cluster analysis grouped all studied species into three groups based on the concentrations of heavy metals (Pb, Cd, Ni, Fe and Al) in leaves (Figure 4). Al concentrations in leaves of C. odorata and I. edulis formed one group; Fe concentrations in P. cecropiifolia, I. edulis, C. ficifolia, P. multijuga and C. odorata, and Al concentrations in P. cecropiifolia, C. ficifolia and P. multijuga formed the second group; while the third group included Cd, Ni and Pb concentrations.

Figure 4. Cluster analysis based on the concentrations of heavy metals (Pb, Cd, Ni, Fe and Al) in leaves of the studied species.

The bioconcentration factor was used to estimate the plants’ ability to absorb certain heavy metals from the soil. The BFC values can be seen in Figure 5. The species analysed differ significantly in their capacity to absorb the heavy metals studied, with P. cecropiifolia having the greatest capacity to retain Pb, C. odorata accumulating the greatest amount of Cd, and C. ficifolia and C. odorata absorbing the most Ni. As for Fe and Al, I. edulis had the highest bioaccumulation potential for both elements.

Figure 5. BCF results for the analysed tree species (The values within the same heavy metal followed by the same letter do not differ significantly according to Tukey’s test).

From the eigenvalues of the correlation matrix of the heavy metals studied and using the PCA method, the initial data set was reduced to two principal components that explained 70.6% of the variability (Figure 6). Based on the PCA analysis, Ni and Cd concentrations in the soil were lower than those of Fe, Pb and Al (PC 1).

Figure 6. PCA analysis of concentrations of heavy metals (Pb, Cd, Ni, Fe and Al) in the leaves and soil of C. odorata, P. multijuga, I. edulis, P. cecropiifolia and C. ficifolia.

When looking at the plants in the coordinate system, determined by the principal components, it can be noted that the PC 1 axis separates the species C. odorata, P. cecropiifolia and P. multijuga, which are on the positive side of the axis and had lower values of Cd and Ni in the soil, from I. edulis and C. ficifolia, which are on the negative side of the axis and are characterised by higher concentrations of Cd and Ni in the soil. The second axis, PC 2, separates the species C. odorata, on the positive side of the axis and with a lower Pb concentration, from the rest of the species.

- DISCUSSION

The leaf material of five forest species, C. odorata, P. multijuga, I. edulis, C. ficifolia and P. cecropiifolia, which commonly inhabit mining areas in the Ecuadorian Amazon region, along with samples of soil found close to each species under study were tested. The objective was to identify which species had the potential to bioaccumulate Pb, Cd, Ni, Fe and Al in order to mitigate possible contamination by any of these elements in Amazonian ecosystems or in soils of different uses, whether agricultural or forestry, in any region of the country where the species grow. A soil analysis between 0 and 30cm was used as an indicator of contamination, because according to some authors, the average maximum concentration is found in the soil layer within this depth range (Šichorová et al., 2004; Wu et al., 2013). Concentrations of heavy metals might be higher at a depth above 30cm, but their chemical forms differ with depth and therefore may not be readily available to plants (Wu et al., 2013). According to Ecuadorian regulations (MAE-TULSMA, 2015), it was found that Cd concentration exceeded the permissible values (0.5mg kg-1), Pb and Ni were below the established limits, while Fe and Al are not regulated. It is important to highlight the bioaccumulate potential of the species, regardless of whether the soil can be considered contaminated or not. If the plants are able to absorb any of the elements considered as contaminants, then this could be a viable option for the bioremediation of contaminated soils.

Previous studies have documented the heavy metal biosorption potential of C. odorata. For example, Akintola and Bodede, (2019) studied seedlings grown in landfill soils and deduced that heavy metal concentrations in contaminated soils indicated metal enrichment in plant tissues. Enrichment coefficients and distribution factors showed the potential of C. odorata as a bioaccumulator species. Thus, they concluded that seedlings of this species can be used to clean up or rehabilitate soils that are contaminated with the heavy metals studied (Cu, Pb, Zn and Co).

The Hg bioaccumulator potential of I. edulis has also been reported, as it has been used for phytoremediation in a mining site contaminated with this heavy metal (Marrugo-Negrete et al., 2016).

For the species P. multijuga, C. ficifolia and P. cecropiifolia, no studies have been reported on the absorption capacity of heavy metals, so this research reports for the first time the bioaccumulate potential of these species.

- CONCLUSION

The bioconcentration factor made it possible to identify the studied species’ (C. odorata, P. multijuga, I. edulis, P. cecropiifolia and C. ficifolia) capacity to bioaccumulate the heavy metals Pb, Cd, Ni, Fe and Al, making it possible to propose reforestation in areas contaminated with these elements to mitigate the negative impacts they may cause on the ecosystem. For Pb-contaminated soils, P. cecropiifolia is recommended as it showed the highest CBF-Pb value. Furthermore, the species C. odorata can be used in Cd and Ni contaminated sites, while I. edulis showed the highest potential for Fe and Al uptake.

As no correlation was found between the concentration of the element in the soil and the leaves, it can be asserted that absorption depends on the bioconcentration capacity of the species. It is therefore suggested that more forest species inhabiting the Ecuadorian Amazon region could be studied as well as other heavy metals in order to broaden the identification of potential species, depending on the pollutant.

COMPETING INTERESTS AND FUNDING

Author Contribution

- Yudel Gracía-Quintana, Luis Ramón Bravo-Sánchez, Yasiel Arteaga-Crespo: Conceived and designed the experiments; Analyzed and interpreted the data; Wrote the paper.

- Dixon Andi Grefa, Jorge de Jesús Guerrón-Torres, Jenny Elizabeth Rosero-Sanchez: Performed the experiments; Contributed reagents, materials, analysis tools or data.

All the authors read and approved the inal manuscript.

FUNDING

This work was supported by Universidad Estatal Amazónica. Ecuador.

CONLICTS OF INTEREST

The authors declare no competing interests.

DATA AVAILABILITY STATEMENTS

The datasets generated during and/or analysed during the current study are available from the corresponding author on reasonable request.

ACKNOWLEDGEMENTS

Thanks to the Universidad Estatal Amazónica for their financial support.

REFERENCES

Akintola O, Bodede IA. 2019. Distribution and accumulation of heavy metals in Red Cedar (Cedrela odorata) wood seedling grown in dumpsite soil. Journal of Applied Sciences and Environmental Management 23: 811. DOI: 10.4314/jasem.v23i5.6

Alahabadi A, Ehrampoush MH, Miri M, Ebrahimi Aval H, Yousefzadeh S, Ghaffari HR, Ahmadi E, Talebi P, Abaszadeh Fathabadi Z, Babai F, Nikoonahad A, Sharafi K, Hosseini-Bandegharaei A. 2017. A comparative study on capability of different tree species in accumulating heavy metals from soil and ambient air. Chemosphere 172: 459–467. DOI: 10.1016/j.chemosphere.2017.01.045

Carling GT, Diaz X, Ponce M, Perez L, Nasimba L, Pazmino E, Rudd A, Merugu S, Fernandez DP, Gale BK, Johnson WP. 2013. Particulate and dissolved trace element concentrations in three southern ecuador rivers impacted by artisanal gold mining. Water, Air, and Soil Pollution 224. DOI: 10.1007/s11270-012-1415-y

Çelik A, Kartal AA, Akdoǧan A, Kaska Y. 2005. Determining the heavy metal pollution in Denizli (Turkey) by using Robinio pseudo-acacia L. Environment International 31: 105–112. DOI: 10.1016/j.envint.2004.07.004

Garrett RG. 2000. Natural sources of metals to the environment. Human and Ecological Risk Assessment (HERA) 6: 945–963. DOI: 10.1080/10807030091124383

Greksa A, Ljevnaić-Mašić B, Grabić J, Benka P, Radonić V, Blagojević B, Sekulić M. 2019. Potential of urban trees for mitigating heavy metal pollution in the city of Novi Sad, Serbia. Environmental Monitoring and Assessment 191. DOI: 10.1007/s10661-019-7791-7

http://www.theplantlist.org/. (n.d.).

Hu Y, Wang D, Wei L, Zhang X, Song B. 2014. Bioaccumulation of heavy metals in plant leaves from Yan׳an city of the Loess Plateau, China. Ecotoxicology and Environmental Safety 110: 82–88. DOI: 10.1016/j.ecoenv.2014.08.021

Janhäll S. 2015. Review on urban vegetation and particle air pollution - Deposition and dispersion. Atmospheric Environment, 130–137. DOI: 10.1016/j.atmosenv.2015.01.052

Kołodyńska D, Krukowska J, Thomas P. 2017. Comparison of sorption and desorption studies of heavy metal ions from biochar and commercial active carbon. Chemical Engineering Journal 307: 353–363. DOI: 10.1016/j.cej.2016.08.088

Kulkarni R, Deobagkar D, Zinjarde S. 2018. Metals in mangrove ecosystems and associated biota: A global perspective. Ecotoxicology and Environmental Safety, 215–228. DOI: 10.1016/j.ecoenv.2018.02.021

Latif A, Bilal M, Asghar W, Azeem M, Ahmad MI, Abbas A, Zulfiqar Ahmad M, Shahzad T. 2018. Heavy Metal Accumulation in Vegetables and Assessment of their Potential Health Risk. Journal of Environmental Analytical Chemistry 05. DOI: 10.4172/2380-2391.1000234

Lazo P, Stafilov T, Qarri F, Allajbeu S, Bekteshi L, Frontasyeva M, Harmens H. 2019. Spatial distribution and temporal trend of airborne trace metal deposition in Albania studied by moss biomonitoring. Ecological Indicators 101: 1007–1017. DOI: 10.1016/j.ecolind.2018.11.053

Li X, Li Z, Lin CJ, Bi X, Liu J, Feng X, Zhang H, Chen J, Wu T. 2018. Health risks of heavy metal exposure through vegetable consumption near a large-scale Pb/Zn smelter in central China. Ecotoxicology and Environmental Safety 161: 99–110. DOI: 10.1016/j.ecoenv.2018.05.080

Liang J, Fang HL, Zhang TL, Wang XX, Liu YD. 2017. Heavy metal in leaves of twelve plant species from seven different areas in Shanghai, China. Urban Forestry and Urban Greening 27: 390–398. DOI: 10.1016/j.ufug.2017.03.006

MAE-TULSMA. 2015. Texto Unificado de Legislación Secundaria Medio Ambiental. Ministerio de Ambiente de Ecuador. Quito.

Mainville N, Webb J, Lucotte M, Davidson R, Betancourt O, Cueva E, Mergler D. 2006. Decrease of soil fertility and release of mercury following deforestation in the Andean Amazon, Napo River Valley, Ecuador. Science of the Total Environment 368: 88–98. DOI: 10.1016/j.scitotenv.2005.09.064

Marrugo-Negrete J, Marrugo-Madrid S, Pinedo-Hernández J, Durango-Hernández J, Díez S. 2016. Screening of native plant species for phytoremediation potential at a Hg-contaminated mining site. Science of the Total Environment 542: 809–816. DOI: 10.1016/j.scitotenv.2015.10.117

Monfared SH, Matinizadeh M, Shirvany A, Amiri GZ, Fard RM, Rostami F. 2013. Accumulation of heavy metal in Platanus orientalis, Robinia pseudoacacia and Fraxinus rotundifolia. Journal of Forestry Research 24: 391–395. DOI: 10.1007/s11676-012-0313-x

Nagajyoti PC, Lee KD, Sreekanth TVM. 2010. Heavy metals, occurrence and toxicity for plants: A review. Environmental Chemistry Letters, 199–216. DOI: 10.1007/s10311-010-0297-8

Nakazawa K, Nagafuchi O, Kawakami T, Inoue T, Yokota K, Serikawa Y, Basir-Cyio M, Elvince R. 2016. Human health risk assessment of mercury vapor around artisanal small-scale gold mining area, Palu city, Central Sulawesi, Indonesia. Ecotoxicology and Environmental Safety 124: 155–162. DOI: 10.1016/j.ecoenv.2015.09.042

Neumann D. 2000. Heavy Metal Stress in Plants: From Molecules to Ecosystems. Phytochemistry 53: 822. DOI: 10.1016/s0031-9422(00)00010-8

Pajević S, Borišev M, Nikolić N, Arsenov DD, Orlović S, Župunski M. 2016. Phytoextraction of heavy metals by fast-growing trees: A review. Phytoremediation: Management of Environmental Contaminants, Volume 3. Phytoremediation, 29–64. DOI: 10.1007/978-3-319-40148-5_2

Palansooriya KN, Shaheen SM, Chen SS, Tsang DCW, Hashimoto Y, Hou D, Bolan NS, Rinklebe J, Ok YS. 2020. Soil amendments for immobilization of potentially toxic elements in contaminated soils: A critical review. Environment International. DOI: 10.1016/j.envint.2019.105046

Panday PK, Coe MT, Macedo MN, Lefebvre P, Castanho AD d. A. 2015. Deforestation offsets water balance changes due to climate variability in the Xingu River in eastern Amazonia. Journal of Hydrology 523: 822–829. DOI: 10.1016/j.jhydrol.2015.02.018

Peña-Carpio E, Menéndez-Aguado JM. 2016. Environmental study of gold mining tailings in the Ponce Enriquez mining area (Ecuador). DYNA (Colombia) 83: 237–245. DOI: 10.15446/dyna.v83n195.51745

Pimm SL, Jenkins CN, Abell R, Brooks TM, Gittleman JL, Joppa LN, Raven PH, Roberts CM, Sexton JO. 2014. The biodiversity of species and their rates of extinction, distribution, and protection. Science 344. DOI: 10.1126/SCIENCE.1246752

Ratul AK, Hassan M, Uddin MK, Sultana MS, Akbor MA, Ahsan MA. 2018. Potential health risk of heavy metals accumulation in vegetables irrigated with polluted river water. International Food Research Journal 25: 329–338

Rehman K, Fatima F, Waheed I, Akash MSH. 2018. Prevalence of exposure of heavy metals and their impact on health consequences. Journal of Cellular Biochemistry 119: 157–184. DOI: 10.1002/jcb.26234

Sawidis T, Krystallidis P, Veros D, Chettri M. 2012. A study of air pollution with heavy metals in Athens city and Attica basin using evergreen trees as biological indicators. Biological Trace Element Research 148: 396–408. DOI: 10.1007/s12011-012-9378-9

Serbula SM, Kalinovic TS, Ilic AA, Kalinovic J V., Steharnik MM. 2013. Assessment of airborne heavy metal pollution using Pinus spp. and Tilia spp. Aerosol and Air Quality Research 13: 563–573. DOI: 10.4209/aaqr.2012.06.0153

Šichorová K, Tlustoš P, Száková J, Kořínek K, Balík J. 2004. Horizontal and vertical variability of heavy metals in the soil of a polluted area. Plant, Soil and Environment 50: 525–534. DOI: 10.17221/4069-pse

Ŝućur KM, Aniić MP, Tomaŝević MN, Antanasijević DZ, Perić-Grujić AA, Ristić MDJ. 2010. Urban deciduous tree leaves as biomonitors of trace element (As, v and Cd) atmospheric pollution in Belgrade, Serbia. Journal of the Serbian Chemical Society 75: 1453–1461. DOI: 10.2298/JSC100319079S

Suzuki K, Yabuki T, Ono Y. 2009. Roadside Rhododendron pulchrum leaves as bioindicators of heavy metal pollution in traffic areas of Okayama, Japan. Environmental Monitoring and Assessment 149: 133–141. DOI: 10.1007/S10661-008-0188-7

Szymanowska A, Samecka-Cymerman A, Kempers AJ. 1999. Heavy metals in three lakes in West Poland. Ecotoxicology and Environmental Safety 43: 21–29. DOI: 10.1006/eesa.1998.1747

Tarras-Wahlber NH, Flachier A, Fredriksson G, Lane S, Lundberg B, Sangfors O. 2000. Environmental impact of small-scale and artisanal gold mining in southern Ecuador: Implications for the setting of environmental standards and for the management of small-scale mining operations. Ambio 29: 484–491. DOI: 10.1579/0044-7447-29.8.484

Tomašević M, Rajšić S, Dordević D, Tasić M, Krstić J, Novaković V. 2004. Heavy metals accumulation in tree leaves from urban areas. Environmental Chemistry Letters 2: 151–154. DOI: 10.1007/s10311-004-0081-8

Tóth G, Hermann T, Da Silva MR, Montanarella L. 2016. Heavy metals in agricultural soils of the European Union with implications for food safety. Environment International 88: 299–309. DOI: 10.1016/j.envint.2015.12.017

Tyler G. 2005. Changes in the concentrations of major, minor and rare-earth elements during leaf senescence and decomposition in a Fagus sylvatica forest. Forest Ecology and Management 206: 167–177. DOI: 10.1016/j.foreco.2004.10.065

Ugolini F, Tognetti R, Raschi A, Bacci L. 2013. Quercus ilex L. as bioaccumulator for heavy metals in urban areas: Effectiveness of leaf washing with distilled water and considerations on the trees distance from traffic. Urban Forestry and Urban Greening 12: 576–584. DOI: 10.1016/j.ufug.2013.05.007

Wang Z, Liu X, Qin H. 2019. Bioconcentration and translocation of heavy metals in the soil-plants system in Machangqing copper mine, Yunnan Province, China. Journal of Geochemical Exploration 200: 159–166. DOI: 10.1016/j.gexplo.2019.02.005

Wu Y, Peng X, Hu X. 2013. Vertical distribution of heavy metal in soil of abandoned vehicles dismantling area. Asian Journal of Chemistry 25: 8423–8426. DOI: 10.14233/ajchem.2013.14770

Wuana RA, Okieimen FE. 2011. Heavy Metals in Contaminated Soils: A Review of Sources, Chemistry, Risks and Best Available Strategies for Remediation. ISRN Ecology 2011: 1–20. DOI: 10.5402/2011/402647

Xiao R, Wang S, Li R, Wang JJ, Zhang Z. 2017. Soil heavy metal contamination and health risks associated with artisanal gold mining in Tongguan, Shaanxi, China. Ecotoxicology and Environmental Safety 141: 17–24. DOI: 10.1016/j.ecoenv.2017.03.002