An LQR-Lagrange Algorithm Generated by Interdisciplinary Integration with Optimal Control and Optimization

Molin Ana, Xueshan Hana,*

aSchool of Electrical Engineering, Shandong University, No.17923 Jingshi Road, Jinan, 250061, China

* Corresponding author

E-mails: molin_an@mail.sdu.edu.cn, xshan@sdu.edu.cn

Running title:  LQR-Lagrange Algorithm

Abstract:  Interdisciplinary integration is a superior method to improve the optimization algorithm. In this paper, control theory and optimization are combined, and the optimization algorithm is regarded as a control process. Based on the premise of optimal control, the state equation corresponding to Lagrange Algorithm is established with the Karush-Kuhn-Tucker (KKT) conditions as the objective. As an optimal control method, Linear Quadratic Regulator (LQR) is utilized to control the calculation process, and an innovative LQR-Lagrange Algorithm is proposed. The Lyapunov stability criterion is applied to analyze the convergence, and it is proved that the proposed LQR-Lagrange Algorithm is bound to converge as long as the parameter matrices   and   are positive definite. The analysis indicates that the influence of parameters in LQR-Lagrange Algorithm on the calculation speed is monotonic, and the elements in   and   has no effect on the convergence. Therefore, the proposed algorithm has a monotonic and user-friendly parameter tuning strategy. The significance and advantages of interdisciplinary integration with control theory and optimization are discussed in the end.

Keywords:  Interdisciplinary integration, Optimization algorithm, Optimal control, LQR-Lagrange Algorithm, Monotonic and user-friendly.

Author contributions:  Molin An: Methodology; Software; Writing – original draft. Xueshan Han: Writing – review & editing.

Acknowledgments:  The authors are grateful for the valuable suggestions provided by the anonymous reviewers. Additionally, they appreciate the hard work of the editors of the OCAM journal in handling this paper..

Funding information:  None.

Conflict of interest statement:  The authors have no conflicts of interest to declare.

- Introduction

Optimization algorithms are important in engineering applications, and the iterative optimization algorithm is widely used in various fields. Lagrange Algorithm is a typical one which is often applied in engineering.1 Many algorithms are based on the improved Lagrange Algorithm, such as Regularized Lagrange Algorithm2 and Alternating Direction Method of Multipliers3. In power systems, Lagrange-based algorithms are widely used in distributed optimization problems.1,4 However, the parameter tuning has always been a difficult problem in engineering applications. Especially, in the time-varying optimization in power systems, the algorithm needs to be faster to compute to the optimal point in limited time.4 Therefore, the improvement of the Lagrange Algorithm is researched in this paper as a basis for the performance improvement in engineering applications.

One typical way to improve an algorithm is interdisciplinary integration, which has a long history. Biology has always been a hot topic. In 1975, J. H. Holland first regarded the calculation process of the optimization algorithm as a process of natural selection and genetics, and innovatively proposed Genetic Algorithm.5 It motivates researchers to propose more advanced algorithms by comparing the optimization algorithm to the biological process, such as Particle Swarm Optimization and Ant Colony Optimization. Since then, biology-based optimization algorithms are widely used.

As another fundamental discipline, the research of control theory applied to optimization algorithms has not been fully and comprehensively studied. There is a long and rich history of using ordinary differential equations to explain, solve or improve optimization algorithms,6-7 but few studies have linked it to control models. In 1997, B. S. GoH formulated the problem of computing the minimum point of an unconstrained function as a sequence of optimal control problems to achieve the long-term optimal during the calculation process.8 Inspired by this study, some researchers have carried out extensive research, introducing related concepts from control theory and dynamical systems into the study of optimization algorithms. For example, the Lyapunov function is used to analyze Momentum Algorithm,9 and the dissipativity theory is applied to study Nesterov’s Accelerated Gradient Algorithm.10 Further, I. M. Ross adopts the optimal control method to solve the nonlinear optimization problem,11 and proposes a derivation of Nesterov’s Accelerated Gradient Algorithm.12 However, the above studies do not give a clear parameter tuning strategy for the algorithm. Although some studies have given the selection of the optimal control trajectory,11 it is still complicated and difficult for users to select the parameters when using the algorithm.

The improvement of the gradient algorithm includes the design of the direction of descent and the step, and the parameter tuning also plays an important role. In the classic gradient method, the influence of the step coefficient on convergence and calculation speed is not monotonic. Therefore, it is difficult to find the best parameters. In addition, the steepest descent method is commonly used,13 but the calculation of the step coefficient is complicated, and its convergence speed is slow in some special cases.14 Nesterov’s Accelerated Gradient Algorithm has garnered interest in applications because of its fast convergence rate.15 Some researches view Nesterov’s technique and many of its generalizations in a systematic way,16 but its original convergence rate is derived by estimate sequences, which has proven difficult to interpret.10 The above studies have improved the direction of gradient descent, but these methods still have limitations in the parameter tuning. For example, most researchers believe that the hyperparameter  in Momentum Algorithm and Nesterov’s Accelerated Gradient Algorithm is generally chosen to be 0.9. However, this is still only a recommended value rather than an optimal choice, which will be illustrated in this paper later. Although the algorithm performance has been improved, there is still a lack of a clear strategy for parameter tuning, which makes it a difficult problem in engineering applications. The above discussion reminds us to propose monotonic and user-friendly parameter tuning strategies.

In control theory, some methods have clear parameter meanings which is helpful. Therefore, control theory is selected to improve the optimization algorithm. In control theory, PID control is an extensively used controller. The parameters P, I, and D have clear meanings in control theory representing proportion, integration and differentiation respectively. Some studies examine the calculation process of numerical integration by control principles and use PI control to improve the performance.17 PID controller is also used to improve optimization algorithms.18-19 But it is well known that the parameter tuning of PID control is complicated and depends on the experience. Therefore, the traditional PID control has limitations in the improvement of the algorithm. In control theory, optimal control seeks the optimal control strategy under certain constraint conditions to make the performance index the best, and it is widely applied in industrial applications,20 such as power systems21 and the vehicle control22. In modern control theory, LQR is the earliest and most mature optimal control method, which is applied in the field of electronics23, helicopter science24, and robotics25. The advantage of LQR is that the parameter tuning strategy is relatively simple, and the performance can be better only by monotonically adjusting  and . Therefore, LQR is applied to improve Lagrange Algorithm in this paper, and an innovative LQR-Lagrange Algorithm is proposed.

The following researches are carried out. The iterative optimization algorithm is regarded as a control process from the perspective of control theory. In order to meet the premise that the stable state is the origin O when using LQR, we rewrite Lagrange Algorithm with the KKT conditions as the objectives. Then, the state equation of the control system corresponding to Lagrange Algorithm is proposed, in which the state variable is the gradient. Furthermore, LQR is applied to control the calculation process and 4 an innovative LQR-Lagrange Algorithm is proposed. Since LQR-Lagrange Algorithm is regarded as a control process, the convergence is judged by the stability of the control system. The Lyapunov stability criterion is used to prove the convergence. The good performance of LQR-Lagrange Algorithm is illustrated with calculation examples. In addition, the monotonic and user-friendly parameter tuning strategy are proposed based on the principle of LQR. At the end, the expansion of the research ideas of this paper are further discussed.

The contributions of this paper are highlighted as follows:

1. Development of LQR-Lagrange Algorithm. The iterative optimization algorithm is regarded as a control process. Based on the KKT conditions, the state equation of the control system corresponding to Lagrange Algorithm is proposed. We apply LQR to control the calculation process, and propose an innovative LQR-Lagrange Algorithm.

2. Proof of explicit correlation between the convergence of the proposed algorithm and the parameters. The convergence of LQR-Lagrange Algorithm is proved by the Lyapunov stability criterion with the condition that  and  is positive definite. Therefore, the initial point and the value of  and  do not affect the convergence of the proposed algorithm, which is helpful for researching the monotonic and user-friendly parameter tuning strategy.

3. The monotonic and user-friendly parameter tuning strategy of LQR-Lagrange Algorithm. The influence of parameters  and  in LQR-Lagrange Algorithm on the calculation performance is analyzed. The parameter tuning strategy is explained with the principle of LQR. Moreover, the analysis shows that the correlation between the parameter and the calculation speed is monotonic.

The rest of paper is organized as follows. In Section 2, the development of LQR-Lagrange Algorithm is introduced. The calculation performance analysis of LQR-Lagrange Algorithm is carried out in Section 3, in which the Lyapunov stability criterion is used to prove the convergence, and the calculation example shows its good performance. Further, the monotonic and user-friendly parameter tuning strategy is also proposed in this section. Section 4 comes to a conclusion.

- Control Model of LQR-Lagrange Algorithm

  - Lagrange Algorithm

Consider an optimization problem with equality constraints and inequality constraints:

()

()

()

Assumption 1:  is the objective function,  is the equality constraint which is affine as  with constant matrix  and  , and  is the inequality constraint which is convex as  with constant matrix  and . Assume that both , , and  are continuously differentiable, and there is a unique optimal point exists for problem .

Let  denote the Lagrange multiplier associated with the equality constraint , and  denote the Lagrange multiplier associated with the inequality constraint .  and  are the vectors of  and .

The Lagrange function is given as:

()

According to Lagrange Algorithm:

()

Based on Equation (5), Lagrange Algorithm is derived:

()

()

()

where ,  and  are step coefficients,  represents the projection onto the nonnegative orthant.

Lagrange Algorithm come into widespread use because of the simple iterative format. However, the parameter tuning of Lagrange Algorithm is difficult, because the influence of parameters on the calculation speed is not monotonic. A large step coefficient will cause the iterative calculation divergence, and a small step coefficient will make the convergence speed slow. Moreover, the interaction between the three step coefficients ,  and  makes it difficult to find the optimal step coefficients.

In general, the calculation speed, convergence and parameter tuning strategy are important. In the following Sections, Lagrange Algorithm is improved from the perspective of control theory for the reasons mentioned in the introduction.

  - The Relationship between Control Theory and Optimization

Control theory mainly studies the method of controlling the process of achieving a certain goal. The optimization calculation is regarded as a control process that starts from the initial value and iteratively calculates to find the optimal value. Therefore, this calculation process can be controlled with a suitable control method.

Lagrange Algorithm is the Difference Equation, which is similar to the state equation in control theory. It inspires us to adopt the control method to improve Lagrange Algorithm.

It should be noted that the stable state of the control system is the origin O in optimal control. Therefore, the premise that we use optimal control to improve Lagrange Algorithm is that the stable state of the calculation process of Lagrange Algorithm is the origin O. In other words, the optimal value of ,  and   are 0, which is obviously not in line with reality. Therefore, Lagrange Algorithm need to be adjusted to meet the premise of using optimal control.

The KKT conditions characterize the optimal value, and the gradient objective of the KKT conditions is 0. It is consistent with the requirement of the stable state in optimal control. Therefore, Lagrange Algorithm is rewritten as a state equation.

First, define the gradient in Lagrange Algorithm as the state variables in the state equation:

()

Then, the state equation is expressed as:

()

where  and  are square coefficient matrices of dimension . The specific format will be explained later with calculation examples.

According to Lagrange Algorithm, there is a restriction:

()

In actual optimization problems, the gradient of  may not converge to 0, which is related to whether the optimal point is on the inequality constraint. And the calculation of the optimization problem with inequality constraints is a multi-stage process, therefore it will be discussed later in Section 3.5.

Based on the state equation, we use the optimal control to control this calculation process, in order to improve the speed and propose a monotonic and user-friendly parameter tuning strategy.

  - LQR-Lagrange Algorithm

LQR is used to control the calculation process of Lagrange algorithm. In order to achieve the best calculation performance, the following quadratic performance index is used:

()

where  and  are square matrices of dimension .

and  respectively represent the penalty of the state variable and the energy consumption of control in the performance index. In classic LQR control,  is a positive semi-definite symmetric constant matrix, and  is a positive definite symmetric constant matrix. But  is defined as positive definite in this paper, and the reason is as follow. Minimizing the performance index  during the calculation process means that the state variables must reach the stable state as quickly as possible and consume less energy consumption of control. The state variable in the performance index  is the gradient, so  influence the convergence speed. The purpose of improving the optimization algorithm by using control theory is to make the algorithm converge to the optimal point quickly and accurately, so every gradient of the variable must be controlled. Therefore,  is a positive definite symmetric constant matrix.

The only optimal control  is obtained:

()

where  is a positive definite constant matrix satisfying Discrete-time Algebraic Riccati Equation (DARE):

()

Therefore, LQR-Lagrange Algorithm is proposed by applying the optimal control :

()

The calculation result of Equation (15) is the gradient, and it is converted to the iterative trajectory of variables:

()

where  is the coefficient matrix, and  is the zero matrix. The specific format of  will be explained by a calculation example later.

The following formula is supplemented to correspond to the restriction condition Equation (11):

()

- Performance Analysis

  - Convergence and Stability

The iterative calculation of the optimization algorithm is regarded as a control process in this paper. Therefore, the convergence of LQR-Lagrange Algorithm is judged by the stability analysis of the corresponding control system.

First, the consistency of the algorithm convergence and the control system stability is explained:

The Global Convergence of the optimization algorithm is defined as: the calculation sequence of an optimization algorithm {} converges to the optimal point for any initial iterate.26

The Asymptotic Stability in the Large of the control system is defined as: every solution converges to the equilibrium state  for every trajectory starting from anywhere in the state space.27

The Asymptotic Stability in the Large is used to explain the relationship between the stability and the convergence. For any initial value (disturbance), the iterative calculation result (trajectory) will eventually converge to the optimal value (equilibrium state).

Therefore, the Lyapunov stability criterion is used to prove the convergence of LQR-Lagrange Algorithm. Assume that the control system represented by Equation (15) is a discrete linear time-invariable system. The following Lyapunov function verifies the convergence of LQR-Lagrange Algorithm:

()

Conclusion 1: The Lyapunov function Equation (18) is positive definite, because  is a positive definite constant matrix that satisfies DARE.

()

The middle item is rewritten next:

()

Substituting the optimal feedback matrix  Equation (13) and DARE Equation (14) into the first three items:

()

Substituting the optimal feedback matrix  Equation (13) into the last two items:

()

Therefore, it can be summarized as:

()

Conclusion 2:  is negative definite because  and  are both positive definite as defined previously.

Conclusion 3: When , .

According to Conclusions 1-3, the control system is Asymptotic Stability in the Large. Therefore, LQR-Lagrange Algorithm is Global Convergence.

In traditional methods, the correlation between parameter and convergence is not clear. For example, parameters, the Lipschitz constant, and convergence are related in some methods, but it is difficult to set the parameters according to the Lipschitz constant. In other methods, ranges or suggested values of parameters are given to ensure convergence, or parameter tuning relies on experiences. Differently, the proposed LQR-Lagrange Algorithm is more advanced because the algorithm must converge as long as  and  are positive definite. Such an explicit correlation makes it easier to tune the parameters according to the principles of LQR controller, which will be discussed in depth in Section 3.4.

  - Performance Analysis

In the following discussion, the proposed algorithm is performed to calculate the quadratic optimization problem as an example for establishing the detailed state equation and analyzing the performance.

The quadratic optimization problem is expressed as:

()

()

()

where  is the variable matrix,  is the positive definite matrix corresponding to the quadratic term, and  is the coefficient matrix corresponding to the linear term. The form of the equality constraints and the inequality constraints are the same as those described in Assumption 1.

is used to establishing the detailed format of the state equation.

Define a diagonal matrix:

()

Then, the parameters in LQR-Lagrange Algorithm Equations (15) and (16) are:

()

()

()

where  represents the identity matrix and  represents the zero matrix.

In Equations (28)-(30), the step coefficients in Lagrange Algorithm are set to 1. The detailed reason is explained later. In LQR-Lagrange Algorithm, the convergence is only related to whether  and   are positive definite. It helps us to propose a monotonic and user-friendly parameter tuning strategy of LQR-Lagrange Algorithm, which is discussed in Section 3.4.

Since the calculation of optimization problems under inequality constraints is a multi-stage process, the optimization problems with equality constraints is only taken as an example for performance analysis. The analysis of optimization problems with inequality constraints is discussed later in Section 3.5.

()

()

The initial point is . The optimal point is  . The condition for the convergence is , .

We adopt Lagrange Algorithm Equations (6)-(8), Nesterov’s Accelerated Gradient Algorithm28, Momentum Algorithm28 and LQR-Lagrange Algorithm Equations (13)-(16) to calculate , and verify the good performance of the proposed algorithm.

The calculation performance of the first three algorithms depends on the selection of the step coefficients. For this reason, the optimal parameters of the first three algorithms are obtained by traversing over all parameters with interval 0.01 from 0.01 to 0.99. In order to make the algorithm perform better, the parameter  in Nesterov’s Accelerated Gradient Algorithm and Momentum Algorithm is set to 0.5. Because when  is set to the recommended value 0.9 generally, the convergence speed is too slow or does not converge in . The selected optimal parameters of the above three algorithms are listed in Table 1.

Table 1

Parameters and results in the calculation example.

| Algorithm | Parameters | Iterations at convergence |
| --- | --- | --- |
| Lagrange Algorithm |  | 13 |
| Momentum Algorithm |  | 17 |
| Nesterov’s Accelerated Gradient Algorithm |  | 11 |
| LQR- Lagrange Algorithm |  | 5 |

In LQR-Lagrange Algorithm, the calculation performance is only related to  and . According to the theory of LQR,  and   respectively represent the penalty for the state variable (gradient) and the energy consumption of control in the calculation process. We need to focus on penalizing the gradient of the variable to make the gradient converge to 0 more fast. Therefore, there is no clear requirement for the punishment of the energy consumption of control. However,  is a positive definite matrix and cannot be a zero matrix in LQR control. For this reason, we consider setting  as a positive definite diagonal matrix with a small value. Therefore, the parameters of LQR-Lagrange Algorithm are set as .

is calculated using the optimal parameters of each algorithm in Table 1. The calculation results are illustrated in Figure 1.

Figure 1

The calculation results by Lagrange Algorithm (Lag), Momentum Algorithm (Mom), Nesterov’s Accelerated Gradient Algorithm (Nes), and LQR-Lagrange Algorithm (LQR-Lag).

In Figure 1, LQR-Lagrange Algorithm converges after only 5 iterations, which shows that LQR-Lagrange Algorithm calculates fast. In the calculation example, we traverse over all parameters to find the optimal ones in Lagrange Algorithm Equations (6)-(8), Nesterov’s Accelerated Gradient Algorithm28 and Momentum Algorithm28, but this is not feasible in actual calculation because the relationship between the parameters and convergence speed is non-monotonic, which will be illustrated in Section 3.4. In addition, the calculation performance of these three methods is also affected by the gradient characteristics of the optimization problem and the position of the initial point, which brings difficulties to the optimal parameter selection.

In contrast, LQR-Lagrange Algorithm has a fast convergence speed with a simple, monotonic, and user-friendly parameter tuning strategy. The discussion of the parameter tuning strategy of the proposed algorithm will be carried out also in Section 3.4. In the next Section, we will continue to analyze the reasons for the fast calculation of LQR-Lagrange Algorithm, and introduce its features and advantages.

  - Gradient and Step in LQR-Lagrange Algorithm

The core idea of LQR-Lagrange Algorithm is to use LQR to control the calculation process and make the gradient approach 0 as quickly as possible in order to converge to the optimal value. We still use problem  and the parameter of LQR-Lagrange Algorithm in Table 1 to analyze the gradient and step in the calculation process of LQR-Lagrange Algorithm. The results are shown in Figure 2 and Figure 3.

Figure 2

The iteration of the gradient in LQR-Lagrange Algorithm.

In Figure 2, each gradient quickly approaches 0, and the curves are relatively smooth and do not produce a large overshoot. Because the value of   corresponding to the gradient is set to be larger than .

The step in LQR-Lagrange Algorithm is analyzed then. Although there is no real step coefficient in Equation (15) of LQR-Lagrange Algorithm, it can still be deduced which is called the step along the gradient direction. The result is shown in Figure 3.

Figure 3

The step along the gradient direction in LQR-Lagrange Algorithm.

In Figure 3, the step along the gradient direction during the calculation is different at each iteration, which means that LQR-Lagrange Algorithm is actually a self-adaptive gradient method of variable steps. It is worth noting that the step along the gradient direction is even negative in Figure 3. Generally, the step coefficient in the gradient descent method is positive, that is, the direction of the calculation is always maintained as the gradient descent. However, LQR-Lagrange Algorithm has a negative step sometimes. It means that under the premise of minimizing the performance index , the direction of calculation may not always be gradient descent. At some iterations during the calculation, a negative step coefficient can make iterative calculations converge more quickly. The variable and negative step along the gradient direction of LQR-Lagrange Algorithm are its unique characteristics.

Although LQR-Lagrange Algorithm has the characteristics of gradient descent with variable step, it still cannot provide a priori the tuning strategy of variable step coefficient in the gradient method. However, LQR-Lagrange Algorithm, as an algorithm without step coefficients, can achieve the best performance in the entire iterative calculation by minimizing the performance index . This is the most prominent advantage of LQR-Lagrange Algorithm.

The essence of LQR-Lagrange Algorithm is a self-adaptive gradient method of variable step, but when deriving Equations (28)-(30), it has been mentioned that we defaulted the step coefficients of Lagrange Algorithm to 1. Because from the perspective of control theory, LQR is a kind of feedback control. LQR changes the eigenvalues of the control system by feeding back the state variable. It makes the control system have more excellent performance under the requirement of minimizing the performance index . Feedback control changes the original structure of the control system. From a mathematical point of view, this is precisely the change from the constant step algorithm to the variable step algorithm. Therefore, regardless of the step coefficients of Lagrange Algorithm is set, LQR-Lagrange Algorithm fed back by LQR can iterate the most suitable distance along the gradient direction (decent or even ascent) every time. Therefore, in order to make Equations (28)-(30) simpler, it is feasible to set the step coefficients to 1 in Lagrange Algorithm.

In addition, if the step coefficients are all set to 1, Lagrange Algorithm will not converge when calculating . In this case, the eigenvalues of the control system Equation (10) are -9.0213, -5.0542, -2.2454, 3.3209. Obviously, the four eigenvalues are not in the unit open disk, which means that the system is unstable. Therefore, the calculation does not converge when using Lagrange Algorithm with such coefficient setting. But when applying LQR-Lagrange Algorithm to calculate, the system matrix with feedback is.The eigenvalues of the control system are 0.0493, -0.0063, -0.0017, -0.0030 with the parameter of LQR-Lagrange Algorithm in Table 1. These four eigenvalues are all in the unit open disk, which shows that the system is stable. Therefore, even if the control system of Lagrange Algorithm is unstable, the control system of LQR-Lagrange Algorithm controlled by LQR is stable.

In general, LQR-Lagrange Algorithm can rapidly converge and does not have complicated step coefficients. It avoids the difficulty of parameter selections in Lagrange Algorithm, and performs optimal feedback control from the perspective of control theory to achieve the best calculation performance.

  - Parameter Tuning Strategy

In LQR-Lagrange Algorithm,  and  in the performance index  are two parameters that determine the calculation performance. We discuss and analyze the tuning strategies of these two parameters.

The first item of  represents the penalty for the gradient. The objective of the gradient in LQR-Lagrange Algorithm is 0, so when  is larger relative to , the gradient must approach 0 faster in order to minimize the performance index .

The second item of  represents the penalty for the energy consumption of control. The optimal control  obtained by LQR control is . In control theory, the second item of  is to reduce the energy consumption in the control process as much as possible. In this paper, the iterative calculation process of the optimization problem is regarded as a control process, hence there is no real optimal control  and its energy consumption. But  and  are still important factors that influence the calculation performance.

is still used to analyze the influence of  and on the calculation performance. Since the value of the matrices  and  cannot be directly measured, in order to compare the relative size of  and  more conveniently,  and  are expressed as:

()

and  indicate the relative value of  and . We set , and set  as 0.01, 1 and 100. Then, LQR-Lagrange Algorithm are used to calculate  to compare the calculation performance. The initial point and convergence conditions are the same as before. The result is shown in Figure 4.

Figure 4

The influence of the relative values of  and on the calculation by LQR-Lagrange Algorithm.

In Figure 4, iterations at convergence decreases as  increases, which is consistent with the previous analysis.

In the following analysis, we set , and increase  from 0.001 to 1000, then use LQR-Lagrange Algorithm to calculate  .The initial point and convergence conditions are the same as before. The results are illustrated in Figure 5, Figure 6 and Figure 9.

First, in order to better illustrate the influence of the parameter tuning on the calculation performance, there is the following definitions.

Define the summation of the gradient in the calculation process of LQR-Lagrange Algorithm as:

()

where  represents the number of iterations at convergence condition.

Define the summation of energy consumption of control in the calculation process using LQR-Lagrange Algorithm as:

()

where .

The coefficients in  and  are the identity matrix  instead of  and  because it is necessary to avoid the influence of the value in  and  on the summation of the gradient and energy consumption of control in comparison.

Figure 5

The influence of  on  and .

In Figure 5, the summation of the gradients  decreases when  is larger, which means that LQR-Lagrange Algorithm converges faster. When  is larger, the summation of control  decreases. These results are consistent with theoretical analysis.

Then, we use two more intuitive indicators to analyze the influence of the size of  and  on calculation performance. These two indicators are the calculation speed (iterations at convergence) and the overshoot during the calculation process.

Figure 6

The influence of  on iterations at convergence.

In Figure 6, the calculation speed is significantly increased with the continuous increase of . In addition, when  is too small or too large than , the calculation speed of LQR-Lagrange no longer changes. Moreover, the tuning strategy of parameters  and  in LQR-Lagrange Algorithm do not change with the dimension of the problem. It solves the difficulties in Lagrange Algorithm that each variable and each Lagrange multiplier has its own step coefficient. Therefore, the more complicated the optimization problem is, the more step coefficients need to be set and adjusted in Lagrange Algorithm. Furthermore, the influence of each step coefficient in Lagrange Algorithm on the calculation speed is not monotonic as is mentioned previously, which brings great difficulties to search the optimal parameters. Momentum Algorithm also has the same weakness.

Lagrange Algorithm and Momentum Algorithm are used to calculate  to analyze the influence of each step coefficient on the calculation speed, which is illustrated in Figure 7 and Figure 8. The initial point and convergence conditions are the same as before. We first choose a basic parameter, and then only change one step coefficient at one time while maintaining others the basic value unchanged. The basic parameters are all set to 0.15 in Lagrange Algorithm, and are the optimal parameters in Table 1 in Momentum Algorithm.

Figure 7

The influence of each step coefficient in Lagrange Algorithm on iterations at convergence.

Figure 8

The influence of each step coefficient in Momentum Algorithm on iterations at convergence.

In Figure 7 and Figure 8, the influence of each step coefficient on the calculation speed is not monotonic and similar to quadratic in both algorithms. Therefore, it is even more difficult to find the optimal parameter combination under the mutual influence of parameters. The comparison of these three methods in Figure 6, Figure 7, and Figure 8 highlights that the parameter tuning strategy of LQR-Lagrange Algorithm is monotonic and user-friendly.

The influence of  on the overshoot in calculation is analyzed next. There are the following definitions:

Opposite direction: The direction in which a certain variable passes through the optimal value from the initial value (or the initial value of the iteration happens to be the optimal value) and keeps moving away from the optimal value.

Overshoot in calculation: The maximum deviation of the calculation result from the optimal value when a variable is iteratively calculated in the opposite direction during the calculation process.

Figure 9

The influence of  on the total overshoot in calculation by LQR-Lagrange Algorithm.

Figure 9 shows the influence of  on the total overshoot in calculation. The height of the curved quadrangle corresponding to each color represents the overshoot in calculation of a variable, and the accumulated height is the total overshoot in calculation of the four variables. In Figure 9, with the increase of , the total overshoot in calculation continues to decrease. It means that the calculation curve is smoother and will not produce large fluctuations and shocks near the optimal value.

In the above analysis, both  and  are based on the identity matrix, and only the coefficients  and  are used to adjust the relative value. In fact, each element on the diagonal line in  and  represents the proportion of each variable in the performance index . It has been verified that the adjustment of the coefficients  and  can already make the calculation performance of LQR-Lagrange Algorithm better. Therefore, the fact that  and  are based on the identity matrix does not have a negative impact on the calculation performance.

In general, the calculation performance of LQR-Lagrange Algorithm is related to the relative value of  and , and the parameter tunings of the proposed algorithm have a monotonic influence on the calculation performance. Therefore, users can easily master the monotonic and user-friendly parameter tuning strategy of LQR-Lagrange Algorithm to make the calculation performance better.

  - LQR-Lagrange Algorithm for the Optimization Problem with Inequality Constraints

As is mentioned before, the calculation of the optimization problem with inequality constraints is a multi-stage control process from the perspective of control theory.

When a certain , the state equation of the control system corresponding to the calculation process is still Equation (15). When a certain , due to the restriction of Equation (17), it has to be forced . In this situation, the state equation of the system has changed.

Define the “reduced-dimension state equation” as the state equation obtained by deleting the row and column corresponding to  in Equation (15). It means that whenever there is a  (forced to 0), the system is reduced by one dimension and the calculation of  is stopped. In this case, we need to recalculate the optimal feedback matrix  using the reduced-dimension state equation, and then use LQR-Lagrange Algorithm to calculate.

However, due to the special position of the initial point, inequality constraints, and the optimal value, there may be a case where  (forced to 0) first and then  again. It means that the iteration point returns from the inside to the outside of the inequality constraint  or eventually converges on it. In order to prevent LQR-Lagrange Algorithm from erroneously stopping the calculation of , we also need to check this case. Therefore, the rule of using LQR-Lagrange Algorithm to calculate the optimization problem with inequality constraints is:

If  after  iterations, the system state equation will not be changed in the next iteration.

If  after  iterations,  and  are forced to 0, and the calculation of  ends. In  iterations, the origin state equation without reducing the dimension is calculated to determine whether  again after the  iterations. If it is, continue to use the original state equation to calculate. If not, use the reduced-dimension state equation to continue the calculation of other variables.

()

()

()

Lagrange Algorithm Equations (6)-(8), Nesterov’s Accelerated Gradient Algorithm28, Momentum Algorithm28 and LQR-Lagrange Algorithm Equations (13)-(16) are still applied to calculate  respectively. The initial point is . The optimal point is .The condition for the convergence is, .

The selection method of optimal parameters is the same as before. The parameter in LQR-Lagrange Algorithm is the initial setting. If the dimensions of the state equation are reduced, the corresponding rows and columns in  and  are also deleted, but the values of other parameters are not changed. The parameter tuning and calculation results are listed in Table 2 and illustrated in Figure 10. In Figure 10, the LQR-Lagrange Algorithm is significantly faster than the other three algorithms. It shows that the proposed algorithm has good performance when calculating optimization problems with both equality and inequality constraints.

Table 2

Parameters and results in the calculation example.

| Algorithm | Parameters | Iterations at convergence |
| --- | --- | --- |
| Lagrange Algorithm |  | 15 |
| Momentum Algorithm |  | 21 |
| Nesterov’s Accelerated Gradient Algorithm |  | 11 |
| LQR-Lagrange Algorithm |  | 5 |

Figure 10

The calculation results by Lagrange Algorithm (Lag), Momentum Algorithm (Mom), Nesterov’s Accelerated Gradient Algorithm (Nes), and LQR-Lagrange Algorithm (LQR-Lag).

- Conclusion

The iterative optimization algorithm is regarded as a control process in this paper. Optimal control is selected to control the calculation process. The stable state in the optimal control is the origin O. In order to meet this requirement, the calculation format of Lagrange Algorithm is modified. The KKT conditions is set as the control objectives and the state equation of the control system corresponding to Lagrange Algorithm is established. As a kind of optimal control method, LQR is utilized to control the calculation process, and an innovative LQR-Lagrange Algorithm is proposed. The convergence is proved by the Lyapunov stability criterion. The analysis shows that LQR-Lagrange Algorithm is bound to converge under the premise that parameters  and  are positive definite.

In addition, the calculation results verify the convergence and the fast calculation speed. The step along the gradient direction during each iteration is also studied, indicating that LQR-Lagrange Algorithm is actually a self-adaptive gradient method. The most distinctive feature of the proposed algorithm is the monotonic and user-friendly parameter tuning strategy. Both theoretical analysis and calculation examples show that the influence of parameters  and  of LQR-Lagrange Algorithm on the calculation speed is monotonic. Since the optimization of with inequality constraints is a multi-stage control process from the perspective of control theory, we also analyzed the calculation strategy in this case, and a calculation example verifies the good performance of LQR-Lagrange Algorithm.

In this paper, LQR is used to control the calculation process, which solves the following difficulties in the traditional gradient method: the parameter tuning is quadratic and inconvenient for users; the influence of the step coefficients on convergence is highly coupled; and it lacks the step tuning strategy. Moreover, compared with the penalty function method, the proposed algorithm does not need to set an additional and complicated penalty function. Therefore, LQR-Lagrange Algorithm is advanced, convenient and user-friendly.

The combination of control theory and optimization algorithm provides a way to improve the algorithm. The innovative idea of this paper is to regard the iterative calculation of the optimization problem as a control process, and transform it through the control method to improve the performance of the algorithm. The essence of LQR-Lagrange Algorithm is the gradient method which is an iterative calculation method. Therefore, Lagrange Algorithm is used as an example in this paper, and the improvement of other gradient method can learn from this idea. Moreover, in addition to LQR control, there are many control methods with various characteristics in control theory, and they can all be applied to the improvement of optimization algorithms.

The research in this paper is instructive for engineering applications, especially in the optimization of the distribution network in power systems. The proposed algorithm has a fast convergence speed, and the parameter tuning strategy is monotonic and user-friendly, which will improve the efficiency of the time-varying optimal scheduling of the distribution network in power systems. Further research in engineering applications will be expanded and reported in future studies.

- References

- Zhou XY, Dall’Anese E, Chen LJ. Online Stochastic Optimization of Networked Distributed Energy Resources. Ieee T Automat Contr. Jun 2020;65(6):2387-2401. doi:10.1109/Tac.2019.2927925

- Fan S, He GY, Zhou XY, Cui MJ. Online Optimization for Networked Distributed Energy Resources With Time-Coupling Constraints. Ieee T Smart Grid. Jan 2021;12(1):251-267. doi:10.1109/Tsg.2020.3010866

- Ghadimi E, Teixeira A, Shames I, Johansson M. Optimal Parameter Selection for the Alternating Direction Method of Multipliers (ADMM): Quadratic Problems. Ieee T Automat Contr. Mar 2015;60(3):644-658. doi:10.1109/Tac.2014.2354892

- Zhou XY, Dall’Anese E, Chen LJ, Simonetto A. An Incentive-Based Online Optimization Framework for Distribution Grids. Ieee T Automat Contr. Jul 2018;63(7):2019-2031. doi:10.1109/Tac.2017.2760284

- Hooker CA. Adaptation in Natural and Artificial Systems - Holland,Jh. Philos Psychol. Sep 1995;8(3):287-299.

- Brown AA, Bartholomewbiggs MC. Some Effective Methods for Unconstrained Optimization Based on the Solution of Systems of Ordinary Differential-Equations. J Optimiz Theory App. Aug 1989;62(2):211-224.

- Felippa CA. Variational-Methods for the Solution of Problems of Equilibrium and Vibrations (Reprinted from Bull Am Math Soc, Vol 49, Pg 1, 1943). Int J Numer Meth Eng. Jul 15 1994;37(13):2159-&. doi:10.1002/nme.1620371303

- Goh BS. Algorithms for unconstrained optimization problems via control theory. J Optimiz Theory App. Mar 1997;92(3):581-604. doi:10.1023/A:1022607507153

- Wilson AC, Recht B, Jordan MI. A Lyapunov Analysis of Accelerated Methods in Optimization. J Mach Learn Res. 2021;22. http://jmlr.org/papers/v22/20-195.html

- Hu B, Lessard L. Dissipativity Theory for Nesterov's Accelerated Method. Pr Mach Learn Res. 2017;70.

- Ross IM. An optimal control theory for nonlinear optimization. J Comput Appl Math. Jul 2019;354:39-51. doi:10.1016/j.cam.2018.12.044

- Ross IM. Generating Nesterov’s accelerated gradient algorithm by using optimal control theory for optimization. J Comput Appl Math. May 15 2023;423. doi:10.1016/j.cam.2022.114968

- Dam HH, Nordholm S, Low SY, Cantoni A. Blind signal separation using steepest descent method. Ieee T Signal Proces. Aug 2007;55(8):4198-4207. doi:10.1109/Tsp.2007.894406

- Abrudan TE, Eriksson J, Koivunen V. Steepest descent algorithms for optimization under unitary matrix constraint. Ieee T Signal Proces. Mar 2008;56(3):1134-1147. doi:10.1109/Tsp.2007.908999

- Nesterov Y, Pardalos PM, Hearn DW, SpringerLink. Introductory Lectures on Convex Optimization : A Basic Course. 1st 2004. ed. Applied Optimization 87. Springer US : Imprint: Springer; 2004.

- Wibisono A, Wilson AC, Jordan MI. A variational perspective on accelerated methods in optimization. P Natl Acad Sci USA. Nov 22 2016;113(47):E7351-E7358. doi:10.1073/pnas.1614734113

- Söderlind G. Automatic control and adaptive time-stepping. Numer Algorithms. Dec 2002;31(1-4):281-310. doi:Doi 10.1023/A:1021160023092

- Xiang ZL, Ji DM, Zhang H, Wu HR, Li YX. A simple PID-based strategy for particle swarm optimization algorithm. Inform Sciences. Oct 2019;502:558-574. doi:10.1016/j.ins.2019.06.042

- An WP, Wang HQ, Sun QY, Xu J, Dai QH, Zhang L. A PID Controller Approach for Stochastic Optimization of Deep Networks. Proc Cvpr Ieee. 2018:8522-8531. doi:10.1109/Cvpr.2018.00889

- Qiu JB, Wang T, Yin S, Gao HJ. Data-Based Optimal Control for Networked Double-Layer Industrial Processes. Ieee T Ind Electron. May 2017;64(5):4179-4186. doi:10.1109/Tie.2016.2608902

- Bian T, Jiang Y, Jiang ZP. Decentralized Adaptive Optimal Control of Large-Scale Systems With Application to Power Systems. Ieee T Ind Electron. Apr 2015;62(4):2439-2447. doi:10.1109/Tie.2014.2345343

- Lin F, Fardad M, Jovanovic MR. Optimal Control of Vehicular Formations With Nearest Neighbor Interactions. Ieee T Automat Contr. Sep 2012;57(9):2203-2218. doi:10.1109/Tac.2011.2181790

- Neacsu DO, Sirbu A. Design of a LQR-Based Boost Converter Controller for Energy Savings. Ieee T Ind Electron. Jul 2020;67(7):5379-5388. doi:10.1109/Tie.2019.2934062

- Liu H, Lu G, Zhong YS. Robust LQR Attitude Control of a 3-DOF Laboratory Helicopter for Aggressive Maneuvers. Ieee T Ind Electron. Oct 2013;60(10):4627-4636. doi:10.1109/Tie.2012.2216233

- Xin GY, Xin SY, Cebe O, et al. Robust Footstep Planning and LQR Control for Dynamic Quadrupedal Locomotion. Ieee Robot Autom Let. Jul 2021;6(3):4488-4495. doi:10.1109/Lra.2021.3068695

- Bonnans JF, Gilbert JC, Lemarechal C, Sagastizabal CA. Numerical Optimization: Theoretical and Practical Aspects. 2nd ed, Springer, Berlin, New York, 2006.

- Ogata K. Discrete-Time Control Systems. 2nd ed, Prentice Hall, Englewood Cliffs, N.J., 1995.

- Sutskever I, Martens J, Dahl G, Hinton G. On the importance of initialization and momentum in deep learning. presented at: Proceedings of the 30th International Conference on Machine Learning; 2013; Atlanta, USA, June 16 - 21, 2013. https://proceedings.mlr.press/v28/sutskever13.html