1. Title of the manuscript

An Online Updated Linear Power Flow Model Based on Regression Learning

2. All the authors' complete names

Molin An, Tianguang Lu*, Xueshan Han

3.  Names of all authors

Molin An, Tianguang Lu*, Xueshan Han

4.   Departmental & institutional affiliations of the authors

Molin An; Tianguang Lu*; Xueshan Han;

School of Electrical Engineering, Shandong University, No.17923 Jingshi Road, Jinan, China

5.    Postal and E-mail address of the corresponding author

tlu@sdu.edu.cn

School of Electrical Engineering, Shandong University, No.17923 Jingshi Road, Jinan, China

6.  Conflict of Interest statement

The authors declare that we have no conflicts of interest in this research.

7.  Funding information (Please mention "Funding: None" if you have not received any support for your research)

Funding: Shandong Excellent Youth Science Fund Project (Overseas): 2022HWYQ-039

8.  Data Availability statement.

The data that support the findings of this study are available from the corresponding author upon reasonable request.

9.   Credit contribution statement

Molin An: Methodology; Software; Writing – original draft

Tianguang Lu*: Writing – review & editing

Xueshan Han: Writing – review & editing

An Online Updated Linear Power Flow Model Based on Regression Learning

Molin An 1, Tianguang Lu 1*, Xueshan Han 1

1 School of Electrical Engineering, Shandong University, No.17923 Jingshi Road, Jinan, China

*tlu@sdu.edu.cn

Abstract: An online updated data-driven linear power flow (LPF) model based on regression learning is proposed in this paper. We obtain a quadratic power flow model through regression learning first, and then derive the normal and incremental forms of LPF models by Taylor expansion. The parameters of LPF model are updated online, which improves the generalization ability. After only one initial regression learning, the proposed data-driven LPF model avoids model retraining when updated. The new parameter of the proposed model is simply calculated according to the real-time measurement data. Therefore, the LPF model we proposed is accurate, generalizable, and greatly minimizes the data consumption and running time. Performance analysis verifies the superiority of the proposed method.

Keywords: linear power flow, regression learning, online updated, data-driven, generalization ability.

- Introduction

Power flow calculation is the cornerstone of power system planning, operation, optimal dispatch and control (Li et al., 2018). However, the proportion of renewable energy in the power system continues to increase (Cai et al., 2022), which aggravates the burden of power flow calculation because of the random fluctuation (Ding et al., 2022). In order to solve the problems caused by the high proportion of renewable energy (Woyte et al., 2006), operators need to respond immediately to the fluctuations of renewable energy (Bolognani et al., 2015) and perform fast power flow calculation in time (Gan et al., 2014). However, the inherent nonlinearity of the AC power flow equation brings great difficulties to solve the above problems (Yang et al., 2019). For example, the optimal power flow problem with AC power flow model is NP-hard (Lehmann et al., 2016). In order to reduce the burden caused by nonlinearity, researchers use LPF model to solve problems including power flow calculation (Bolognani et al., 2016), voltage control (Jabr, 2018), optimal power flow (Zhou et al., 2018; Yang et al., 2018), economic operation (Wang et al., 2021), and dispatch plan formulation and execution (Xie et al., 2008). Therefore, LPF model should have the features of accurate results, fast running, simple model, and convenience.

LPF research is studied in two types: model-driven and data-driven (Tan et al., 2020). The most representative model-driven method is DC power flow, which is widely used in industry (Wang et al., 2007). Distflow model is suitable for distribution network with radial structure (Baran et al., 1989), and the corresponding LPF model is obtained by Taylor expansion (Chen et al., 2018). Under certain conditions, some variables are approximated as constants to derive LPF model (Yuan et al., 2018; Christakou et al., 2013). In addition, some studies redefine the variables to obtain new auxiliary variables to form LPF equation (Li et al., 2018). Nevertheless, the above methods are contingent on system parameters, including impedance and network topology, significantly impacting accuracy. Furthermore, the derivation and computation involved in these methods are intricate, resulting in limited applicability (Yang et al., 2019).

In modern power systems, the wide application of measurement equipment makes it possible to generate LPF models through data-driven methods (Chen et al., 2016). The data-driven LPF model are not contingent on the structure and parameters of the power system (Liu et al., 2022). It analyses the historical data to obtain the linear correlation between power flow variables (Angelosante et al., 2010; Shao et al., 2021), such as noise-aided regression analysis (Zhang et al., 2019), robust mapping rules (Yu et al., 2017), partial least squares and Bayesian linear regression methods (Liu et al., 2019).

However, the existing data-driven LPF model has shortcomings, which are reflected in practical applications. It has a large demand for historical data, and the data quality used in regression learning is also strict (Liu et al., 2020). For example, bad data can greatly affect the accuracy of the model (Liu et al., 2022). Moreover, in time-varying optimal power flow researches (Zhou et al., 2018), data-driven LPF model can effectively reduce the difficulty of solving the problem and improve the running speed. Nevertheless, the data-driven LPF model needs to be updated over time to avoid inaccurate results caused by linearization errors, which is due to overfitting or bad data as mentioned above and leads to poor generalization ability (the ability of the model to maintain accuracy on new data over time without model retraining). This issue is typically addressed by either expanding the sample dataset or continuously retraining the model in real-time. However, it will pose a significant data burden and diminish running speed, contradicting the imperative for swift adaptations in time-varying optimization.

To overcome these difficulties and shortcomings above and then pave the way for our future research on time-varying optimization and tie-line power control, we hope to develop a fast and accurate data-driven LPF power flow model that can be updated online without model retraining. Therefore, a data-driven online updated LPF model based on regression learning is proposed in this paper. The pure quadratic term of the node power is used as one of the variables to obtain the quadratic model by performing regression learning on the historical data of power flow. In order to obtain LPF model, Taylor expansion is used to expand the quadratic model into the normal and incremental forms of LPF model. The performance analysis demonstrates a significant enhancement in accuracy with the proposed method compared to the traditional data-driven LPF model. In practical application, one single regression learning process is sufficient to initialize the proposed LPF model, eliminating the need for model retraining and reducing data consumption. Post Taylor expansion, the model no longer requires real-time regression learning, utilizing measured node power data for continuous online updates. This approach fortifies the model’s generalization ability and improves operational efficiency.

The main contributions of this paper are as follows:

The proposed data-driven LPF model is updated online without model retraining. By applying the Taylor expansion to the quadratic model obtained by regression learning, we derive the normal and incremental forms of LPF model respectively. Following one single initialization by regression learning, the proposed method eliminates the necessity for model retraining during updates. Instead, the model is dynamically updated online by collecting real-time power data, thereby augmenting its generalization ability. This approach not only reduces running time but also minimizes data consumption, resulting in a substantial improvement in efficiency.

- Theoretical Analysis and Model Derivation

Two types of LPF models are broadly used: the normal form Eq. 1, and the incremental form Eqs. 2-4 which is also regarded as sensitivity equation (Guo et al., 2019):

Uk=RPk+XQk+A  (1)

where P and Q are the vectors formed by the active and reactive power of the node respectively, and R, X, A are parameter matrixes approximated by network physical models or obtained over data-driven learning and training.

Uk+1=Uk+RΔP+XΔQ  (2)

ΔP=Pk+1-Pk   (3)

ΔQ=Qk+1-Qk   (4)

The incremental form Eqs. 2-4 is widely used as a state equation in optimization of the power system by Model Predictive Control (MPC) (Guo et al., 2020).

We will devise a method that can provide LPF models in both forms.

The active and reactive power of all nodes are:

P=(p1,p2,…pn)T   (5)

Q=(q1,q2,…qn)T   (6)

The vector of pure quadratic terms of active and reactive power of all nodes are:

Ppure2=(p12,p22,…pn2)T   (7)

Qpure2=(q12,q22,…qn2)T   (8)

The model trained by regression learning is:

Uik=RUiquaPpure2k+XUiquaQpure2k

+RUilinPk+XUilinQk+AUiqua   (9)

where the superscript of the coefficient matrix is the coefficient name, and the subscript is the corresponding variable of n nodes (i=1,2,…,n).

After expand Eq. 9 by using Taylor expansion, we further derive LPF model.

The normal form is:

Uik+1=RUinorPk+1+XUinorQk+1+AUinor   (10)

RUinor=2RUiqua.×P(k)T+RUilin   (11)

XUinor=2XUiqua.×Q(k)T+XUilin   (12)

AUinor= Uik-2RUiquaPpure2(k)T-RUilinP(k)

-2XUiquaQpure2(k)T-XUilinQk   (13)

Fig. 1.  Flowchart of the proposed method

The incremental form is:

Uik+1=Uik+RUiincΔP+XUiincΔQ  (14)

RUiinc=2RUiqua.×P(k)T+RUilin   (15)

XUiinc=2XUiqua.×Q(k)T+XUilin   (16)

where .× represents the multiplication of the elements at the corresponding positions in the matrix.

Model modification is performed when needed:

Uik=Ui,measurek   (17)

The Lagrange remainder after Taylor expansion is:

Z(k)=[PkT,QkT]T   (18)

∆Z=Zk+1-Z(k)   (19)

R1Z=12∆ZT∇2UZξ∆Z  ξ∈(K,K+1) (20)

The steps of using the proposed method in practical applications are as follows, which is illustrated in Fig. 1:

1) Regression learning is used to train the quadratic model Eq. 9 on the collected historical data.

2) Perform Taylor expansion on the quadratic model to obtain the LPF model Eqs. 10-16.

3) Set the modification time and when it reaches, the latest node power measurement data is used to correct the model online by Eq. 17.

4) Calculate the power flow by the proposed model.

5) Back to 3).

6) End if necessary.

Compared with the traditional data-driven LPF model, the quadratic model trained in this paper more precisely reflects the correlation between the variables in the power flow equation. Instead of approximating from nonlinear power flow equations of higher order, we carry out Taylor expansion from quadratic model after regression learning, which discards fewer remainders to make the error reduced. In addition, the traditional data-driven LPF model needs model retraining frequently when used. However, the parameters of time k+1 in Eqs. 10-16 comprehend the historical data at the last time k. Hence, a singular regression learning is conducted for model initialization, after which the previous measurement data is leveraged for subsequent online updates. It circumvents the substantial data consumption associated with model retraining, leading to enhanced efficiency during model updates. The efficacy is substantiated through performance analysis.

- Performance Analysis

The performance analysis of the proposed method and the traditional data-driven LPF model (Zhou et al., 2018) is carried out in this section by using a total of 86400 seconds of data for 24 hours in IEEE 33-node network. The data set of first 1000 seconds is used to train the model on the voltages of the other 32 nodes (except node 0) through regression learning, and the all-day data is used for verification and comparison to exhibit the good performance of the model we proposed.

  - Model training by regression learning

First, we compare the training error of the proposed method and the traditional data-driven LPF model (Zhou et al., 2018). The training process of the proposed model is introduced in the following. We used Regression Learner APP in MATLAB to train the data through the Interactions Linear method. The training mode is selected as Pure Quadratic. In this mode, the pure quadratic term of the node voltage is taken as one of the variables for regression learning together with the node voltage first-order term. Five-fold cross validation is used to evaluate the model. The dataset is divided into five equal-sized parts, each of which is called a fold. One of the folds is the validation set and the remaining four are applying for training and it is repeated five times until each fold is used as a validation set. The evaluation indicators

(a)

(b)

(c)

Fig. 2.  Comparison of model evaluation indicators

(a) MSE, (b) MAE, (c) RMSE

are calculated by taking the average of five. The final model is trained on the

Fig. 3.  Error distributions of 32 nodes in 24h

entire dataset. The results are illustrated in Fig.2 (The R² of both models is 1, so no comparison is performed).

In Fig.2, the proposed quadratic model has great advantages in the comparison of MSE, MAE and RMSE, and is significantly minor than the traditional linear model in the order of magnitude. It verifies that the proposed quadratic model approximates the power flow equation more precisely. In order to obtain the LPF model, the quadratic model will be performed Taylor expansion then. Although it results in a slight error increase due to the remainder Eq.20, the subsequent analysis will exhibit that the LPF model after Taylor expansion still has less error than the traditional data-driven linear model.

In practical engineering, the training data can be selected based on the measured data. For example, the model can be trained using power flow data from yesterday or more historical data. The training and validation processes introduced in this paper are only examples. There is also a risk that the model will not be accurate enough using the last 1,000 seconds data as mentioned above, as opposed to using a larger amount of historical data. However, in such data cases, the model proposed in this paper already achieves good results, indicating that it is less dependent on data, and subsequent analysis will further illustrate its strong generalization ability.

  - Performance comparison

According to the previous analysis, the proposed method has advantages in accuracy and efficiency, and can update online. The proposed method with modification and without modification are compared with the traditional data-driven LPF model (Zhou et al., 2018) in IEEE 33-node and 69-node networks. In order to compare the generalization ability of the models, we assumed that the traditional data-driven LPF model performs model training with only one initialization which is consistent with the proposed method. The modification time interval is 5 minutes. Fig. 3 - Fig. 7 are the results for the 33-node network (except node 0), and Fig.8 - Fig. 9 are the results for the 69-node network (except node 0). The simulation is carried out by MATLAB R2018b, i7-8750H 2.20GHz Intel Core processor, 8GB memory.

Considering the low user load in the dataset employed in this study and the absence of photovoltaic (PV), the power flow data exhibit minimal fluctuations during this time period. Consequently, the errors of the LPF model are too negligible to warrant meaningful comparisons. Therefore, data spanning only from 6:00 to 24:00 (18h, 64801 seconds, 32 nodes, the total amount of data is 2073632) are utilized for comparison, aiming to validate the merits of the proposed method.

Fig. 4.  The max error in the 32 nodes

Fig. 5.  The mean error in the 32 nodes

The box plot illustrated in Fig. 3 present the error distributions of the 2073632 errors. The error data obtained are all valid and has no outliers. Consequently, we expand the range of upper and lower bounds to cancel the display of outliers. As shown in Fig.3, the margin of error, the range of concentration, and the median of the proposed model are all minor, which also clearly demonstrate our model’s superior precision. As mentioned above, the quadratic model trained by regression learning is more precise than the traditional data-driven LPF model. However, the remainder of Taylor expansion slightly diminishes its accuracy. The result in Fig.3 exhibits that the LPF model after Taylor expansion is still dominant. In other words, the improvement of the quadratic model affects the accuracy more than the deterioration of the error introduced by the remainder of the Taylor expansion.

Subsequently, these data will be further analysed in terms of time and node respectively to examine and illustrate the advantages of the proposed model.

Fig. 6.  The max error in a day

Fig. 7.  The mean error in a day

Fig. 4 and Fig. 5 exhibit the error comparison over time. In terms of both the max error or the mean error, the proposed method has much lower error than the traditional data-driven LPF model. When modification is supplemented, the error is further reduced. Besides, it illustrates that the generalization ability of the traditional data-driven LPF model is weak. Therefore, to enhance the accuracy of the traditional data-driven LPF model, frequent regression becomes imperative, leading to substantial and recurrent data consumption and increased operational costs. Evidently, the proposed method exhibits superior accuracy and robust generalization ability when compared to the traditional one. Because it requires only one initial training for regression learning, while the model is updated online based on real-time data measurement.

Fig. 8.  The max error in the 68 nodes

Fig. 9.  The mean error in the 68 nodes

Fig. 6 and Fig. 7 exhibit the error comparison in different nodes. It illustrates that the node error of the traditional data-driven LPF model will increase significantly closer to the end of the branch. Moreover, the error decreases significantly when using the proposed method (without modification), and further reduced by modification. In addition, from the above results, the accuracy of the method gradually decreases towards the end direction as the branch is extended. In the future work, we will further study the reasons for this problem and propose further improvements to the proposed method to advance the calculation accuracy of the end nodes of the branch.

Fig. 8 and Fig. 9 also exhibit the error comparison by time in the 69-node network, which can verify the performance of the proposed method in the larger system. Since we set the change trend of the load and PV curve in 69 nodes to be approximate to that in 33 nodes, the results in Fig. 8 and Fig. 9 are also approximate to those of Fig. 4 and Fig. 5. It illustrates that the proposed method is data-driven that are not contingent on the power system structure and parameters. Regardless of the system network structure, it is performed as long as the historical power flow data is measured and saved. Therefore, the proposed method can be applied to larger systems.

Table 1 Comparison in running time and memory

| Index | Traditional data-driven method | The proposed method |
| --- | --- | --- |
| Time for initializing training | 0.016s | 24.88s |
| Memory for initializing | 1455KB | 678KB |
| Time for model updating once | 0.016s | 0.002s |
| Memory for model updating once | 1455KB | 96KB |
| Time for total running (per minute in 1 day) | 23.04s | 27.76s |
| Memory for total running (per minute in 1 day) | 1.998GB | 0.132GB |
| Time for total running (per minute in 7 days) | 161.28s | 45.04s |
| Memory for total running (per minute in 7 days) | 13.987GB | 0.923GB |

We also analyse the efficiency in terms of running time and amount of memory data to verify the advantages of the proposed method. As mentioned in the previous analysis, the proposed method only needs one initial regression learning and can update online. However, the traditional data-driven LPF model has poor generalization ability, and it is necessary to repeat the regression learning frequently to update the model for improving accuracy. Therefore, in the next analysis, the traditional data-driven LPF model performs continuous model retraining when updating, while the proposed method performs only one initial training and updates the model parameters online. Table 1 is the statistical results.

It is obvious that the proposed method has advantages in running time and memory usage for a single update. In terms of data consumption, the proposed method is far less than the traditional data-driven LPF model. It greatly reduces the data burden and brings great convenience for practical engineering use. In addition, the comparison of the running time in Table 1 illustrates the superiority of the proposed method in long-term use. Although it takes a slightly longer time for regression learning to initialize the proposed model, the trained model is used for a long time without model retraining through online real-time parameter updates. However, the traditional data-driven LPF model takes a short time to initialize, but it has weak generalization ability and cannot update parameters online. It leads to frequent model retraining in use. As the running time increases, the cost of model retraining also increases. In the minute-level update test, the total computation time of the proposed method in one day is slightly more than that of the traditional data-driven LPF model. Furthermore, with the continuous extension of the use over days, the proportion of the initialization time of the proposed method in the total running time consistently diminishes, and the total running time is less than that of the traditional method. The results of a 7-day run verify it. With the increase of the number of days, the accumulation of model retraining time of the traditional one in use is far more than that of the proposed method. Therefore, the proposed method is enduring, sustainable, and more efficient in the long run.

- Conclusion

We propose a data-driven online updated LPF model based on regression learning in this paper which only needs to perform once regression learning for initialization, and the parameters is updated online by using the measurement data. It circumvents a large amount of data consumption and running cost when retraining the model as the traditional data-driven method using in engineering. The pure quadratic term of the node power is taken as one of the variables to obtain the quadratic power flow model, which is more precise than the traditional data-driven LPF model. The proposed LPF model is derived by the Taylor expansion of the quadratic model. However, the remainder of Taylor expansion slightly diminishes its accuracy. The analysis in this paper exhibits that the LPF model after Taylor expansion is still dominant. In other words, the improvement of the quadratic model affects the accuracy more than the deterioration of the error introduced by the remainder of the Taylor expansion. The normal and incremental forms of LPF model is derived after the Taylor expansion of the quadratic model. The former is used in fast power flow calculation, time-varying optimal dispatching and other practical power system management. Since the latter form is consistent with the state equation in control theory, it is used in various power pursuit or online optimization based on control models such as MPC. We will apply the proposed method in our future research on time-varying optimization and tie-line power control by SMPC.

The analysis exhibits that compared with the traditional data-driven method, the proposed method greatly improves the calculation accuracy and the model has stronger generalization ability. In a long time of practical use, its low running cost, fast speed and high accuracy are further revealed. It indicates that the method proposed in this paper has broad application prospects in practical use.

- References

Angelosante, D., Bazerque, J. A., and Giannakis, G. B. (2010). Online adaptive estimation of sparse signals: Where RLS meets the l_1-norm. IEEE Trans. Signal Process. 58, 3436–3447. doi:10.1109/tsp.2010.2046897.

Baran, M., and Wu, F. F. (1989). Optimal sizing of capacitors placed on a radial distribution system. IEEE Trans. Power Del. 4, 735–743. doi:10.1109/61.19266.

Bolognani, S., and Zampieri, S. (2016). On the existence and linear approximation of the power flow solution in power distribution networks. IEEE Trans. Power Syst. 31, 163–172. doi:10.1109/tpwrs.2015.2395452.

Bolognani, S., Carli, R., Cavraro, G., and Zampieri, S. (2015). Distributed reactive power feedback control for voltage regulation and loss minimization. IEEE Trans. Autom. Control 60, 966–981. doi:10.1109/tac.2014.2363931.

Cai, S., Xie, Y., Wu, Q., Zhang, M., Jin, X., and Xiang, Z. (2022). Active and reactive power coordinated two-stage MG Scheduling for resilient distribution systems under uncertainties. IEEE Trans. Smart Grid 13, 2986–2998. doi:10.1109/tsg.2022.3149816.

Chen, P., Xiao, X., and Wang, X. (2018). Dynamic optimal power flow model incorporating interval uncertainty applied to distribution network. IET Gener. Transmiss. Distrib. 12, 2926–2936. doi:10.1049/iet-gtd.2017.1874.

Chen, Y. C., Wang, J., Dominguez-Garcia, A. D., and Sauer, P. W. (2016). Measurement-based estimation of the Power Flow Jacobian matrix. IEEE Trans. Smart Grid 7, 2507–2515. doi:10.1109/tsg.2015.2502484.

Christakou, K., LeBoudec, J.-Y., Paolone, M., and Tomozei, D.-C. (2013). Efficient computation of sensitivity coefficients of node voltages and line currents in unbalanced radial electrical distribution networks. IEEE Trans. Smart Grid 4, 741–750. doi:10.1109/tsg.2012.2221751.

Ding, Y., Morstyn, T., and McCulloch, M. D. (2022). Distributionally robust joint chance-constrained optimization for networked microgrids considering contingencies and renewable uncertainty. IEEE Trans. Smart Grid 13, 2467–2478. doi:10.1109/tsg.2022.3150397.

Gan, L., and Low, S. H. (2014). Optimal Power Flow in direct current networks. IEEE Trans. Power Syst. 29, 2892–2904. doi:10.1109/tpwrs.2014.2313514.

Guo, Y., Wu, Q., Gao, H., Chen, X., Ostergaard, J., and Xin, H. (2019). MPC-based coordinated voltage regulation for distribution networks with distributed generation and Energy Storage System. IEEE Trans. Sustain. Energy 10, 1731–1739. doi:10.1109/tste.2018.2869932.

Guo, Y., Wu, Q., Gao, H., Huang, S., Zhou, B., and Li, C. (2020). Double-time-scale coordinated voltage control in active distribution networks based on MPC. IEEE Trans. Sustain. Energy 11, 294–303. doi:10.1109/tste.2018.2890621.

Jabr, R. A. (2018). Linear Decision Rules for control of reactive power by distributed photovoltaic generators. IEEE Trans. Power Syst. 33, 2165–2174. doi:10.1109/tpwrs.2017.2734694.

Lehmann, K., Grastien, A., and Van Hentenryck, P. (2016). AC-feasibility on tree networks is NP-hard. IEEE Trans. Power Syst. 31, 798–801. doi:10.1109/tpwrs.2015.2407363.

Li, Z., Yu, J., and Wu, Q. H. (2018). Approximate linear power flow using logarithmic transform of voltage magnitudes with reactive power and transmission loss consideration. IEEE Trans. Power Syst. 33, 4593–4603. doi:10.1109/tpwrs.2017.2776253.

Li, Z., Yu, J., and Wu, Q. H. (2018). Approximate linear power flow using logarithmic transform of voltage magnitudes with reactive power and transmission loss consideration. IEEE Trans. Power Syst. 33, 4593–4603. doi:10.1109/tpwrs.2017.2776253.

Liu, Y., Li, Z., and Zhao, J. (2022). Robust data-driven linear power flow model with probability constrained worst-case errors. IEEE Trans. Power Syst. 37, 4113–4116. doi:10.1109/tpwrs.2022.3189543.

Liu, Y., Li, Z., and Zhou, Y. (2022). Data-driven-aided linear three-phase power flow model for Distribution Power Systems. IEEE Trans. Power Syst. 37, 2783–2795. doi:10.1109/tpwrs.2021.3130301.

Liu, Y., Wang, Y., Zhang, N., Lu, D., and Kang, C. (2020). A data-driven approach to linearize power flow equations considering measurement noise. IEEE Trans. Smart Grid 11, 2576–2587. doi:10.1109/tsg.2019.2957799.

Liu, Y., Zhang, N., Wang, Y., Yang, J., and Kang, C. (2019). Data-driven power flow linearization: A regression approach. IEEE Trans. Smart Grid 10, 2569–2580. doi:10.1109/tsg.2018.2805169.

Shao, Z., Zhai, Q., Wu, J., and Guan, X. (2021). Data based Linear Power Flow Model: Investigation of a least-squares based approximation. IEEE Trans. Power Syst. 36, 4246–4258. doi:10.1109/tpwrs.2021.3062359.

Tan, Y., Chen, Y., Li, Y., and Cao, Y. (2020). Linearizing Power Flow Model: A hybrid physical model-driven and data-driven approach. IEEE Trans. Power Syst. 35, 2475–2478. doi:10.1109/tpwrs.2020.2975455.

Wang, B., Tang, N., Bo, R., and Li, F. (2021). Three-phase DLMP model based on linearized power flow for distribution with application to Der Benefit Studies. Int. J. Elect. Power Energy Syst. 130, 106884. doi:10.1016/j.ijepes.2021.106884.

Wang, H., Murillo-Sanchez, C. E., Zimmerman, R. D., and Thomas, R. J. (2007). On computational issues of market-based Optimal Power Flow. IEEE Trans. Power Syst. 22, 1185–1193. doi:10.1109/tpwrs.2007.901301.

Woyte, A., Thong, V. V., Belmans, R., and Nijs, J. (2006). Voltage fluctuations on distribution level introduced by Photovoltaic Systems. IEEE Trans. Energy Convers. 21, 202–209. doi:10.1109/tec.2005.845454.

Xie, L., and Ilic, M. D. (2008). Model predictive dispatch in electric energy systems with intermittent resources. 2008 IEEE International Conference on Systems, Man and Cybernetics. doi:10.1109/icsmc.2008.4811248.

Yang, Z., Xie, K., Yu, J., Zhong, H., Zhang, N., and Xia, Q. X. (2019). A general formulation of linear power flow models: Basic theory and error analysis. IEEE Trans. Power Syst. 34, 1315–1324. doi:10.1109/tpwrs.2018.2871182.

Yang, Z., Xie, K., Yu, J., Zhong, H., Zhang, N., and Xia, Q. X. (2019). A general formulation of linear power flow models: Basic theory and error analysis. IEEE Trans. Power Syst. 34, 1315–1324. doi:10.1109/tpwrs.2018.2871182.

Yang, Z., Zhong, H., Bose, A., Zheng, T., Xia, Q., and Kang, C. (2018). A linearized OPF model with reactive power and voltage magnitude: A pathway to improve the MW-only DC OPF. IEEE Trans. Power Syst. 33, 1734–1745. doi:10.1109/tpwrs.2017.2718551.

Yu, J., Weng, Y., and Rajagopal, R. (2017). Robust mapping rule estimation for power flow analysis in distribution grids. 2017 North American Power Symposium (NAPS). doi:10.1109/naps.2017.8107397.

Yuan, H., Li, F., Wei, Y., and Zhu, J. (2018). Novel linearized power flow and linearized OPF models for active distribution networks with application in distribution LMP. IEEE Trans. Smart Grid 9, 438–448. doi:10.1109/tsg.2016.2594814.

Zhang, J., Chen, Z., He, C., Jiang, Z., and Guan, L. (2019). Data-driven-based optimization for power system VAR-voltage sequential control. IEEE Trans. Ind. Informat. 15, 2136–2145. doi:10.1109/tii.2018.2856826.

Zhou, X., Dall’Anese, E., Chen, L., and Simonetto, A. (2018). An incentive-based online optimization framework for distribution grids. IEEE Trans. Autom. Control 63, 2019–2031. doi:10.1109/tac.2017.2760284.