Performance evaluation of polygonal and cylindrical ball mills using discrete element method

*Mathew Neto Kyalo, James Kuria Kimotho, Hiram Muriithi Ndiritu

*Corresponding author email address: mnkyalo@jkuat.ac.ke

Affiliation: Department of Mechanical Engineering, Jomo Kenyatta University of Agriculture and Technology

P. O. Box 62000 – 00200 Nairobi, Kenya

# ABSTRACT

Ball mills are commonly used in chemical and mineral processing industries for particle size reduction. In mineral processing, cylindrical drums with wear resistant materials are used various operation scales. Polygon shaped mills have found application by artisanal and small-scale miners (ASM) due technological and economic reasons. Evaluating this kind of mills is key in improving or replacing them based on technological data. A modest improvement in grinding efficiency and mill durability can lead to significant economic and environmental benefits. In this study, the performance of polygonal shaped mills was evaluated and compared with cylindrical profiles considering power draw, collision energy dissipation, relative wear and operational stability. Discrete element method (DEM) was used to carry out simulations to describe mechanical behavior of grinding media and the interaction with the mill walls. Polygonal shaped mills without lifters promoted more interparticle interaction with limited centrifuging of particle even at higher speeds compared to cylindrical mills without lifters. Installation of lifters in polygon increased inter-particle collisions and reduce power draw and relative wear. Cylindrical mill with lifters had high interparticle collisions similar to polygonal profile with much lower cumulative relative wear and high operational stability.

Keywords: Polygonal ball mills, Discrete Element Method, Power draw, Collision Energy, Artisanal and Small-Scale Miners

# INTRODUCTION

Ball milling is widely applied in mineral and chemical processing industries for particle size reduction. In mineral processing, ore is reduced into required size using various machines to liberate valuable components. Ball mills are usually the largest consumers of energy in mineral concentration circuits [1]. Manufacturers of ball mills desire to achieve high mineral recovery at low cost focusing mainly on energy consumption and particle size [2]. There are two interventions for upgrading comminution in the mining and processing industries; one of them is the introduction of new affordable technologies, while the other one is incremental improvement in the technology application [3]. Technology improvement in operating conditions of the comminution equipment explores optimization of processing parameters. This can lead to reduced operating costs, increased throughput and improved downstream processed as a result of better output particle size Artisanal and Small Scale Mining (ASM) sectors employ more people than medium and large scale miners combined, thus being a potential for improving community livelihoods [4]. There are approximately 3,000 ball mills used by artisanal and small-scale gold miners in East African region. Polygon shaped are commonly used in the in the region in grinding of gold ore. The mills are usually not lined and users rely on large thickness of the material ranging from 18 –25 mm of mild steel to obtain longer lifespan. The wide distribution of polygonal mills can be attributed capability of fabrication without the need of rolling machines. Cylindrical mills are the conventional mills used continuous grinding operations. Adoption of continuous processing systems with primary crushers, wet type ball mills and downstream liberation processes could be more efficient and profitable. However, it would require artisanal miners to form larger formal groups with well managed centralized processing centers [5]. Some interventions are ongoing such as promoting formalization of this form of mining activity, but due to societal set up and technology bottlenecks, this might not be feasible. Until then, the polygonal shaped mills will be widely used, thus a modest improvement of grinding efficiency and mill longevity can lead to significant economic and environment benefits. With improved technology in ball milling, artisanal and small-scale miners would be able to improve processing efficiency of ores and add more value and increasing output. Although a ball mill is a simple piece of equipment, a wide range of breakage mechanisms occur in the mill making the process very difficult to understand, thus numerical simulation methods play a key role in prediction of engineering systems [6]. These methods help researchers to explore different scenarios quickly and efficiently at reduced costs. Discrete element method (DEM) has been used to simulate discrete systems of particles over the past thirty years and is widely accepted [7]–[10]. This method models particles as discrete entities assigning motion based on Newton’s Second Law of Motion. Appropriate contact models are chosen for particle-particle as well as particle-geometry interactions [11]. The earliest applications of DEM in ball mill was done by Mishra et.al. [12], where 2D discs were used to simulate particles in a ball mill with close comparison of torque between simulation and experimental results. This method could predict the power draw of the mill, the distribution of collision forces inside the mill, and effects of lifters in a mill. Since that time phenomenological improvements have been achieved on DEM method extending the concept to 3D applications with improvements in algorithms and particle models [13]. A major limitation of discrete element modeling is the high computational demands associated with modeling the large number of particles that exist in a system [14]. Clearly [15], carried out DEM modelling of 5 m ball mill and successfully predicted  charge profile, particle segregation state, power draw, torque, liner wear and also analyzed collisional forces. Using DEM, charge segregation was predicted and found to be affected by speed of rotation, thus it is possible to avoid this occurrence [15].  In a later paper, Cleary simulated full 3D models representing the entire charge particle distribution and was  able to predict charge profiles and shoulder and toe positions with good accuracy [16]. Extensive studies on full 3D industrial mill analysis were carried out elucidating effect of lifters on mill performance observed in industrial ball mills [17]–[20]. Recently Rasoul et, al studied the effect of ball size distribution on power draw and found that, mill filling of 35%  had the maximum effect on power draw [21]. Another recent application of DEM in ball milling has been explored by Francisco et.al. on the possibility to characterize load torque of tumbling ball mills in the frequency domain and from this characterization, they established a basis of a methodology capable of estimating the mill filling level [22]. DEM has also been used to simulate liner wear reducing years of experimental testing [23], [24]. In general, DEM has been used successfully as a means to understand, assist in design and optimization of ball mills. Most of the research has focused on improving ball milling performance mainly targeting the large industries in a view of improving their performance. Small scale like those that use polygonal mills have not received focus in dissemination of research findings for improve their ball milling activities. The main objective of this study is to compare the performance of polygonal and cylindrical shaped mills and avail data that can be used further in improving existing polygonal mills or replacing them. Studies on significance of mill shape are not available in literature, yet polygonal shaped mills are widely used by ASM. Discrete element method was used to study the effect of material fill, lifters and mill speed on the charge behavior, power draw and relative wear. The experimental set up and data acquisition method, then parameter identification and simulation procedures are described. Finally results of the simulation are stated and discussed focusing the effect of mill profiles on charge behavior, collision energy, power draw and relative wear.

# Facilities and Methods

## Experimental set up

A laboratory scale ball mill was set up including an option interchanging mills as shown in Figure 1. Hexagonal and cylindrical mill profiles were considered for the experiments. The cross-sectional area and axial length for both mills were 876.27cm2 and   of 355 mm respectively. The inner cylindrical surfaces and back-end walls were lined with natural rubber of 4 mm. The front end-wall was made of transparent perspex cover to allow observation of particles behavior in the mill. Both end-walls were flat and featureless to minimize their effect on the charge behavior and torque. The critical speed of the mill was calculated as a function of the mill diameter. Critical speed of a ball mill is the speed at which a single ball will just remain against the wall of the mill  for a complete cycle of mill rotation due to centrifugal force [38]. The speed of the electric motor was controlled by a variable frequency drive (VFD) allowing speeds from zero to beyond 100% of the critical speed. The VFD was part of a control module that hosted a timing module, emergency switch and on - off switches. Rotation speed was measured by tachometer and displayed on a LED screen. A rotary torque sensor with 200 N.m capacity, powered by 24V DC voltage was used measure shaft torque in form of a voltage signal. The voltage signal was amplified and acquired using a National instrument USB-6009 multi-function data card. A laptop computer was used to log the data and convert voltage into torque. The specification of the components used in the experimental setup are shown in Table 1. The mill speed was varied in the range of 65%-95% of critical speed. The filling levels were in the range of 9%- 48% of mill volume A digital camera was used to record charge motion through the transparent front. The shoulder and toe positions of charge profile were  measured in an anticlockwise direction from a zero position set at the 3 O’ Clock position [39].

Rotary torque Sensor

Mill

Electric

Motor

Tachometer

Figure 1: Laboratory mill experimental set up

Table 1: Experimental set up

| Component | Specifications |
| --- | --- |
| Electric motor | Three phase 1.5HP, 240 V, 50Hz. |
| Motor controller | VFD, Timer, Motor overload protection, On/off switches, Emergency button, indicators |
| Reducer | Helical geared unit with coaxial shaft in-line with the motor shaft. Reduction ratio 7.5 |
| Dynamic Torque sensor | ∅28 axle Torque range: 130-400Nm, Max speed 2000rpm. Signal output of 0-10v, Power supply: 24VDC/500mA, |
| Data acquisition | National Instruments USB-6009 Multifunction DAQ, input 0-10V, Output analogue/digital. Sampling Frequency; each analogue input - 48 kS/s. analog outputs -150 S/s. Resolution: analogue inputs 14-bit analog outputs 12-bit. |
| Mills | Hexagonal and Cylindrical profiles, Area: 876.27cm2, Length: 355 mm, critical speed 73.19 r.p.m |
| Lifters (Steel) | Rectangular profile: height:10 mm, width:20 mm |
| Grinding media | Material: Steel mm, density 7850, Mean ∅24.30, bulk density 4460 kg/m3 |

## DEM simulations

Discrete element numerical simulations were implemented using Altair EDEM software. Two mill drums, one hexagonal and another one cylindrical were compared under varied set of conditions to compare performance characteristics. Speed was varied in five discrete values as; 55%. 65%, 75%,85%, 95% of the critical speed. The fill levels were varied at: 9%, 19%, 28% 37%,48% of mill volume. The levels of fill and speed combinations were used to create a set of simulation conditions that covered the functional space of these variables. Further, six rectangular lifters were introduced in each of the mill profiles to study the effect of lifters performance. The effect of number of regular polygon sides on performance was studied by varying the number of sides at 4, 6, 8, 12 and 16-sides under identical operating conditions to evaluate. The cross-sectional area and length of the polygon was identical to the cylindrical mill. Entirely, there were 103 computational simulations. The variable parameters were speed of rotation, fill level and mill profile. The mill geometries were imported to Altair EDEM software using STEP-file format from CAD Modelling software. Grinding media geometry was approximated as perfect sphere of 12.15 mm diameter. The material properties of grinding media and mill are listed in Table 2. Inter-particle and particle-liner interactions were characterized by coefficients of restitution, static friction and rolling friction. Each simulation started with random generation of particles from a virtual factory allowing them to fall down under gravity. Filling process proceeded until all particles attained zero velocities forming packed bed as a result of energy dissipation due to damping. The simulations were run for eight seconds, ad results extracted out of the last two seconds. Static images The motion of charge profile was recorded by obtaining snapshots at intervals of 0.1 seconds  Data was exported and analyzed to obtain power draw, collision energy, relative wear and operational stability.

Table 2: Parameters used for DEM simulations

| Category | Parameter | Value/type |
| --- | --- | --- |
| Media (Steel) | Poisson’s ratio | 0.3 |
|  | Young’s modulus | 2.08 ×1011 |
|  | Density | 7800 Kgm3 |
| Boundary (Natural Rubber) | Poisson’s ratio | 0.5 |
|  | Young’s modulus | 3.33×106 |
|  | Density | 1522 Kgm3 |
| Paired | Particle-Particle (ϵ p-p) | 0.75 |
|  | Particle-Boundary (ϵ p-w) | 0.45 |
|  | Particle-Particle(μs, p-p) | 0.5 |
|  | Particle-Boundary (μs, p-w) | 0.65 |
|  | Particle-Particle (μr, p-p) | 0.02 |
|  | Particle-Boundary (μr, p-w) | 0.01 |
| Common | Gravity acceleration | 9.81 m/s2 |
|  | Contact model | Hertz Mindlin |
|  | Timestep (% critical timestep) | 20% |

## Calibration methodology

Calibration of contact parameters was carried out to find a set that would produce a good match between simulation and experimental results. The calibrated parameter procedure was an inverse problem realized by using results of simulations to infer the values parameters characterizing milling. A bench scale ball mill described in previous section was used to create typical flow regime and bulk behavior. Power and charge motion were used to provide comparative measurements between experiment and simulation. The parameters considered for calibration are the six paired ones shown in Table 2. The rest of material and interaction parameters were set based on literature [40]–[43]. A sensitivity study was run to select parameters with the largest effects on responses for purposes of reducing number of experiments. Thirteen parameter combinations within the experimental space were created for sensitivity study. Three of the experiments were replicated to ensure the reliability of the collected data.  The initial parameter range shown in Table 3. Rolling resistance was fixed at 0.01 for both interparticle and particle boundary similar to other studies [10], [45]. The null hypothesis proposed that paired parameters had no influence on power draw and charge behavior. A significance value (α) of 0.05 was selected to test the null hypothesis. A p-value is a statistical approach to hypothesis testing using calculated probability to determine whether there is evidence to reject the null hypothesis. If the test produced a p-value >0.05, then there was statistical significance between one independent value and power draw or charge behavior. The factors that had little influence on responses were fixed after screening experiment. A second design of simulation experiments followed the completion of the screening design, considering only the parameters that were found to significantly influence the results. A central composite design which included central and axial points was used to refine the experiment. Regression models were used to provide empirical relations between the response and the simulation factors. Experimental results were used as target values for multiple response prediction of simulation parameters. To simplify characterization of charge motion, only mean shoulder position was used. The shoulder position from simulation was measured from sequential image frames obtained when the charge motion had reached quasi static state. A total of sixteen images in a span of two seconds were considered for each set of comparison to increase accuracy of computations. An error of magnitude less than 2% for power variation in magnitude and less than 4 degrees for shoulder position between simulation and actual values was deemed acceptable for this comparative study. If the response was not satisfactory, the process of the calibration was repeated until a set of parameters that approximated the responses with least error were obtained as shown in Figure 2.

Figure 2: Parameter identification methodology

Table 3: Parameter range for Screening experiments without lifter

| Coefficient | Interaction | Symbol | Parameter range |
| --- | --- | --- | --- |
| Restitution | Particle-Particle | ϵ p-p | 0.4-0.8 |
|  | Particle-Boundary | ϵ p-w | 0.3-0.7 |
| Static friction | Particle-Particle | μs, p-p | 0.3-0.6 |
|  | Particle-Boundary | μs, p-w | 0.5-0.9 |
| Rolling friction | Particle-Particle | μr, p-p | 0.01 |
|  | Particle-Boundary | μr, p-w | 0.01 |

# Results and discussion

## Parameter identification

From the experiment, it was ascertained that the coefficients of restitution had low influence on power and charge behavior. The coefficients of static friction for interparticle interactions and particle wall interactions had the greatest influence and were considered in the response surface experiment. Coefficients of restitution were fixed based on literature and preliminary simulation tests values and are shown in Table 7. The low and high values of interparticle coefficient of friction (μs, p-p) were set at 0.3-0.6 and for particle wall ( μs, p-w)  at 0.4-0.9 based on value projection from the screening experiment. These values were fixed as the axial values in a randomized central composite design. In analysis of variance, the model was found to be statistically significant and relationships between power draw and μs, p-p was established and also a relationship between power and μs, p-w. Using statistical prediction, best combination of parameters was used to obtain the closely approximated experimental values of power draw of 136.88W and shoulder angle of 160.3°. The combination of the parameters which best approximated the experimental values were μs, p-p=0.5 ; μr, p-w=0.65. The coefficient of rolling friction between particles and mill was set higher than that of interparticle interaction due to prevailing material properties. The two factors had an almost linear relationship within the limits of experiment and the solution was assumed to be unique due to absence of points of inflection. The calibrated parameter values were used to validate simulations at different speeds and fill levels and the results are shown in Table 4. During validation, the calibrated parameters were held constant and fill volume and speed were varied. The percentage error between experimentally observed values of power draw and simulation values was less than 2%. The error between DEM shoulder angle and experimental values was less than 6.2% and was close to the experimentally observed values. Duplicated experiments on shoulder angle measurements deviated with error of 1- 2° which has also been reported in literature [42].

Table 4 : Validation of Calibrated paremeters

| Fill Level | Speed | Power (W) |  |  | Shoulder Position (deg) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Experimental | Simulation | Error (%) | Experimental | Simulation | Error (%) |
| 26kg | 65% | 87.21 | 83.90 | 3.8 | 170.6 | 170.4 | 0.1 |
|  | 75% | 100.41 | 96.45 | 3.9 | 170.9 | 168.2 | 1.6 |
|  | 85% | 115.38 | 112.72 | 2.3 | 169.7 | 167.1 | 1.6 |
| 39kg | 65% | 117.94 | 118.02 | 0.1 | 162.3 | 162.5 | 0.1 |
|  | 75% | 136.88 | 138.11 | 0.9 | 159.9 | 160.3 | 0.2 |
|  | 85% | 158.20 | 154.76 | 2.2 | 158.3 | 158.0 | 0.2 |
| 52kg | 65% | 147.22 | 145.13 | 1.4 | 153.4 | 148.8 | 3.1 |
|  | 75% | 171.78 | 169.24 | 1.5 | 149.6 | 146.2 | 2.3 |
|  | 85% | 196.53 | 193.24 | 1.7 | 148.3 | 141.7 | 4.7 |

Table 5: Summary of calibrated parameters

| Coefficient | Interaction | Value |
| --- | --- | --- |
| Restitution | Particle-Particle, ϵ p-p | 0.75 |
|  | Particle-Boundary ϵ p-w | 0.45 |
| Static friction | Particle-Particle, μs, p-p | 0.5 |
|  | Particle-Boundary, μs, p-w | 0.65 |
| Rolling friction | Particle-Particle, μr, p-p | 0.01 |
|  | Particle-Boundary, μr, p-w | 0.02 |

Figure 3 shows sample experiment snapshots and corresponding DEM prediction for different fill levels and speeds. The particles in the DEM snapshots are colored according to their speeds at the given instance with speed varying from low speed in blue to high speed in red. Charge behaviors snapshots of simulation compared to experiment results at same conditions are consistent in terms of the shoulder position, free surface of charge and location of bulk toe position. This consistency of DEM simulation results in terms the charge behavior of the ball mill indicates that the prediction of the ball mill environment using the calibrated parameters was reasonable.

| - 19%V, 75%C | - 28%V, 75%C | - 37%V, 75%C |
| --- | --- | --- |

Figure 3: Comparison between experimental and simulation charge profile

## Charge profile and motion

Charge motion profile influences the amount of energy available for particle breakage and affects mill wear. The particles were colored by velocity to aid in visualization. The coloring code shows the translation velocity; blue is slow and red is fast. The direction of rotation is clockwise. Figures 4  and 5 show charge motion profile for cylindrical and hexagonal mills without lifters.  At fill levels 9-37%V fill, particles were lifted up the mill and cascaded through a free surface to the toe position. Above 48%V fill and speed above 85%C cataracting streams began to break away from the shoulder position. Charge motion of cylindrical mill without lifters show particles moving up to a shoulder position then cascading on a free surface to the toe position. Cataracting streams appeared at higher mill speeds and higher media fills as shown in Figure 4. In general, the overall charge structure was ‘kidney shaped’ with a core consisted of particles moving at low speed. Force transmission from the mill to charge for this profile was predominantly due to frictional forces and was not sufficient to effectively transfer all the force due to slip. Charge surging [46], which is a phenomenon that occurs when the entire charge layer slides down and slumps changing the toe and shoulder position was observed in the cylindrical profile with lifters.

|  | 19 %V | 37%V | 54%V |
| --- | --- | --- | --- |
| 65%C |  |  |  |
| 85%C |  |  |  |
| 105%C |  |  |  |

Figure 4: Charge profile cylindrical mill profile without lifters mills

Compared to cylindrical mill, hexagonal mills without lifters showed a higher media particle lift with more expanded charge as shown in  Figure 5. The kidney shaped profile observed in cylindrical profiles was present, but the region of slow-moving particles often referred to a ‘dead zone’ was moving up and down controlled by the vertices. The hexagonal profile of the mill accumulated media particles at vertices, then the polygon sides moved the particles along the cascading free surface. Some particles tended to break off the free surface at higher speeds and higher fill levels. The charge flow was discontinuous with enhanced particle interaction. The centrifuging of charge was present at high speeds.

|  | 19 %V | 37%V | 54%V |
| --- | --- | --- | --- |
| 65%C |  |  |  |
| 85%C |  |  |  |
| 105%C |  |  |  |

Figure 5: Charge motion for hexagonal mill profile without lifters

Figure 6 shows the effect of increasing the number of polygon side all the simulations run at 75%C speed with 28%V fill. The number of particles moving at high speed decreased with increase in number of polygon sides such that the hexagonal profile accelerated particles at highest speeds and increasing the number of sides to 8, 12 and 16, particles moving at high speeds decreased with least of them observed in the 16 sided polygon. This indicates that, the angle of the vertex of a regular polygon determines the charge flow behavior. The charge motion of a 16 - sided polygon and a cylindrical mill without lifters had subtle visually observable differences.

| (a) | (b) | (c) |
| --- | --- | --- |
| (d) | (e) | (f) |

Figure 6: Charge profile different polygonal shaped mills at 28%V and 75%C;

(a) 4 side, (b) 6 side, (c) 8 side (d) 12 side (e) 16 side, (f) cylindrical

Charge profile of the hexagonal mill with six lifters was some similarities to hexagonal mill without lifter but there was enhanced particles lift, more expanded charge and greater particle interactions as shown in Figure 7. The lifters expanded charge thereby reducing dead zone and and increasing average velocity of the particles.

|  | 19 %V | 37%V | 54%V |
| --- | --- | --- | --- |
| 65%C |  |  |  |
| 85%C |  |  |  |
| 105%C |  |  |  |

Figure 7: Charge profile hexagonal mill profile, six lifters’ mills

Cylindrical mill with six lifters, produced a flow consisting of a cataracting region above the bulk charge characterized by particles raised and thrown by lifters as shown in Figure 8. At low fill levels, 9-19%V fill and speed of 75%C, particles were lifted to a region around 11 O’clock position and as the speed increased the particles started to centrifuge. The lifters transferred energy to media superseding the role of particle-mill friction in lift of particles. As the charge level increased, a kidney shaped core was formed and the tendency to cataracting was reduced due to the effects of weight and volume of overlying particles on outer particle layers.

|  | 19 %V | 37%V | 54%V |
| --- | --- | --- | --- |
| 65%C |  |  |  |
| 85%C |  |  |  |
| 105%C |  |  |  |

Figure 8: Charge profile cylindrical mill profile, six lifters’ mills

Figure 9 shows average particle velocity for the four mill profiles with increasing fill level at contant speed of mill revolution. Generally, particles in hexagonal mill with six lifters had the highest average velocity due to enhanced acceleration brought by hexagon profile and lifters. At constant mill speed, the average velocity for profiles with lifters decreased with increase in fill due to increase in the ‘dead zone’ constituted of slow moving particles. Velocity increased with increase in fill level for the mills without lifters. Wall friction dominated lifting charge for mills without lifters and it was a function of normal force but independent of speed. Increase in fill level promoted particle lift due to increaed normal force, increasing frictional force on the outer layers of the charge consequently reducing slip. The effect of polygon vertices led to increased velocity for the hexagon mill without lifters compared to the cylindrical one.  The average velocity for the the four profiles seems to be converging from 48 %V feed level with increasing speed, implying that effect of lifters  diminish at fill levels above 48%V fill at  constant speed.

(a)

(b)

Figure 9: Variation in average particle velocity with fill level for four mill configurations at, (a) 65% Critical speed, (b) 95% Critical speed

## Collisions and Collision Energy

Collision energy loss was used to describe the energy available for particle breakage during a collision event. High average interparticle collision energy loss indicates that there was more energy available for particle breakage. The energy dissipated was assumed to be used for particle breakage in this comparative study. The collision environment is described by average inter-particle collision energy as shown in  Figure 10. Cylindrical profile without lifters shows least energy dissipation that increases gradually with speed for all fill levels below 48% V. The hexagonal profile without lifters shows higher average collision energy loss with a steeper gradient compared with cylindrical profile without lifters due to higher lift and charge expansion caused by the hexagon vertices. This shows that hexagonal profiles with no lifters are efficient at high speeds, although as explained in the next section, these profiles are not power efficient at high speeds. Above 37%V fill levels and speeds the collision energy peaks and declines due to media breaking from the bulk charge. Cataracting streams of particles limited interparticle interaction but increased particle-geometry collisions. Similar peaks are expected for 19% V and 28% V fill at speeds outside the experiment range.

(a)

(b)

(c)

48% Fill Volume

19 % Fill Volume

Figure 10: Variation of inter-particles average collision energy loss with four combinations of drum shape and lifters

Variation of average collision energy loss with speed for mill profiles with lifters at all fill levels increased to peak values and declined thereafter. These mill profiles attained peak values earlier as compared to the mills without lifters due to the presence of lifters. The decline in energy dissipation was caused by particles breaking from bulk charge leading to either or both centrifugal and cataracting motion. Hexagonal profiles with lifters had the highest collision energy loss at speeds below 75% C at all fill levels which can be attributed the absence of centrifuging streams observed cylindrical profiles. The main reasons for peak reduction of collision energy loss are presence of centrifugal force and reduction increase fill.

## Power draw

The variation of power draw as function of speed for each mill profile is shown in Figure 11. As expected, increasing fill level at constant speed increased power draw due to inertia of additional mass. Cylindrical profile without lifters shows almost linear increase in power draw at 9-28% fill levels.  Higher fill levels produced peaks within the experiment range at about 85% C due to higher shoulder position that increased charge torque arm. Power draw was reduced after the peak due to streams of media particles that started to cataract. A similar trend was observed for the hexagonal profile without lifters except that linearity is reduced and the curves gradients reduce slightly due to the lifting action enhanced by vertices of the hexagon. Profiles with lifters show different trends with a notably substantial reduction in power draw attributed to efficiency of lifters. Cataracting of media is observed at speed as low as 55%V at fills of 9%-19%.  The peak power draw values shift to the right as fill increase and are approximately at 65%C, 75%C and 80%C for 28%V, 37%V and 48%V respectively.

(a)

(b)

(c)

(d)

Figure 11: Variation of power for each profile at different charge fills

(a) Hexagonal Cylindrical profile with no lifters, (b) Cylindrical profile with lifters (c) Hexagonal profile with no lifters (d) Hexagonal profile with lifters

Figure 12 shows variation power draw for the mill profiles at constant fill levels. It is clear that lifters increase power efficiency of the mills by improving energy transfer from the mill to the charge. The higher power drawn by mills without lifters is attributed to slip of charge on the walls as particle-wall friction is main mode of energy transfer. The hexagonal profile with lifters drew more power than cylindrical profile with lifters at the same speed. The vertices of hexagonal mill congregate some charge particles, elevates them and shortly after, the hexagonal sides accelerate the particles charge through the free surface or part of them cataract depending on speed of rotation. The hexagonal profile without lifters was more power efficient compared to cylindrical profile without lifters at speeds higher that 75%C and fills higher than 37%V. Figure 12 (c) shows that power for hexagonal drum without lifters, peaked at speed in the range of about 85% of critical speed, the incorporation of lifters reduced peak power draw to about 75% critical speed. This can which can be attributed to the emergence of high energy streams of charge caused by the effects of lifters.

(a)

(b)

(c)

(d)

Figure 12: Variation Power between mill profiles

Figure 13 shows variation of power draw with number of sides of different profiles ranging from six sides to a completely cylindrical surface. A rotation speed of 75%C which is in the range of peak power in most mills was selected factors held constant while varying power draw. In a mill under constant operating conditions, rotation speed of mill that produces peak power signifies high mill efficiency in terms of particle breakage [47]. Power peaked for 6-sided mill and then decreased almost linearly with increase in number of sides indicating. As the number of sides increased, the vertexes became shallower accumulating less media subsequently reducing media lift. Moreover, the decrease in the velocities of the particles reduced the energy imparted to the mill. The most commonly adopted size in practice is an octagonal shape with length of 812 mm and 762 mm across flats. The number of sides of polygonal drum is obviously dependent on the diameter of the drum. The cross-sectional area of a drum would affect the optimum number of sides and thus a scale up model can be developed in future with this perspective.

Figure 13: Effect of number of polygon sides on power draw

## Operational Stability

Ball mills operate a process of elevating particles from the toe region to the shoulder region of the drum. The process generates impact among the particles and the drum and contribute to generation of torque. The instantaneous torque on the drum was calculated from the results of DEM simulation and sample results are shown in Figure 14. The experiments were run under similar condition, operating speed 75% of critical speed and fill level 28% of mill volume. Instantaneous torque was highly variable due to impulsive and complex nature of the interactions between particles and mill. Standard deviation of instantaneous torque shown in Figure 14 was used as an indicator of operation stability. A square mill had the highest standard deviation of torque due to discontinuous motion of the charge imposed by the vertices.  As the number of vertices are reduced, the mills become more stable. Introduction of lifters to the hexagonal mill reduced the standard deviation of torque from 9.6 to 8.9. Lifters in the hexagon redirected some particles towards the toe of the mill reducing impact against the drum. Cylindrical mill without lifters had least instability, addition of lifters produced slight increase in instability.

Figure 14: Standard deviation of torque of mills

## Relative wear

Normal and tangential cumulative forces were used to indicate relative wear between the models at different conditions. Generally, the average normal cumulative geometry force increased with material fill due to increase in number of particles interacting with the mill wall. At constant fill, normal cumulative force remained almost constant with increase in speed as shown in Figure 15 (a) and (b). The hexagonal profile without lifters shows the highest relative wear at all conditions compared to other profiles as shown in Figure 15 (c) and (d). Tis ca be explained the effect of vertices increased media particle lift in the hexagonal profile. At the point on the shoulder position when gravity force overcame centrifugal forces, the accumulated particles at the vertices would be accelerated by the sides of the polygon and cascade at high speed on the free surface after which they impact the mill wall at the toe. At the toe position, the sides of the hexagon accelerated particles from hexagon side towards adjacent side through a vertex increasing impact force on the geometry. This added to the incoming cascading particles contribute to accumulation of high impact forces of on the toe position of the hexagonal mill without lifters. At high speeds, and high fill levels some cataracting streams would form and directly impact the mill leading to higher average cumulative force.

The hexagonal mill with lifters had unexpectedly low cumulative energy force compared to the profile similar profile without lifters. Addition of lifters at the center of each side parallel to the mill length changed the particle motion considerably leading to large reduction in impact force on the mill. At the shoulder position, a lifter at the center of the hexagon side elevated some media particles to a higher position expanding the charge while other particles followed cascading motion. The lifted particles would later fall on the toe position and interact with particles that had been cascading, thereby reducing particle – geometry interaction. At the toe position, lifters on hexagon profile trapped media particles and reduced the speed of the cascading particles that would have moved at high speed to the mill wall. The combined effect of the lifter at the shoulder position which created a dispersed charge inside the mill reducing particle geometry interaction and the effect of the lifter at the toe position which trapped particles limiting their acceleration to the mill wall led to overall reduction in relative wear of the mill for hexagonal mill with lifters. The cylindrical profiles showed least average cumulative force due to the effects of their geometry on charge behavior. The toe position was higher and had limited particle flow in direction opposite the mill direction which reduced impact on mill geometry. Cascading and cataracting particle streams fell on the toe position on other particles also reducing geometry impact. These profiles promoted centrifuging which reduced the intensity of geometry wear but significantly reduced availability of interparticle collision energy.

(a)

(b)

(c)

(d)

Figure 15: Variation of Cumulative Force on equipment with speed of rotation

# Conclusion

In this study, the effect of mill profile and lifters on performance was studied. In absence of lifters, polygon shaped mill have higher performance in terms of availing more collision energy to charge as compared to a cylindrical profile without lifters. The largest draw back in polygon shape as demonstrated in the case of a hexagonal mill higher predicted wear as compared to other mills in this study. Another shortcoming is the high operational instability due to the effect of vertices. Modification of the hexagonal mill by incorporation lifters improved the performance significantly in terms of increasing collision energy to particles, reducing power draw and reducing relative wear. Thus, in practice, the performance of polygonal shaped mills can be improved by addition of appropriate shape and size of lifters to improve particle breakage and reduce mill wear. A cylindrical mill profile without lifters resulted in least performance in terms of energy transfer to media particles with comparison to its large power draw. Installing lifters to cylindrical profile increased the energy transfer efficiency to particles for breakage without increasing relative wear. Installation of lifters is the conventional practice used in milling. Cylindrical mills provided good performance in terms of energy transfer to media particles at a very low wear rate and high operational stability. In this study, relative wear model used does not show the extends of wear in terms of depth which can be quantified economically, hence other wear models can be used to obtain better estimationof wear. Using DEM simulation method used was found to be reasonable in predicting behavior media particles in polygonal and cylindrical shaped mills. A criterion for selecting the number of sides for polygon sides for a scale up model can be developed and validated in future.

# Acknowledgement

The authors acknowledge the financial support from Japan International Cooperation Agency support through Africa-ai-Japan project (Year - 2018/2019; iPIC/05/18) used in fabrication of the experimental ball mill and acquisition of software. Jomo Kenyatta University of Agriculture and Technology provided support laboratory research facilities and equipment. Authors have no conflict of interest relevant to this article.

# References

[1]	S. Morrell, “The prediction of power draw in wet tumbling mills,” University of Queensland, 1993.

[2]	N. Chimwani, “an Attainable Region Approach To Optimizing Product Size Distribution for Flotation Purposes,” University of the Witwatersrand, 2014.

[3]	A. Mwanga, “Test Methods for Characterising Ore Comminution Behavior in Geometallurgy,” Luleå University of Technology, Sweden, 2014.

[4]	T. Yager, “Kenya Minerals Yearbook 2013,” in US Geological survey 2013, Area Reports: International - Africa and the Middle East Minerals Yearbook 2011, no. August, 2013, p. 6.

[5]	J. (UNITAR) De Haan and B. (UNITAR) Turner, Handbook for Developing National ASGM Formalization Strategies within National Action Plans. 2018.

[6]	M. H. Wang, R. Y. Yang, and A. B. Yu, “DEM investigation of energy distribution and particle breakage in tumbling ball mills,” Powder Technol., vol. 223, pp. 83–91, 2012.

[7]	B. K. Mishra and R. K. Rajamani, “The discrete element method for the simulation of ball mills,” Appl. Math. Model., vol. 16, pp. 598–604, 1992.

[8]	P. W. Cleary and R. D. Morrison, “Understanding fine ore breakage in a laboratory scale ball mill using DEM,” Miner. Eng., vol. 24, no. 3–4, pp. 352–366, Feb. 2011.

[9]	L. M. Tavares, “A review of advanced ball mill modelling,” KONA Powder Part. J., vol. 2017, no. 34, pp. 106–124, 2017.

[10]	X. Bian, G. Wang, H. Wang, S. Wang, and W. Lv, “Effect of lifters and mill speed on particle behaviour, torque, and power consumption of a tumbling ball mill: Experimental study and DEM simulation,” Miner. Eng., vol. 105, pp. 22–35, 2017.

[11]	DEM Solutions Ltd, WHAT IS (DEM), Theoretical background behind the Discrete Element Method. Endingburg, UK: Springer Netherlands, 2017.

[12]	B. K. Mishra and R. K. Rajamani, “The discrete element method for the simulation of ball mills,” Appl. Math. Model., vol. 16, no. November, pp. 598–604, 1992.

[13]	R. K. Rajamani, S. Rashidi, and N. Dhawan, “Advances in discrete element method application to grinding mills,” Miner. Process. Extr. Metall., vol. 100, no. Morell 1992, pp. 117–128, 2014.

[14]	C. O ’sullivan, “Particle-Based Discrete Element Modelling: A Geomechanics Overview,” in 12th International Association for Computer Methods and Advances in Geomechanics (IACMAG), 2008, pp. 498–505.

[15]	P. W. Cleary, “Predicting charge motion, power draw, segregation and wear in ball mills using discrete element methods,” Miner. Eng., vol. 11, no. 11, pp. 1061–1080, Nov. 1998.

[16]	P. W. Cleary, “Recent advances in DEM modelling of tumbling mills,” Miner. Eng., vol. 14, no. 10, pp. 1295–1319, 2001.

[17]	L. K. Nordell, A. V Potapov, and J. A. Herbst, “Comminution Simulation Using Discrete Element Method (DEM) Approach – From Single Particle Breakage to Full-Scale SAG Mill Operation,” Chem. Technol., pp. 26–29, 2003.

[18]	A. B. Makokha and M. H. Moys, “Towards optimising ball-milling capacity: Effect of lifter design,” Miner. Eng., vol. 19, no. 14, pp. 1439–1445, 2006.

[19]	I. A. Gabibov, F. M. Gamidov, and P. P. Chakraborty, “Results of Improvements of the Type Sag Mills, Used in the Azerbaijan International Mining Company,” News Ural State Min. Univ., vol. 2, no. 2(50), pp. 102–106, 2018.

[20]	Z. Yin, Y. Peng, Z. Zhu, Z. Yu, and T. Li, “Impact load behavior between different charge and lifter in a laboratory-scale mill,” Materials (Basel)., vol. 10, no. 8, 2017.

[21]	R. Panjipour and K. Barani, “The effect of ball size distribution on power draw, charge motion and breakage mechanism of tumbling ball mill by discrete element method (DEM) simulation,” Physicochem. Probl. Miner. Process, vol. 54, no. 2, pp. 258–269, 2018.

[22]	F. Pedrayes, J. G. Norniella, M. G. Melero, J. M. Menéndez-Aguado, and J. J. del Coz-Díaz, “Frequency domain characterization of torque in tumbling ball mills using DEM modelling: Application to filling level monitoring,” Powder Technol., vol. 323, pp. 433–444, 2018.

[23]	J. T. Kalala, M. M. Bwalya, and M. H. Moys, “Discrete element method (DEM) modelling of evolving mill liner profiles due to wear. Part I: DEM validation,” Miner. Eng., vol. 18, no. 15, pp. 1386–1391, 2005.

[24]	P. W. Cleary and P. Owen, “Development of models relating charge shape and power draw to SAG mill operating parameters and their use in devising mill operating strategies to account for liner wear,” Miner. Eng., vol. 117, no. December 2017, pp. 42–62, 2018.

[25]	C. O’Sullivan, Particulate discrete element modelling. 2011.

[26]	P. A. Cundall and O. D. L. Strack, “A discrete numerical model for granular assemblies,” Géotechnique, vol. 29, no. 1, pp. 47–65, Mar. 1979.

[27]	B. . Mishra, “A review of computer simulation of tumbling mills by the discrete element method: Part I—contact mechanics,” Int. J. Miner. Process., vol. 71, no. 1–4, pp. 73–93, Sep. 2003.

[28]	C. T. Jayasundara, M. Khanal, and D. Adhikary, “Application of Discrete Element Method in Grinding Mills and Composite Particle Breakage,” Neural, Parallel Sci. Comput., 2014.

[29]	H. P. Zhu, Z. Y. Zhou, R. Y. Yang, and A. B. Yu, “Discrete particle simulation of particulate systems: Theoretical developments,” Chem. Eng. Sci., vol. 62, no. 13, pp. 3378–3396, Jul. 2007.

[30]	P. Cleary, “Modelling comminution devices using DEM,” Int. J. Numer. Anal. Methods Geomech., vol. 25, no. 1, pp. 83–105, 2001.

[31]	R. D. Morrison and P. W. Cleary, “Using DEM to model ore breakage within a pilot scale SAG mill,” Miner. Eng., vol. 17, no. 11–12, pp. 1117–1124, 2004.

[32]	M. H. Abbaspour-fard, “DISCRETE ELEMENT MODELLING OF THE DYNAMIC BEHAVIOUR OF NON-SPHERICAL PARTICULATE MATERIALS,” University of Newcastle upon Tyne, 2000.

[33]	A. Podlozhnyuk, S. Pirker, and C. Kloss, “Efficient implementation of superquadric particles in Discrete Element Method within an open-source framework,” Comput. Part. Mech., vol. 4, no. 1, pp. 101–118, 2017.

[34]	R. Bharadwaj, “What is Discrete Element Method?,” 2014.

[35]	S. J. Burns and K. J. Hanley, “Establishing stable time-steps for DEM simulations of non-collinear planar collisions with linear contact laws,” Int. J. Numer. Methods Eng., vol. 110, no. 2, pp. 186–200, Apr. 2017.

[36]	M. Otsubo, C. O’Sullivan, and T. Shire, “Empirical assessment of the critical time increment in explicit particulate discrete element method simulations,” Comput. Geotech., vol. 86, pp. 67–79, 2017.

[37]	A. E. Inc, “Relative Wear,” 2022. [Online]. Available: https://2022.help.altair.com/2022/EDEM/index.htm#t=Creator%2FPhysics%2FAdditional_Models%2FRelative_Wear.htm. [Accessed: 23-Aug-2023].

[38]	R. P. King, “Technical Notes 8, Grinding,” Media, no. October. 2000.

[39]	P. Owen and P. W. Cleary, “The relationship between charge shape characteristics and fill level and lifter height for a SAG mill,” Miner. Eng., vol. 83, pp. 19–32, Nov. 2015.

[40]	Y. Peng, T. Li, Z. Zhu, S. Zou, and Z. Yin, “Discrete element method simulations of load behavior with mono-sized iron ore particles in a ball mill,” Adv. Mech. Eng., vol. 9, no. 5, pp. 1–10, 2017.

[41]	E. T. Tuzcu and R. K. Rajamani, “Modeling breakage rates in mills with impact energy spectra and ultra fast load cell data,” Miner. Eng., vol. 24, no. 3–4, pp. 252–260, 2011.

[42]	P. W. Cleary and P. Owen, “Effect of particle shape on structure of the charge and nature of energy utilisation in a SAG mill,” Miner. Eng., vol. 132, no. October 2018, pp. 48–68, 2019.

[43]	P. W. Cleary and P. Owen, “Effect of operating condition changes on the collisional environment in a SAG mill,” Miner. Eng., vol. 132, no. June, pp. 297–315, 2019.

[44]	J. Sklar, Minitab® Manual. 2013.

[45]	S. Jiang et al., “Discrete element simulation of particle motion in ball mills based on similarity,” Powder Technol., vol. 335, pp. 91–102, 2018.

[46]	S. Agrawala, R. K. Rajamani, P. Songfack, and B. K. Mishra, “Mechanics of media motion in tumbling mills with 3d discrete element method,” Miner. Eng., vol. 10, no. 2, pp. 215–227, 1997.

[47]	A. Datta, B. K. Mishra, and R. K. Rajamani, “Analysis of power draw in ball mills by the discrete element method,” Can. Metall. Q., vol. 38, no. 2, pp. 133–140, 1999.