Fault diagnosis of wind turbine bearings based on improved dung beetle optimizer optimized LSTM

Wei WEI 1, Yucen YUAN 1

1 College of New Energy and Power Engineering, Lanzhou Jiaotong University, Lanzhou, China

Correspondence: Wei WEI (weir_0719@163.com)

Abstract

Due to the nonlinearity, non-stationarity and noise interference of the rolling bearing fault signals of wind turbines, the extraction of features is challenging. This paper proposes a fault diagnosis method for wind turbine bearings based on the improved dung beetle optimizer (IDBO) optimized Long Short-Term Memory (LSTM) network. Firstly, Levy flight strategy and T-distribution perturbation strategy are integrated into the traditional dung beetle optimizer (DBO) to optimize the algorithm. The optimized IDBO algorithm solves the problems of low convergence accuracy and easy entrapment into local optima in the traditional dung beetle algorithm. Secondly, the IDBO algorithm is combined with the long short-term memory method to build the IDBO-LSTM fault diagnosis model. The advantage of fewer parameters in this algorithm enables the model to reduce the overall number of parameters and computational complexity while ensuring the accuracy of temporal feature extraction. Finally, this pa-per takes the bearing open dataset of Case Western Reserve University as an example to compare the proposed method with other models through optimization tests. The experimental results show that the IDBO-LSTM model outperforms other models, with a fault diagnosis accuracy rate of 98.3%. This demonstrates the superiority of the proposed improved dung beetle optimizer and effectively improves the accuracy of fault diagnosis for rolling bearings of wind turbines.

Keywords: fault diagnosis; wind turbine bearing; improved dung beetle optimizer; long short-serm memory

1. Introduction

Wind power generation is a clean and renewable form of energy that does not rely on limited fossil fuel resources [1]. As concerns about climate change and environmental issues continue to rise, the importance of wind power generation is becoming increasingly prominent. Wind turbine bearings are one of the key components of wind power systems, playing a crucial role in the reliability and stability of the system. During operation, wind turbine bearing failures occur at a relatively high rate. Therefore, timely and accurate diagnosis and handling of bearing failures can effectively prevent downtime and production interruptions caused by these failures, thus improving the overall availability of wind power systems. Consequently, it is essential to diagnose bearing operation failures in wind turbines in a timely manner [2].。

Traditional fault detection and diagnosis methods are classified into three categories: model-based methods, knowledge-based reasoning methods, and data-driven methods [3]. Among them, data-driven fault diagnosis methods, represented by machine learning, have distinct advantages over the other two approaches. Traditional machine learning-based fault diagnosis methods typically involve three steps: first, denoising and preprocessing of the raw data, including time-frequency domain feature analysis [4], time-frequency diagram analysis [5], and filtering [6]; then, manual feature extraction, including feature analysis and feature selection; and finally, fault identification using a classifier. While machine learning methods have achieved significant success in many fields, they also have some drawbacks, such as difficulty in handling high-dimensional and large-scale data, sensitivity to data distribution, the need for a large amount of labeled data, and limitations in model generalization capabilities. These limitations make it increasingly difficult to meet the demands of modern system fault diagnosis. Specifically, the data denoising and preprocessing process, as well as manual feature extraction, require professionals with strong expertise and considerable experience, making the performance of fault classification and identification heavily dependent on the quality of manual feature extraction. This approach has become inadequate in terms of accuracy and automation of feature extraction to meet the needs of modern industry.

The development of deep learning has gone through several important stages, from Deep Belief Networks (DBN) to Convolutional Neural Networks (CNN), and then to Recurrent Neural Networks (RNN), with each step driving significant progress in the field. First, the Deep Belief Network (DBN) was one of the early deep learning models. DBN employed a two-stage learning process involving unsupervised pre-training and supervised fine-tuning. In the unsupervised pre-training stage, the network initializes parameters by adjusting weights layer by layer. Subsequently, in the supervised fine-tuning stage, the entire network is adjusted through optimization algorithms to enhance performance on specific tasks. The emergence of DBN overcame the vanishing or exploding gradient problem encountered in training deep neural networks, advancing deep learning technology. Next, the rise of Convolutional Neural Networks (CNN) further propelled machine learning in the field of image processing. CNN used operations such as convolution and pooling, enabling the network to automatically extract local features from images and achieve translation invariance. This structure allowed CNN to achieve remarkable results in tasks such as image classification and object detection. Additionally, with the advent of optimization algorithms like batch gradient descent and stochastic gradient descent, the training speed and performance of CNNs were further improved. Finally, the introduction of Recurrent Neural Networks (RNN) provided powerful tools for handling sequential data. By incorporating self-connected nodes, RNNs could capture temporal dependencies in sequential data, leading to significant advances in tasks like natural language processing and speech recognition. However, early RNNs faced difficulties in training and issues like vanishing gradients. To address these problems, researchers proposed the Long Short-Term Memory (LSTM) model, which further expanded the applications of RNN-type neural networks [7]. Today, deep learning is gradually being applied to feature extraction and pattern classification of rolling bearing signals. For example, Qu Jianling [10] and others proposed a One-Dimensional Convolutional Neural Network (1D-CNN) for fault diagnosis of wind turbine rolling bearings, achieving good results in fault identification accuracy and generalization under different loads; Zhou Tao and others [11] proposed a rolling bearing fault diagnosis method based on Channel and Spatial Reconstruction and Progressive Convolutional Neural Networks (CSRP-CNN), achieving excellent fault diagnosis results; Gao Shuzhi and others [12] proposed a rolling bearing fault diagnosis method based on Maximum Correlated Kurtosis Deconvolution (MCKD) and CNN. The CNNs used in the above studies did not consider the temporal dependencies hidden in rolling bearing data with time-series characteristics. Compared to traditional CNNs, Long Short-Term Memory (LSTM) networks have some notable advantages, such as preventing vanishing or exploding gradients, being suitable for sequential data, and being capable of learning long-term dependencies [13]. Considering that LSTM models can automatically adapt to the complex characteristics of wind turbine bearing fault signals, they are better at extracting fault features. In this paper, LSTM is introduced as the main model for the fault feature extraction layer. However, due to its relatively complex internal structure, the training efficiency is much lower than traditional RNNs with the same computational power, the required training time is longer, and issues such as overfitting, difficulty in determining optimal parameters, and long training times remain. As working conditions become increasingly complex, the impact of these problems will become more severe, urgently requiring corresponding methods to address them [14].

In response to the above issues, this paper proposes an approach that improves the dung beetle algorithm to optimize LSTM, significantly enhancing the model's diagnostic accuracy. The optimized dung beetle algorithm LSTM combines the global search capabilities of the dung beetle algorithm with LSTM's sequence modeling ability, offering advantages such as strong global search ability, avoidance of local optima, and improved convergence speed. Finally, through experimental comparisons using data from the rolling bearing platform at Case Western Reserve University in the United States as simulation input, it is demonstrated that the proposed IDBO algorithm achieves better optimization results, and the IDBO-LSTM model diagnostic method has higher accuracy.

2. Research Method

2.1 LSTM Basic Principle

Long Short-Term Memory (LSTM) is a variant of Recurrent Neural Networks (RNN), designed to address the problem of traditional RNNs struggling to capture long-term dependencies [15]. LSTM introduces some key structures, including the cell state and gating mechanisms, which allow it to more effectively capture and process long-term dependencies in time series data. The structure of the LSTM unit is shown in Figure 1.

Figure 1.  LSTM unit structure

The LSTM unit assigns temporal features to the input data while providing time dependency. LSTM achieves long-term control through the cell state. The cell state's long-term control function is primarily implemented through three gates: the input gate, forget gate, and output gate. The cell state effectively handles the sequential filtering and updating of input data, granting it temporal dependency, which aids in the classification and prediction of time-series signals.

The learnable weights in LSTM are the input weights W、the recurrent weights R and bias b. Matrix W、R and b are respectively the concatenations of input weights, recurrent weights, and the biases for each component. The expressions are as follows:

(1)

In the expressions, i, f, and o correspond to the input gate, forget gate, and output gate, respectively; g is the candidate value. The cell state expression at time t is:

(2)

Here, ⊗ denotes the Hadamard product (element-wise multiplication of vectors). The expression for the hidden state at time t is:

(3)

In the expression, σc is the activation function. Typically, the hyperbolic tangent function is used as the activation function for the cell state in the LSTM layer.

The main function of the forget gate is to selectively discard information. Its expression is:

(4)

In the expression, ht-1 represents the information from the previous step's output, and xt represents the input data. Both are processed through the activation function σg to retain some and discard some information. The input gate's role is to handle the input at the cur-rent sequence position. Its expression is:

(5)

The role of the output gate is to determine the output value based on the cell state. Its expression is:

(6)

The candidate value can be computed as:

(7)

2.2 Basic Principle of Dung Beetle Optimization Algorithm

The Dung Beetle Optimizer (DBO) is a novel algorithm proposed by Jian Ka Xue and Bo Shen in 2022, inspired by the behaviors of dung beetles, including rolling dung balls, dancing, foraging, stealing, and reproducing [16]. This algorithm simultaneously considers global exploration and local exploitation, resulting in fast convergence speed and high accuracy.

In the DBO algorithm, the position changes of the dung beetles responsible for rolling the dung balls are as follows:

(8)

In the expression, t represents the current iteration number, and xi denotes the position of the i-th dung beetle. k is a constant representing the deflection coefficient;α∈(0,1) is a random number; b is a natural coefficient, either -1 or 1; and Xw is the global worst position.

When encountering obstacles, dung beetles will dance to reorient their direction. The position update is as follows:

(9)

In the expression, tan(θ) is the deflection angle coefficient.

To simulate the egg-laying area of female dung beetles, a boundary selection strategy is used, which is defined as:

(10)

In the expression, X* is the current best position; Lb represents the lower bound, and Ub represents the upper bound.

During the iteration process, the position of the egg ball is dynamically changing, defined as follows:

(11)

In the expression, Bi represents the position of the egg ball, and b1 and b2 are random vectors of size 1×D. Additionally, we need to establish an optimal foraging area to guide the dung beetles in their foraging behavior. The optimal foraging area is defined as follows:

(12)

In the expression, Xb is the global best position. Therefore, the position update for the small dung beetles is as follows:

(13)

In the expression, C1 is a random number following a normal distribution; C2∈(0,1) is a random vector. Additionally, assuming Xb is the best location for competing for food, the position update for dung beetles exhibiting stealing behavior is described as follows:

(14)

In the expression, g is a random vector following a normal distribution with a mean of 0 and a variance of 1; S is a constant.

2.3 Attention Mechanism

Attention Mechanism (Att) is a commonly used technique in computer science and machine learning that allows models to process sequential data more accurately and efficiently [17]. In traditional neural networks, each neuron's output depends solely on the outputs of all the neurons in the previous layer. However, in the attention mechanism, each neuron's output not only depends on the previous layer's out-puts but also assigns different weights to different parts of the input data. This allows the model to focus more on the key information within the input sequence, thereby improving the model's accuracy and efficiency. For example, in [18], an Inception-CNN-based model was used for rolling bearing fault classification tasks. After extracting fault features with CNN, an attention mechanism was added to balance the importance of different features, improving classification accuracy. In [19], a rolling bearing fault diagnosis method demonstrated higher accuracy compared to other methods and wide convolution models, achieving high detection accuracy even in environments with few labeled samples and noise, showcasing better accuracy and robustness. Experiments show that introducing the attention mechanism resulted in lower loss function values and higher model accuracy. Therefore, in fault diagnosis tasks, the essence of introducing the attention mechanism lies in filtering key information from complex fault features and weakening or filtering out redundant information to improve fault diagnosis accuracy.

3. Wind Turbine Bearing Fault Diagnosis Based on Improved Dung Beetle Algorithm Optimized LSTM

3.1 Improved Dung Beetle Optimization Algorithm

The DBO is improved by incorporating the Levy flight strategy [20] and the T-distribution perturbation strategy [21]. During the iterative optimization process, the Levy flight strategy is applied to perturb the elite dung beetle individuals and generate candidate solutions. Greedy selection is then used to eliminate dung beetle individuals with poor fitness, thereby enhancing the local optimization ability of the algorithm. In the later stages of iteration, the T-distribution perturbation mechanism is adopted to expand the dung beetles' search range, preventing them from getting stuck in local optima and significantly improving the global optimization capability of the dung beetle optimization algorithm.

3.1.1 Levy Flight Random Walk

The foraging behavior of many organisms follows a Levy flight pattern, where the step length follows a heavy-tailed probability distribution. In this paper, this mechanism is introduced into the dung beetle algorithm to further enhance the population's local exploitation and global exploration abilities.

The position update formula for Levy flight random walk is as follows:

(15)

In the expression: α is the step length factor; levy is the random step length, and the formula is as follows:

(16)

In the expression: v is a random number following a standard normal distribution; μ is a random number following a normal distribution with a mean of 0 and a standard deviation of σ, where the formula for σ is as follows:

(17)

In the expression: β is set to 1.5; α is set to 1; Γ represents the Euler second integral (Gamma function).

3.1.2 T-distribution Perturbation Strategy

In a T-distribution, as the degrees of freedom decrease, the curve becomes flatter, the middle of the curve lowers, and the tails on both sides of the curve lift more significantly. Using T-distribution to perturb the positions of dung beetle individuals allows for population variation. Then, the objective function value for the mutated dung beetle positions is updated. If the result is better than the previous generation's optimal function value, the current optimal value is updated. The T-distribution perturbation formula is as follows:

(18)

In the expression: xti is the new position of the i-th cuckoo nest in the population after mutation; xi is the individual position before mutation; t(iter) is the T-distribution value, with the degrees of freedom determined by the number of iterations. In the early stages of algorithm iteration, the iter value is small, and the result generated by the T-distribution function approximates Cauchy mutation, with strong global search capability. In the later stages, as the iter value increases, the mutation becomes similar to Gaussian mutation, possessing good local search capability, thereby improving the algorithm's search accuracy.

3.2 IDBO-LSTM Algorithm Basic Principle

Given the non-stationarity, noise interference, significant individual variability, and low signal-to-noise ratio characteristics of rolling bearing data, and the fact that existing models, such as CNN alone, do not account for the hidden temporal dependencies in rolling bearing data, while LSTM alone does not leverage the powerful spatial feature extraction capabilities of CNN, this paper proposes a hybrid deep learning model based on the dung beetle algorithm for wind turbine rolling bearing fault diagnosis, known as IDBO-LSTM [22]. This approach addresses the issue where standard CNNs, due to having too many convolutional layers, tend to overfit the model when training on rolling bearing data. Compared to LSTM and CNN, the advantage of IDBO-LSTM having fewer parameters allows the model to ensure the accuracy of temporal feature extraction while reducing the overall parameter count and computational load. The model first takes the number of neurons in the first layer of the LSTM network, s1, the number of neurons in the second layer, s2, the learning rate, e, and the number of iterations, m, as the optimization targets of the IDBO algorithm. The particle positions are randomly initialized based on the value range of each parameter. Then, the LSTM neural network model is established using the particle positions corresponding to these parameters. The system's monitoring data is used to train and predict with the ID-BO-LSTM model. The mean square error of the model is taken as the fitness value for the improved particle swarm algorithm, and the fitness function is de-fined as follows:

(19)

In the expression, n is the number of test data samples; yi=30 represents the true value of the test data; yi' is the predicted value of the test data. By comparing the fitness values of each particle, the current individual best bpesti and the current global best solution gbesti can be obtained. Finally, using the optimal particle's position information, the optimized neural network model is constructed to complete the establishment of the IDBO-LSTM model.

3.3 Fault Diagnosis Process Based on IDBO-LSTM Optimization Algorithm

The LSTM model optimized by the dung beetle algorithm will be used for fault diagnosis of wind turbine rolling bearings. The specific process flowchart is shown in Figure 2.

Step 1: Define the objective function f(x) and initialize the positions of the dung beetle individuals X= (1, …, m). This includes: function initialization, randomly generating n initial positions Xi (i=1, 2, …, n) for the dung beetles, and setting parameters such as population size and maximum number of iterations；

Step 2: Select the fitness function and calculate the objective function value for each dung beetle position to obtain the current optimal function value and the corresponding individual positions；

Step 3: Update the positions of all dung beetles. Combine the Levy flight strategy to calculate the objective function values for candidate individual positions, compare them with the previous generation's optimal function value, and if they are better, update the current population's optimal individual positions；

Step 4: Calculate the distance between the positions of other dung beetles and the best position. Compute the objective function values for the candidate individual positions. Compare these values with the previous generation's optimal function value, and if the new values are better, update the current population's optimal individual positions；

Figure 2.  Fault diagnosis flowchart based on IDBO-LSTM

Step 5: Use the T-distribution perturbation strategy to update the positions of the other dung beetles. Calculate the optimal function values of the up-dated dung beetle positions, and compare them with the previous generation's optimal values. If the new values are better, update the current optimal value；

Step 6: Check if the maximum iteration count has been reached or if the minimum error requirement has been satisfied. If so, return to Step 2; otherwise, proceed to the next step；

Step 7: Output the global optimal individual position and the optimal function value.

4. Experimental Analysis

4.1 Description of Experimental Dataset

The experimental dataset used in this study is sourced from the Case Western Reserve University Bearing Dataset. This dataset is one of the most renowned and widely used datasets in the field of bearing fault diagnosis. It is provided by the Bearing Data Center of Case Western Reserve University in the United States, aimed at offering benchmark data for researchers in bearing fault diagnosis. The dataset includes different types of bearing faults, such as inner ring faults, outer ring faults, and rolling element faults, as well as normal operating conditions of bearings. During data collection, accelerometers were installed at the drive end and fan end of the bearings to record vibration signals. The dataset has a high signal-to-noise ratio, which is conducive to extracting fault features. In the experiment, the bearing load was uniformly set to 1HP, and the sampling frequency was 12kHz, with fault diameters of 0.1778 and 0.3556. The fault locations were the inner ring, rolling element, and outer ring, along with the selection of nor-mal state bearing signals. For each bearing condition, 100 sets of data were selected (a total  of 700 samples), with each sample containing 1,024 data points. Out of these 700 samples, 560 were randomly selected as training samples, while the remaining 140 were used as testing samples, as shown in Table 1.

Table 1.  Data label division

| Fault location | Diameter/mm |  |
| --- | --- | --- |
|  | 0.1778 | 0.3556 |
| Inner ring | Inner1 | Inner2 |
| Rolling fault | Ball1 | Ball2 |
| Outer ring | Outer1 | Outer2 |
| Normal | Normal |  |

Scaling input data to eliminate the impact of units on the model is a standard process in preparing data for deep learning models. This method normalizes the input data by adjusting the mean and standard deviation so that the data has zero mean and unit standard deviation, as follows:

(20)

In the expression, Sin refers to the input data sequence.

Figure 3 shows the original vibration signals of the faults. From Figure 3, it can be observed that there are certain differences between different rolling bearing faults. However, judging the type of bearing fault solely based on the original vibration signal is insufficient. Therefore, to improve the efficiency of bearing fault diagnosis, it is necessary to extract deeper feature information. Time-domain characteristic parameters can be used to determine whether there is a fault in the rolling bearing. However, to accurately identify the detailed information of the fault, such as type, location, and severity, frequency characteristics of the rolling bearing must be analyzed. Thus, a com-bination of time-domain and frequency-domain parameters is selected as the fault feature parameters for rolling bearing fault diagnosis.

Figure 3.  Original Vibration Signals Corresponding to Different Fault Diameters

4.2 Experimental Verification

This paper proposes an IDBO-LSTM-Att wind turbine bearing fault diagnosis model, which com-bines the DBO algorithm with Lévy flight strategy and T-distribution perturbation, and incorporates an attention mechanism. The training set and test set are divided in a 7:3 ratio.  The test results are presented in Figure 4.

Figure 4. Comparison of Test Results for the IDBO-LSTM Model

To visually demonstrate the diagnostic performance of the model, a confusion matrix is utilized to visualize the classification results on the test set. To mitigate the potential bias introduced by a single data split, the input data is randomly shuffled 10 times, resulting in different training and test sets for each iteration. The average of the 10 diagnostic outcomes is taken as the final accuracy of the model, as shown in Figure 5. This approach proves that the proposed method exhibits excellent classification performance, along with robust and superior generalization capabilities.

(a) Training set confusion matrix

(b) Test set confusion matrix

Figure 5. Confusion Matrices for the Test and Training Sets

4.2.1 Fault Feature Extraction Layer Comparison Experiment

Figure 6 presents the comparison of experimental results based on different fault feature extraction layers. Compared to RNN and BP, LSTM ad-dresses the issues of gradient vanishing and gradient explosion in traditional RNNs by introducing memory cells and gating mechanisms. LSTM is better equipped to capture long-term dependencies in time-series data, making it excel in tasks involving long sequences and long-term dependencies. The primary advantages of LSTM over traditional BP neural networks lie in its superior ability to process sequential data and its significant edge in mitigating gradient vanishing problems. Figure 6 illustrates the change in accuracy and loss values of LSTM, BP, and RNN models as the number of network iterations in-creases. It can be observed that LSTM outperforms the other two models significantly in terms of both accuracy and loss values.

(a) accuracy

(b) magnitude of the loss

Figure 6. The variation process of accuracy and loss values of three models with increasing iteration times

4.2.2 Optimization Algorithm Comparison Experiment

Through fault diagnosis experiments, the effectiveness and superiority of the proposed model in this paper are validated. To achieve this, several optimization algorithms including DBO, IDBO, Sparrow Search Algorithm (SSA), Whale Optimization Algorithm (WOA), Grey Wolf Optimizer (GWO), and Northern Goshawk Optimization (NGO) are selected for a comparative study on test function optimization. To ensure fairness, common parameters among all algorithms are set consistently, with the population size and maximum number of iterations set to 100 and 500 respectively. To mitigate errors arising from randomness, each test function is subjected to 10 independent trials. The average fitness iteration convergence curves are plotted and compared based on con-vergence speed and iteration count. Figure 7 illustrates these comparisons.

Figure 7. Accuracy and loss values of the test set

As can be observed from Figure 7, under the same level of accuracy, among the six optimization algorithms, IDBO requires the fewest iterations, indicating that the introduction of the Lévy flight strategy to eliminate beetle individuals with poor fitness im-proves the population quality, thereby enhancing the convergence speed. This demonstrates the strong local search capability of the IDBO algorithm. As the number of iterations increases, the convergence curves of DBO, SSA, WOA, GWO, and NGO tend to flatten out, indicating stagnation, and their optimization accuracy remains relatively low. In contrast, the IDBO algorithm exhibits a fluctuating decrease, suggesting that the incorporation of the T-distribution strategy helps the algorithm escape from local extrema and enhances its global search capability.

(a) Comparison of Accuracy of Test Set in Two Models

(b) Comparison of loss values of the test set in two models

Figure 8. Comparison of Accuracy and Loss Values of Test Set in Two Models

Through fault diagnosis experiments, the effectiveness and superiority of the proposed model in this paper are validated. The DBO and IDBO models are sequentially trained, with a gradient descent batch size of 100 and a total of 500 epochs. After each epoch of training, the classification accuracy and loss values on the test set are observed. Accuracy reflects the degree of agreement between predicted and true categories, while loss value indicates the discrepancy between the model's predictions and the actual values.

Figure 9. Accuracy and loss values of DBO and IDBO test sets

As shown in Figure 8, and with Figure 9 illustrating the com-parison of loss values between the two models on the test set, it can be observed that: The DBO model exhibits unstable training performance, achieving its best results around the 400th iteration but failing to maintain this performance, indicating poor robustness. In contrast, the IDBO model achieves a training accuracy close to 100% around the 90th iteration and maintains this level until the end of training. Simultaneously, the loss value ultimately converges to zero, indicating that the predicted values are extremely close to the target values. This demonstrates that the proposed model in this paper possesses strong robustness.

Figure 10. Average accuracy of fault classification for each model

As depicted in Figure 10, the test set accuracy rates of the various models are 96%, 84%, 82.5%, 94%, 95.5%, and 97.8%, respectively. For the same fault data, the fault diagnosis accuracy rates of the WOA and GWO models are below 90%, while the other four models achieve an accuracy of over 90%. Notably, the IDBO-LSTM-Att model achieves a fault diagnosis accuracy rate of 97.8%, outperforming the other five models. This result underscores the effectiveness of the method proposed in this paper.

4.2.3 Optimization Algorithm Comparison Experiment

To validate the necessity of incorporating the attention mechanism, this paper constructs two models, IDBO-LSTM and IDBO-LSTM-Att, for experimental comparison. The experimental results based on different models are presented in Table 2.

Table 2.  Experimental results based on different models

| Metrics | models |  |
| --- | --- | --- |
|  | IDBO-LSTM | IDBO-LSTM-Att |
| accuracy | 97.8 | 98.3 |

The core idea of the attention mechanism is to assign a weight value to each element in the input sequence, where these weights determine the degree of focus the model gives to different parts of the input sequence during processing. In the context of wind turbine bearing fault diagnosis, the attention mechanism layer can automatically learn and adjust the attention given to different input features, enabling the model to more effectively capture key information related to faults. Specifically, the attention mechanism can dynamically adjust the model's focus on different parts of the input sequence at each time step, allowing the model to concentrate more on the time periods or features related to fault characteristics, thereby reducing the interference from irrelevant information. As seen in Table 2, the addition of the attention mechanism layer results in a 0.5% improvement in accuracy. In the diagnosis results of wind turbine bearing data, the accuracy of the IDBO-LSTM-Att hybrid algorithm is higher than that of the IDBO-LSTM algorithm. Experiments show that by incorporating the attention mechanism layer into IDBO-LSTM, the model's performance in detecting wind turbine bearing faults can be further enhanced. Such a model can better utilize the information in time-series data, effectively identify abnormal patterns and fault signals in bearings, thereby enabling early warning and diagnosis of wind turbine bearing faults, reducing maintenance costs, and improving the reliability and availability of wind power equipment.

5. conclusion

Addressing the issues of weak fault characteristics in wind turbine rolling bearings, where corresponding signals exhibit strong nonlinearity, non-stationarity, and noise interference, as well as the inherent spatial and temporal information contained in the signals of wind turbine rolling bearings, this paper proposes a fault diagnosis method that utilizes LSTM neural networks to extract frequency domain features from vibration signals, combined with the classification advantages of IDBO. The specific summary is as follows:：

1) A novel IDBO algorithm is proposed. Through comparisons with DBO, SSA, WOA, GWO, and NGO algorithms, it is demonstrated that the ID-BO algorithm exhibits superior capabilities in escaping local minima, achieving overall optimization synergy, and converging faster.

2) The IDBO-LSTM model is utilized for fault diagnosis, and experimental comparisons are con-ducted. The fault diagnosis method adopted in this paper achieves a diagnostic accuracy rate of up to 98.3%, demonstrating higher accuracy and superior performance compared to the other five models.

3) In the IDBO-LSTM model, an attention mechanism layer is introduced and compared against a model without this layer. After incorporating the attention mechanism layer, the fault diagnosis accuracy rate improves by 0.5%. This result validates the effectiveness and practicality of using an attention mechanism for wind turbine bearing fault diagnosis.

Therefore, the method proposed in this paper demonstrates certain advantages in achieving bearing fault diagnosis through vibration signals. Further re-search can be conducted to explore the performance of this method under different operating conditions in wind turbine bearing fault diagnosis.

References

- RENY, SUGANTHAN PN, SRIKANTHN. A novel empirical mode decomposition with support vector regression for wind speed forecasting[J]. IEEE Transactions on Neural Networks and Learning Systems, 2023, 27(8): 1793-1798

- Guo Junfeng, Sun Lei, Wang Miaosheng, etc. Fault diagnosis method for rolling bearings based on 1D LeNet-5 model [J] Journal of Lanzhou University of Technology, 2023, 49 (05): 34-41.

- BP A, MN B, GT C. Recursive variational mode extraction and its application in rolling bearing fault diagnosis[J]. Mechanical Systems and Signal Processing, 2022, 165: 108-321．

- Jing Dongxing, Chen Yanghui, Quan Zhe. Wind turbine bearing fault diagnosis method based on 1D-CNN and SWLSTM [J] Mechanical strength, 2023, 45 (06): 1309-1317.

- Xie Dongdong, Shen Yanxia. Wind turbine bearing fault diag-nosis based on CLPSO-IDBN [J] Combination Machine Tool and Automation Processing Technology, 2023 (10): 109-119.

- Dou Jinxin, Xue Zhengkun, Yu Xiaoguang, etc. Fault diagnosis of aviation hydraulic pipeline clamps based on optimized CNN [J] Machine Tool and Hydraulic, 2022 (16): 188-194.

- LI X, YI X, LIU Z, et al. Application of novel hybrid deep leaning model for cleaner production in a paper industrial wastewater treatment system[J]. Journal of Cleaner Production, 2023(2): 126-343.

- NIU G, X YI, CHEN C, et al. A novel effluent quality pre-dicting model based on genetic-deep belief network algorithm for cleaner production in a full-scale paper-making wastewater treatment[J]. Journal of Cleaner Production, 2022(265): 121-787.

- FAN J, BI S, XUR, et al. Hybrid lightweight deep-learning model for sensor-fusion basketball shooting-posture recogni-tion[J]. Measurement, 2022(189): 110-595.

- Zhang Tao, Wang Chaoyang, Wu Xinhui, etc. Fault diagnosis of motor bearings based on multidimensional features and IGWS-SVM[J] Journal of Weapon Equipment Engineering, 2023, 44 (09): 149-154.

- Qu Jianling, Yu Lu, Yuan Tao, etc. Adaptive fault diagnosis algorithm for rolling bearings based on one-dimensional con-volutional neural network [J] Journal of Instrumentation, 2018, 39 (7): 134-143.

- Zhou Tao, Yao Dechen, Yang Jianwei. Fault diagnosis of rolling bearings based on channel and spatial recombination network [J/OL] Mechanical strength, 2024, 41 (8): 21-25.

- Gao Shuzhi, Shi Shuo, Zhang Yimin. Fault diagnosis of rolling bearings based on adaptive MCKD and CNN [J/OL] Mechan-ical Design and Manufacturing, 2024, 56 (12): 35-40.

- Wang Pengcheng, Deng Aidong, Ling Feng, etc. Fault diagno-sis of rolling bearings in wind turbines based on PSO-SEBD [J] Vibration and Shock, 2023, 42 (07): 281-288.

- Cao Mengzhou, Zhang Yan. Classification of Power Quality Disturbances Based on Convolutional Long Short Term Memory Networks [J] Power System Protection and Control, 2020, 48 (2): 7.

- Wei Wei, Zhao Xiaoqiang, Ding Yanhua, etc Fault diagnosis of high-speed rail onboard equipment based on BiLSTM and improved attention mechanism [J] Journal of Railway, 2022, 44 (11): 53-62.

- Liu Yayue, Zhao Qian. CNN-LSTM ultra short term power load forecasting based on clustering empirical mode decomposi-tion[J]. Power Grid Technology, 2021, 45 (11): 4444-4451.

- Jiang Jian, Wei Xiaoyuan. Fault diagnosis of rolling bearings under adaptive attention LSTM ResNet [J/OL]. Manufacturing Technology and Machine Tools, 2023, 34 (18): 53-61.

- Gui Binbin, Li Zelan, Liu Yongzhi, etc. Fault diagnosis of fan rolling bearings based on wavelet packet and PSO-BP[J]. China's New Technologies and Products, 2023 (11): 5-9.

- Zhu Hao, Ning Qian, Lei Yinjie, etc. Rolling bearing fault classification based on attention mechanism-Reception CNN model[J]. Vibration and Shock, 2020, 39 (19): 84-93.

- Zhu Xiangbing, Li Yuanjiang, Wang Jianhua, etc. Wind driven optimization algorithm based on Levy flight mechanism[J]. Computer and Digital Engineering, 2023, 46 (10): 1943-1950.

- Li Bin, Gao Peng, Guo Ziqiang. Improving the Beetle Algo-rithm to Optimize LSTM for Photovoltaic Array Fault Diagno-sis[J/OL]. Journal of Power Systems and Automation, 2023, 24 (9): 1-11.