Experimental study on identification of aerodynamic damping matrix for an operating wind turbine by artificial excitation

Zhaowei Wang1,2,3, Shengxin Lu4, Ningyu Wu4, Xugang Hua1,2,3, Chao Chen1,2,3*

1Key Laboratory for Wind and Bridge Engineering, Hunan University, Changsha, China

2College of Civil Engineering, Hunan University, Changsha, China

3State Key Laboratory of Bridge Engineering Safety and Resilience, Hunan University, Changsha, China

4Hebei Construction Investment Offshore Wind Power Co., Ltd., Tangshan, China

Experimental study on identification of aerodynamic damping matrix for an operating wind turbine by artificial excitation

Abstract

Quantitation of damping is of great significance for the design and condition assessment of wind turbines. The authors’ previous theoretical and numerical studies showed that compared to damping ratios, a modal aerodynamic damping matrix can better describe the damping coupling in the fore-aft (FA) and side-side (SS) tower motions. In the present study, an improved damping identification method was first proposed to identify this damping matrix with artificial exciters and then verified by using OpenFAST simulations under different excitation frequencies, excitation force amplitudes, and different turbulent wind fields. Following the numerical study, a scaled wind turbine model with a geometric scale ratio of 1/75 was carefully designed based on the NREL 5 MW wind turbine prototype, in which the scaled blade design follows the rule in thrust coefficient similarity. An identification study was performed with this scaled model by a series of wind tunnel tests. The modal aerodynamic damping matrix was identified under steady-state harmonic excitation in the operating state and compared with the identified results by a free decay method and the theoretical values. The results experimentally confirm the correctness of the aerodynamic damping matrix theory under uniform wind and the feasibility of the improved identification method in practice.

Keywords: wind turbine; aerodynamic damping; damping identification; wind-tunnel test;

Introduction

Rapid development in wind energy has been witnessed in recent years, and it is expected that the total capacity of newly installed onshore wind turbines in the world will exceed 100 GW in 2024 [1]. The breakthrough of wind turbine technology suitable for low-wind-speed areas and the development of megawatt-level horizontal axis wind turbine manufacturing technology have stimulated the rapid growth of wind power installation, leading to the increasing height of wind turbine towers and rotor diameters [2]. Meanwhile, the wind turbine structural system is becoming more flexible, and issues related to vibration, such as large vibration amplitude and fast fatigue damage accumulation, are becoming increasingly prominent. Damping estimation is essential for accurate dynamic analysis of wind turbines and follow-on risk assessment, as damping is a key parameter to limit the vibration amplitude.

Damping in onshore wind turbines mainly includes aerodynamic damping, structural damping, soil/foundation damping and damping from dissipative devices. Currently, two methods are mainly adopted to quantify damping in wind turbines: theoretical derivation and on-site measurement. Some studies have shown that aerodynamic damping plays a dominant role among different types of damping [3,4], and many researchers attempted to use analytical formulae to quantify aerodynamic damping. Valamanesh and Myers [5] derived the theoretical expressions of aerodynamic damping based on the assumption of rigid blades, zero shaft inclination and yaw angle. Chen et al. [6] modified the traditional aerodynamic damping theory expression by considering flexible blades, small shaft inclination and yaw angle. Xi et al. [7] proposed a semi-analytical solution for the modal aerodynamic damping ratio based on Blade Element Momentum (BEM) theory by considering the translational and rotational degrees of freedom of the tower top. Chen et al. [8] considered the wind turbine tower’s motions in the FA and SS directions simultaneously and derived the theoretical expression of the asymmetric aerodynamic damping matrix based on BEM theory, successfully explaining the aerodynamic damping coupling phenomenon in the FA and SS directions for operating wind turbines. They further introduced the modal aerodynamic damping matrix of the two-degree-of-freedom wind turbine model based on the modal superposition method, and proposed an aerodynamic damping matrix identification method based on frequency response functions (FRFs) and harmonic excitations [9]. Liu et al. [10] compared the effect of two different analytical expressions of aerodynamic damping for the fatigue analysis of wind turbines. Although the above theoretical derivations clarify the mechanism of aerodynamic damping, they are based on some idealised assumptions, and are only verified by simplified numerical simulations, lacking relevant experimental verification.

For on-site measurements, artificial excitations can be first applied on wind turbines and then vibration data are collected for identification. For instance, Hansen et al. [11] excited a wind turbine by changing the pitch angles of the blades, while Osgood et al. [12] released the steel wire rope tensioned at the top of the tower. Koukoura et al. [13] used a ship to hit an offshore wind turbine, and Oh et al. [14] installed an active mass damper to excite an onshore wind turbine so that the resultant free decay responses can be used to identify the modal parameters. Moreover, "overspeed stop" tests were frequently conducted to identify wind turbine modal parameters in on-site measurements [15–17]. Usually, artificially excitations are difficult to apply and require installing additional devices, so wind turbine vibration responses collected under ambient excitations are more often used for modal identification. For example, Ozbek and Rixen [18] used photogrammetry and laser interferometry to measure the response of wind turbines under environmental excitation and conducted modal analysis. Hu et al.[19] successfully identified the modal parameters of wind turbines using the least squares complex frequency-domain method under environmental load excitation, while Dong et al. [20,21] and Dai et al. [22] have improved the traditional stochastic subspace identification method to solve the harmonic interference problem caused by wind turbine rotation [23,24]. However, structural damping from aerodynamic damping cannot be separated from the total damping measured on site, and a large amount of data under ambient excitations is needed to accurately identify the modal parameters. It is more difficult to artificially excite free decay responses when wind turbines are operating.

Wind tunnel tests are important to study wind-induced dynamic responses and verify wind-resistant designs. Compared with on-site measurements for wind turbines, wind tunnel tests have advantages in better controlling variables in a laboratory condition. With the development of the similarity theory and the maturity of wind tunnel test technology, it becomes possible to carry out wind tunnel tests in terms of wind turbine aerodynamic damping. The scale effect caused by the difference in Reynolds number has been successfully solved by selecting low Reynolds number airfoils to redesign the model blade [25–30]. In the early study by Sinclair [31] in 1994, the results of aerodynamic damping from wind tunnel tests were compared with the theoretical values, and it was found the experimental results fluctuated around the theoretical values. More recently, Fontecha et al. [32] investigated the aerodynamic damping of a wind turbine using the forced vibration method in wind tunnel tests. Meng et al. [33] designed a wind turbine model with a scale ratio of 1/100 and identified the aerodynamic damping of the model. Zhang et al. [34] identified the modal parameters of the scaled wind turbine model using Kalman filtering, stochastic subspace technique and stochastic subspace identification method. However, the above wind tunnel tests only investigated the aerodynamic damping ratio in the FA direction but did not consider the aerodynamic coupling phenomenon between the FA and SS directions.

In this paper, the identification method of modal aerodynamic damping matrix proposed in [9] was first improved. With a carefully designed scaled model of a wind turbine, a wind-tunnel experimental study was conducted to verify the feasibility of the theory and identification method associated with modal aerodynamic damping matrix proposed in the authors’ previous studies [8,9]. This paper is organized as follows. Section 2 introduces the theory of modal aerodynamic damping matrix and the identification method based on steady-state harmonic excitation. Section 3 verifies the identification effect under turbulent wind with different excitation frequencies, excitation force amplitudes, and mean wind speeds based on the NREL 5 MW fully-coupled OpenFAST numerical analysis model. Section 4 introduces the design of the wind turbine aeroelastic scaled model based on the similarity of thrust coefficients and shows the test results of the thrust coefficients. The identification results of modal aerodynamic damping matrix under steady-state harmonic excitation and the free decay identification results are compared with the theoretical values. Section 5 concludes the paper.

Identification method

Theoretical expression of the modal aerodynamic damping matrix

For wind turbines in operation, Chen et al.[8] found that there is a coupling of aerodynamic damping between towers’ fore-aft and side-side vibrations, and conventionally used damping ratios cannot reflect this aerodynamic coupling phenomenon. The derivation of aerodynamic damping matrix can be briefly introduced as follows. In an operating wind turbine, the relative wind velocity seen by each blade element is first determined by considering the inflow wind speed and the effect of the rotor’s translation and rotation caused by tower vibration, then the resultant aerodynamic loads on the tower from the rotor can be calculated by BEM Theory. Next, it is found that the aerodynamic loads on the tower top can be divided into two parts: a static components and a “damping” component proportional to the velocities[8]. The “damping” component can be written by an aerodynamic damping matrix multiplied by a velocity vector. The aerodynamic damping matrix is derived under the assumption of constant inflow wind field. Considering a turbulent and non-uniform wind field, a more general expression for the aerodynamic damping matrix CAero[9] is obtained as

|  | CAero=cxxcxycyxcyycxθxcxθycyθxcyθycθxxcθxycθyxcθyycθxθxcθxθycθyθxcθyθy, | (1) |
| --- | --- | --- |

where x and y denote translational motions in the FA and SS directions respectively, and θx and θy denote rotational motions about x and y axes respectively. The terms in Eq. (1) can be added at relevant positions in the damping matrix of the tower top nodes if a finite element model is created to model the wind turbine. Furthermore, a 22 modal aerodynamic damping matrix can be derived via the modal reduction technique applied to the equation of motion of the finite element model. If only considering the dominant first bending modes of the wind turbine tower, the modal reduction technique leads to a 2-DOF modal model. The derivation of the 2-DOF model and 22 modal aerodynamic damping matrix is briefly introduced in Appendix A. The modal aerodynamic damping matrix can be expressed by

|  | CAero=cxxcxycyxcyy, | (2) |
| --- | --- | --- |

where cxy and cyx are non-zero and not identical, making the modal aerodynamic damping matrix asymmetric. This modal aerodynamic damping matrix is set as the identification target throughout this study.

When a turbulent inflow wind field is considered, the modal aerodynamic damping matrix becomes time-varying, but under a turbulent wind field Chen [35] verified that the response computed by the 2-DOF model with a constant modal aerodynamic damping matrix is very close to that of the fully-coupled FAST model, which demonstrates that the constant modal aerodynamic damping matrix can adequately represent the damping characteristics of the wind turbine for non-uniform and time-varying wind fields.

Improve identification method based on steady-state harmonic excitation

Chen et al.[9] proposed an method (details in Appendix B) to identify the modal aerodynamic damping matrix based on FRFs and harmonic excitations, but the damping identification method requires prior knowledge of the mass and stiffness matrices which can be obtained by traditional OMA methods on a parked turbine, possibly causing accumulation of identification errors. For a NREL 5 MW wind turbine, when the amplitudes of the harmonic excitation is lower than 5 kN, the damping matrix estimation is poor, and it is very difficult to design a shaker suitable to be installed at the top of wind turbine towers with the excitation force’s amplitude larger than 5 kN. Furthermore, the identification result by this method could be distorted when the frequency of the harmonic excitation is close to the wind turbine's natural frequency[35]. To overcome the above deficiencies, an improved version of the modal damping matrix identification method proposed by Chen et al.[9] is introduced in this section.

Given a dynamic system with mass, stiffness and damping matrices denoted as M, K and C respectively, the FRF matrix Haω can be computed given the measured acceleration responses after the system is excited by a harmonic load F(ω) with the frequency ω. According to the general damping matrix identification method proposed by Chen et al.[36], the damping matrix for a given FRFs can be expressed as:

|  | C=-ωHaNω-1Gaω, | (3) |
| --- | --- | --- |

where

|  | Gaω=-HIaωHRaω-1. | (4) |
| --- | --- | --- |

The real part and imaginary part of Haω are HRaω and HIaω respectively. Chen[9] proposed that the undamped or “normal” FRF matrix HaNω can be determined based on the known stiffness and mass matrices by HaNω=-ω2M+K-1. The relationship between Haω, Gaω, and HaNω is according to [36]:

|  | HaNω=I+iGaωHaω, | (5) |
| --- | --- | --- |

in which HaNω and Gaω are real matrices. Inserting the expressions of the real and imaginary parts of Haω into Eq. (5), we can obtain

|  | HaNω=HRaω-GaωHIaω+iGaωHRaω+HIaω. | (6) |
| --- | --- | --- |

The left-hand side of Eq. (6) must be a real matrix, so the imaginary part of the right-hand side must be a zero matrix, and thus the following expression can be obtained:

|  | HaNω=HRaω-GaωHIaω. | (7) |
| --- | --- | --- |

This means HaNω can be calculated from the measured FRF Haω directly. Consequently, given the measured FRF matrix, the damping matrix can be obtained by combing Eqs. (3), (4) and (7).

Numerical model and analysis results

Fully coupled numerical analysis model in OpenFAST

A wind turbine model was established in OpenFAST to generate dynamic responses under harmonic excitations and test the newly developed identification method. A NREL 5 MW reference onshore wind turbine model was modelled and its basic properties are shown in Table 1, assuming the tower bottom is clamped to ground. The schematic of the wind turbine model under steady-state harmonic excitations is shown in Fig. 1. To be consistent with the derivation of aerodynamic damping matrix, the blades are assumed to be rigid. Also, the centre of mass of the RNA is moved to the tower top, and the moment of inertia of the RNA relative to the tower top is set to zero. The wind loads on the tower are calculated with a drag coefficient of the quasi-stationary aerodynamic model. The settings “classic BEM theory with the Prandtl and Glauert’s corrections” were chosen in OpenFAST’s AeroDyn module[37]. The structural damping of the tower was set to be default in OpenFAST and the measured structural damping ratio for the first bending mode is 0.37%. In order to apply the steady-state harmonic excitation, the modified OpenFAST code is compiled using Microsoft Visual Studio 2015 to generate a new OpenFAST executable file, which can be used to apply harmonic forces with different frequencies and amplitudes in the FA/SS directions of at the tower top[9].

Table 1 Basic properties of the modified NREL 5 MW reference onshore wind turbine

| Basic properties | Numerical value |
| --- | --- |
| Rotor Diameter, R | 126 m |
| Tower Height | 87.6 m |
| Hub Height | 90 m |
| Rotor Mass | 110000 kg |
| Nacelle Mass | 240000 kg |
| Tower Diameter, D | 3.87-6.00 m |
| Tower Thickness, t | 19-27 mm |
| Cut-In, Rated, Cut-Out Wind Speed | 3 m/s, 11.4 m/s, 25 m/s |
| Cut-In, Rated Rotor Speed | 6.9 rpm, 12.1 rpm |
| Natural Frequency | 0.34 Hz |

Fig. 1. Schematic of the NREL 5 MW wind turbine under steady-state harmonic excitations.

The turbulent wind field was generated by the wind field generator-TurbSim[38]. The height and width of the wind field were set to 178 m and 180 m to cover the entire wind turbine, and the wind field was divided into a 21 × 10 rectangular grids. Fig. 2 shows the generated wind field with a mean wind speed of 20 m/s at the hub and a turbulence intensity of 10%. Fig. 2 (a) shows the time series of the wind speed at the hub height, and Fig. 2 (b) shows the spatial distribution of wind speeds at different moments.

| (a)Wind speed time series at hub height | (b)Wind field distribution of wind speed |
| --- | --- |

Fig. 2. Turbulent wind field applied to the NREL 5 MW numerical model.

Identification results at different harmonic excitation frequencies

A series of 2000 s acceleration time series under steady-state harmonic excitations were generated by the OpenFAST model and used to calculate the FRF matrix. The mean wind speed and the rotor rotation speed were set to 20 m/s and 12.1 rpm, while the blade pitch angle and the turbulence intensity were set to 17.47°and 10%. The harmonic excitation frequency varies from 0.2 Hz to 0.5 Hz, and this range contains the resonance frequency of 0.34 Hz. The frequency increment for the harmonic forces is 0.01 Hz, except for the range from 0.32 Hz to 0.36 Hz where a smaller frequency increment of 0.004 Hz was chosen to obtain higher resolutions around the resonance frequency. The harmonic force amplitude was set to 10 kN, and the sensitivity of excitation amplitude will be discussed in the next section.

The identification results under different harmonic excitation frequencies are compared to the theoretical values and shown in Fig. 3. When using the identification method proposed by Chen et al. [37], there is a sudden jump of the identified modal damping values when the harmonic excitation of frequency is near the natural frequency. But the improved identification method in this paper does not cause this sudden jump. It can be found that the identification results are more accurate when the excitation frequency is selected near the natural frequency. In the excitation frequency range far from the natural frequency, the identification results of the two identification methods are almost the same.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig. 3. Comparison of the analytical and identified modal damping matrix coefficients cxxa, cxyb , cyxc and cyyd under different harmonic excitation frequencies with data from the OpenFAST model.

Identification results for different harmonic excitation amplitudes

In order to further verify the identification effect of the improved identification method under different excitation force amplitudes, the same conditions with the mean wind speed of 20 m/s and turbulence intensity of 10% were selected. The amplitude of the harmonic excitation force was changed from 500 N to 10000 N in steps of 500 N. In order to avoid the distortion of the identification results, the excitation force frequency of the identification method proposed by Chen was set to 0.4 Hz[35], while the excitation force frequency of the improved identification method was set to the natural frequency of 0.34 Hz. The comparison of the identification results under different excitation force amplitudes is shown in Fig. 4. The identification results by Chen’s identification method have large errors when the excitation force amplitude is less than 5 kN, while the improved identification method proposed in this paper can still obtain relatively accurate identification results for all items of the damping matrix when the excitation force amplitude is 1 kN.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig. 4. Comparison of the analytical and identified modal damping matrix coefficients cxxa, cxyb ; cyxc and cyyd under different harmonic force amplitudes with data from the OpenFAST model.

Identification results under different wind speeds

In order to further verify the identification effect of the improved identification method in this paper under different wind speeds, the mean wind speeds from 3 m/s~25 m/s in 1 m/s steps were selected. The harmonic excitation frequency of the improved identification method was set to 0.34 Hz and the amplitude of the excitation force was set to 1 kN. The excitation frequency of the identification method proposed by Chen was set to 0.4 Hz[35], and the amplitude of the excitation force was set to 10 kN. The identification results are shown in Fig. 5. The relative dispersion (RD) is used to quantify the differences between the identified values and theoretical values. Considering the theoretical value vector CA and the identification value vector CI of the aerodynamic damping coefficients, the RD is defined as:

| RD=CATCI2CATCACITCI, | (8) |
| --- | --- |

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig. 5. Comparison of the analytical and identified modal damping matrix coefficients  cxxa, cxyb ; cyxc and cyyd with harmonic force excitations for mean wind speeds from 3 m/s to 25 m/s in 1 m/s steps with data from the OpenFAST model.

For the identified values by Chen’s identification method and those by the improved identification, the RD values of cxx are 0.985 and 0.992 respectively, the RD values of cxy are 0.793 and 0.986 respectively, the RD values of cyx are 0.981 and 0.990 respectively and the RD values of cyy are 0.936 and 0.999 respectively. It can be seen that the improved identification method can still achieve more accurate identification results with the excitation force amplitude of 1 kN for wind speeds ranging from 3 m/s to 25 m/s. Importantly, it is possible to practically install an exciter at the tower top that can produce excitation forces with an amplitude of 1 kN and thus realize the identification of damping matrix in field tests.

Design of wind turbine scaled model and wind tunnel test results

Similarity criteria and design parameters

The design parameters of wind tunnel test models are usually determined by dimensional analysis based on the geometric scale ratio, and typically consider some dimensionless parameters such as Reynolds number, Froude number, Strouhal number, Cauchy number, etc. However, it is difficult for a scaled model to simultaneously satisfy all similarity rules [29]. The prototype of this scaled model is the NREL 5 MW onshore fixed wind turbine, and the influence of gravity on the aerodynamic performance of the model is relatively small. Therefore, the similarity in Froude number is not strictly met in this scaled model design. This experiment was conducted in the first test section of the HD-2 wind tunnel laboratory for large boundary layer at Hunan University, which has a length of 17 m, width of 3 m and height of 2.5 m, and the blockage rate is 29.56%. The geometric scale ratio of this scaled model is 1:75, and Table 2 provides some design parameters of the NREL 5 MW wind turbine model.

Table 2 NREL 5 MW wind turbine model design parameters

| Argument | Prototype | Similarity Coefficient | Scaled Model Parameter |
| --- | --- | --- | --- |
| Blade Length(m) | 63 | 1/n | 0.84 |
| Individual Blade Mass(kg) | 17740 | 1/n3 | 0.0421 |
| Hub Mass(kg) | 56780 | 1/n3 | 0.1346 |
| Nacelle Mass(kg) | 240000 | 1/n3 | 0.5689 |
| Tower Mass(kg) | 347460 | 1/n3 | 0.8236 |
| Blades+ Hub + Nacelle: Tower | 1:1 | - | 1:1 |
| Tower Height(m) | 87.6 | 1/n | 1.168 |

Design and machining of blade models

Maintaining the same tip speed ratio (TSR) ensures that the Strouhal number is similar. The similarity of the thrust coefficient with the variation of blade element TSR can ensure the aerodynamic similarity for the scaled wind turbine model[39]. In order to reduce the influence of scaling effect brought by the difference of Reynolds number, the design is based on the optimisation of the single SD7032 airfoil along the blade spanwise direction and the blade data is provided by Hu[40]. The cross sections of the NACA64-618 airfoil and SD7032 airfoil are shown in Fig. 6. The low-Reynolds-number SD7032 airfoil has a significantly thinner cross-section than that of the NACA64-618 airfoil. The lift and drag coefficients for the SD7032 airfoil and the NACA64-618 airfoil at small angles of attack (-10° to 25°) were calculated using the open-source blade design tool QBlade [41], and the comparison results are shown in Fig. 7(a). With different Reynolds numbers, the lift and drag coefficients calculated for the two airfoils are close when the angle of attack is from -10° to 12°. The lift and drag coefficient curve of the SD7032 airfoil was extended for the angles of attack in the range of -180°~180° using QBlade [41], and the calculation results are shown in Fig. 7(b).

Fig. 6. Section shape comparison of NACA64-618 airfoil and SD7032 airfoil.

| (a) | (b) |
| --- | --- |

Fig. 7. Comparison of lift and drag coefficients for NACA64-618 airfoil and SD7032 airfoil at angles of attack from -10° to 25° (a) and comparison of lifting and drag coefficients within ±180° angle of attack for SD7032 airfoil (Re=40350) (b).

Fig. 8 (a) shows the curves of thrust coefficients of the NREL 5 MW prototype blade, the blade designed exactly according to the geometric scaling ratio and the optimized blade with the varying TSRs. Fig. 8 (a) shows that there is a significant difference between the thrust coefficients of the blade designed completely according to the geometric scaling ratio and the prototype blade, but the thrust coefficients of the optimized blade apparently better agree with that of the prototype blade. To further verify the similarity between the optimised blade and the prototype blade, the thrust ratios of all airfoils from the blade root to the tip for the optimised blade and prototype blades were calculated by the BEM theory under the rated wind speed condition, as shown in Fig. 8 (b). The thrust ratio curve of the optimised blade well matches that of the prototype blade, illustrating the rationality of the single airfoil optimization method. The blade of the scaled wind turbine model is made of carbon fiber and epoxy resin glue in a phenolic plastic mold with good high-temperature resistance. The weight of a single blade is 138 g, which is 3.28 times of the target mass, and the scaled blade is shown in Fig. 9.

| (a) | (b) |
| --- | --- |

Fig. 8. Thrust coefficient curves with respect to TSR (a) and blade element thrust ratio along radial direction (b).

Fig. 9. The scaled blade.

Design of other parts of the wind turbine model

Wind turbine scaled models can be usually classified as motor-driven models or upstream wind-driven models when considering different ways to drive the rotor rotation. He et al.[39] recommends the use of servo motor-driven models which can be actively controlled, as in this way TSRs can be accurately adjusted. Therefore, the scaled wind turbine model in this study uses a servo motor with a rated output of 200 W and a 1:5 reducer as the drive device for rotor rotation. The model blade is overweight due to the limitation of the production material. Therefore, it is necessary to adjust the nacelle mass and tower mass so that expected vibration behaviour can be obtained. Tower stiffness was also appropriately modified to obtain the required dynamic characteristics. A stainless steel tube with an outer diameter of 40 mm and an inner diameter of 37 mm is used to provide stiffness and support the scaled rotor-nacelle assembly according to the design method of equivalent stiffness. The outer shell was fabricated by 3D printing using PE plastic material to mimic the aerodynamic shape of the tower. The components of the wind turbine model are shown in Fig. 10.

Fig. 10. The components of the wind turbine scaled model.

Wind tunnel test instrument selection and arrangement

To measure the thrust coefficient of the wind turbine model, a six-component balance with a maximum range of 500 N was used for the force measurement device, and the force measurement device is shown in Fig. 11 (a). The artificial excitation device is a small shaker with the maximum excitation force of 20 N. The frequency and amplitude of the shaker can be adjusted by a sweep signal generator. The amplitude of the excitation force is measured by a dynamic force sensor. The wind speed for each test condition was measured by an anemometer fixed at 5 m in front of the model. Accelerometers were arranged at the top back and left center of the tower to determine the vibration response of the wind turbine model during the test. In addition, accelerometers were arranged in the fore-aft and side-side directions at the bottom of the wind turbine model at distances of 40 cm, 60 cm, 80 cm, and at the tower top, and the harmonic excitation device is shown in Fig. 11 (b).

| (a) | (b) |
| --- | --- |

Fig. 11. Arrangement of force measurement test device (a) and harmonic excitation device (b).

Correction of wind tunnel test blockage rate

Considering the blockage effect caused by the rotation of the blade of the wind turbine in the wind tunnel test process, the wind tunnel walls will affect the aerodynamic performance of the wind turbine model. According to experience, the swept area of the wind turbine model impeller should not exceed 10% of the cross-sectional area of the wind tunnel test section, in order to ensure that the target aerodynamic forces are not affected by the wall interference[42]. In literature, Fitzgerald[43] compared the correction methods of wind tunnel blockage rate proposed by Glauert, Hackett-Wilsden, Sørensen and Mikkelsen[44] through wind tunnel tests. Ryi et al.[45] used a scaled model of the wind turbine to set different blockage rates to verify the correctness of Glauert’s proposed blockage effect correction method. This paper uses Glauert’s correction method for wind tunnel test blockage effect [43] and this method can be expressed by:

|  | v'=1-τA2×C×1+2×τv, | (9) |
| --- | --- | --- |
|  | τ=TρAv2, | (10) |

where v' is the corrected equivalent wind speed, v is the measured wind speed of the wind tunnel test, T is the thrust measured in the wind tunnel test, ρ is the air density, A is the cross-sectional area of propeller, and C is the cross-sectional area of the wind tunnel.

Analysis of wind tunnel test results

Wind turbine model dynamic characteristics measurement results

The scaled model is excited by the shaker in the FA and SS directions respectively, and the acceleration responses in the FA and SS directions were collected. The natural frequency and structural damping ratio of the scaled model were calculated from the free decay curves of acceleration in the parked condition. Fig. 12 shows the measured free decay curves and power spectrums for the parked wind turbine model. The natural frequency and the structural damping ratio in the FA direction are fFA=6.7 Hz and ξFA=2.86% respectively, and these values in the SS direction are fSS=7.66 Hz and ξSS=3.02% respectively. By performing power spectrum calculation on the free decay responses at different measuring points, the first-order mode shapes are identified by normalizing the power spectrum values of each measuring point, as shown in Fig. 13. The first-order mode shape obtained from the test is very similar to that of the NREL 5 MW prototype wind turbine, indicating that the scaled model design is reasonable.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig. 12. Free decay curves in FA direction (a) and SS direction (c), power spectrum in FA direction (b) and SS direction (d).

| (a) | (b) |
| --- | --- |

Fig. 13. First-order mode shapes identified by the free decay response in FA direction (a) and SS direction (b).

Thrust coefficient measurement results

Verification of the aerodynamic performance of the model blades is key to ensure the rationality of the wind turbine model design. Considering the fast rotor rotation speed and the blockage effect by the blades, the wind loads applied on the tower during the test only accounts for a small proportion of the total wind load on the entire wind turbine. Therefore, the thrust at the bottom of the wind turbine model measured by the force measuring balance can be approximately regarded as equal to the thrust on the rotor.

In this paper, the thrust coefficient measurement test was carried out under the operating conditions of pitch angles -6°, -4°, -2°, 0°, 2°, 4°, 6°, 8°, and 10° in a uniform inflow wind field. During the test, the wind speed was first measured after the inflow wind field became stable, then the rotational speed of rotor was set, and finally, the data from the six-component balance was collected. As the blockage rate has a high value of 29.56%, the measured wind speed in the wind tunnel was corrected using the blockage effect correction method proposed by Glauert[43]. The thrust coefficient test results at 0° pitch angle were compared with the design target value, and the thrust coefficient measurement results at different pitch angles are shown in Fig. 14.

The measured thrust coefficient in the wind tunnel test is very close to the target value calculated by the QBlade [41] at 0° pitch angle. The processing error of the blade and the wind loads on the tower may cause the differences in the measured thrust coefficients. Compared with the blades designed according to the geometric scale ratio, the aerodynamic performance of the blades has been greatly improved. At the same TSR, the thrust coefficient shows a gradually decreasing trend with the increase of the pitch angle. In the range of pitch angles from -6° to 8°, the thrust coefficient increases with the increase of the TSR, but the growth rate of the thrust coefficient is gradually decreasing. At 10° pitch angle, the thrust coefficient first increases and then decreases with the increase of the TSR.

| (a) | (b) |
| --- | --- |

Fig. 14. Measured and target thrust coefficients with respect to TSR at 0° pitch angle (a) and measured thrust coefficient at different pitch angles.

Identification results of modal aerodynamic damping matrix

In the wind tunnel test for the modal aerodynamic damping matrix identification, the wind turbine rotor speed was set to 30 rpm, the excitation frequency was set to 6 Hz, and the wind tunnel test wind speed was from 4 m/s and 20 m/s. The test conditions were selected at pitch angles of -4°, -2°, 0°, and 2°. The steady-state harmonic excitation was applied to the top of the tower in the FA and SS directions using a shaker, and the acceleration responses of the tower top in the FA and SS directions are shown in Fig. 15. The harmonic excitation measured by the dynamic force sensor is shown in Fig. 16. The free decays in the FA and SS directions were also measured in the operating state, using the shaker to apply an initial force. The free decay responses are shown in Fig. 17.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig. 15. Acceleration responses in the FA direction (a) and SS direction (b) under steady-state harmonic excitation in the FA direction, acceleration responses in the FA direction (c) and SS direction (d) under steady-state harmonic excitation in the SS direction.

Fig. 16. Steady-state harmonic excitation load by the dynamic force sensor.

| (a) | (b) |
| --- | --- |

Fig. 17. Free decay response in the FA direction (a) and SS direction (b) at each measurement point of the wind turbine tower under pitch angle of 0° and uniform wind of 6 m/s.

Using the improved damping matrix identification method proposed in this paper, the modal damping matrix can be identified from the measured acceleration responses under harmonic excitations. From the free decay responses, the damping ratio can be calculated by curve fitting and then converted into the diagonal terms in the damping matrix by the relation between the damping coefficient and the damping ratio for the first mode. It should be noted the identified damping matrix accounts for total damping of the scaled model. The acquisition of separate modal aerodynamic damping is to subtract the identified structural damping in the parked state from the identified total damping. The identified damping coefficients are compared with the theoretical values in Fig. 18 and Fig. 19 for different pitch angles. The following conclusions can be drawn from the comparison between the calculated theoretical values and the identified values by the wind tunnel test:

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig. 18. Theoretical value and identification results of the modal aerodynamic damping matrix: cxx and cyx (a), cxy and cyy (b) identified at -4° pitch angle, cxx and cyx (c), cxy and cyy (d) identified at -2° pitch angle.

(1) The absolute value of each item in the modal aerodynamic damping matrix of the scaled model increases when the wind speed increases. Among them, the cxx representing the non-coupling aerodynamic damping in the FA direction is greater than the diagonal term cyy representing the non-coupling aerodynamic damping in the SS direction. Compared to the diagonal terms, cxy and cyx representing the aerodynamic coupling between the FA and SS directions are relatively small.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig. 19. Theoretical value and identification results of the modal aerodynamic damping matrix: cxx and cyx (a), cxy and cyy (b) identified at 0° pitch angle, cxx and cyx (c), cxy and cyy (d) identified at 2° pitch angle.

(2) Under the uniform wind condition, the identified values in the aerodynamic damping matrix fluctuate near the theoretical values, successfully verifying the correctness of the derived theoretical expressions of the modal aerodynamic damping matrix. The error in the identification results could come from the error in blade processing, the assumption of blade rigidity when calculating the theoretical value, and the influence of the RNA inertia moment. Additionally, it was found that the fast rotor rotation (more than 30 rpm) can cause an apparent vibration component in the tower top accelerations with the rotation frequency, and filtering this vibration component can lead to some error in identification results.

(3) The diagonal elements of the modal aerodynamic damping matrix identified from the free decay responses are generally larger than the theoretical values. At a pitch angle of -2°, for the identified values by the free decay method and those by the improved identification, the RD values of cxx are 0.9525 and 0.9977 respectively, and the RD values of cyy are 0.9228 and 0.9541 respectively. This indicates that the identified results by the free decay method are more dispersed relative to the theoretical values compared to the identified values by the improved identification method.

(4) The identified values of the off-diagonal elements of the modal aerodynamic damping matrix also fluctuate around the theoretical values. Compared with the traditional identification method, the improved identification method in this paper is proved to be able to capture the aerodynamic coupling in the FA and SS direction.

(5) Combined with the findings in the studies by Chen et al.[8,9] where the modal aerodynamic damping matrix theory is developed, the wind tunnel test results confirm the existence of aerodynamic damping coupling effect in practice and further indicate that the modal aerodynamic damping matrix can better reflect the actual damping characteristics of wind turbines.

Research limitations and prospects

The blockage rate for the wind tunnel test is relatively large (29.56%) and may have some influence on the test results’ accuracy even with corrections. To verify the influence of the large blockage rate on the thrust coefficient measurement, a comparative test was carried out in the second test section of the large boundary layer HD-2 wind tunnel laboratory at Hunan University. This test section is larger with a width of 5.5 m and a height of 4.4 m, but it has a maximum wind speed of 6 m/s and less steady wind field. For the scaled wind turbine model, the blockage rate in the second test section is 9.16% and does not exceed the normally adopted blockage rate limit of 10%. In this case, the measured results are not disturbed by the wall surface[42], so the blockage rate is not corrected. The measured thrust coefficients in different conditions are shown in Fig. 20. When the blockage rate is 29.56%, the corrected thrust coefficient curve is under the measured thrust coefficient curve when the blockage rate is 9.16% and closer to the target curve obtained by QBlade [41], which further confirms the rationality of the wind tunnel test configuration.

In practical wind tunnel tests, it is hard to completely avoid the effect caused by large blockage rates due to the limited test section dimensions. It is shown in the literature [45] that the Glauert’s method can effectively reduce the influence of large blockage rats at lower thrust and lower speed. Therefore, in order to ensure the accuracy of the test results, the rotor rotation speed was set to 30 rpm and the wind tunnel test wind speed was from 4 m/s and 20 m/s.

Fig. 20. Comparison of the measured thrust coefficients and numerically calculated values.

The proposed identification method with artificial exciters can accurately identify the damping matrix under a small excitation force amplitude. Although this paper only presents a wind-tunnel study, it is completely possible to design exciters with light masses to generate steady-state excitations to excite full-size wind turbines. In the literature, exciters have been used to excite wind turbine components. For example, Thomsen et al.[46]realized the excitation of wind turbine blades in the operating state by using the exciter installed in the nacelle. In addition, active tuned mass dampers could be used to harmonically excite the wind turbine (if installed), and this method was recommended by Oh and Ishihara[14].

Conclusions

This paper first develops the improved modal damping matrix identification method with artificial excitation and numerically verifies this method by OpenFAST simulations. This provides a theoretical basis for using an exciter to provide steady-state harmonic excitation for wind turbine modal parameter identification. In addition, the wind tunnel test study of aerodynamic damping matrix identification shows that the modal aerodynamic damping matrix identification results based on FRFs under steady-state harmonic excitation are quite close to the modal aerodynamic damping matrix theoretical values at different pitch angles. Furthermore, the identified values by the free decay method in the operating state verify the reliability of the aerodynamic damping matrix theory. The present study shows that the modal aerodynamic damping matrix derived by Chen et al.[8,9] under uniform wind can reflect the actual damping characteristics of the wind turbine, and the improved identification method in this paper has practical feasibility to identify damping values. As this paper only conducted relevant research on aerodynamic damping under the uniform wind fields in a controlled laboratory condition, it is necessary to study the feasibility of the identification method under turbulent wind conditions and the applicability of the modal aerodynamic damping matrix theory for full-size wind turbines in the future.

Acknowledgements

The financial supports from the National Natural Science Foundation of China (No. 52108280), the National Science Fund for Distinguished Young Scholars (No. 52025082) and the Natural Science Foundation of Hunan Province (No. 2023JJ70001). are greatly appreciated.

- References

[1]	Y.M. Berkmen, A. Lande, GWEC, Global wind report 2023, Global Wind Energy Council report, 2023., Am J Roentgenol Radium Ther Nucl Med 125 (1975).

[2]	S. Rodrigues, C. Restrepo, E. Kontos, R. Teixeira Pinto, P. Bauer, Trends of offshore wind projects, Renewable and Sustainable Energy Reviews 49 (2015) 1114–1135.

[3]	L. Xiong, Z. Xianmin, L. Gangqiang, C. Yan, Y. Zhiquan, Dynamic Response Analysis of the Rotating Blade of Horizontal Axis Wind Turbine, Wind Engineering 34 (2010) 543–559.

[4]	C. Chen, P. Duffour, Modelling damping sources in monopile-supported offshore wind turbines, Wind Energy 21 (2018) 1121–1140.

[5]	V. Valamanesh, A.T. Myers, Aerodynamic Damping and Seismic Response of Horizontal Axis Wind Turbine Towers, J. Struct. Eng. 140 (2014) 04014090.

[6]	Y. Chen, D. Wu, Y. Yu, W. Gao, An improved theory in the determination of aerodynamic damping for a horizontal axis wind turbine (HAWT), Journal of Wind Engineering and Industrial Aerodynamics 213 (2021) 104619.

[7]	R. Xi, P. Wang, X. Du, K. Xu, C. Xu, J. Jia, A semi-analytical model of aerodynamic damping for horizontal axis wind turbines and its applications, Ocean Engineering 214 (2020) 107861.

[8]	C. Chen, P. Duffour, P. Fromme, Modelling wind turbine tower-rotor interaction through an aerodynamic damping matrix, Journal of Sound and Vibration 489 (2020) 115667.

[9]	C. Chen, P. Duffour, K. Dai, Y. Wang, P. Fromme, Identification of aerodynamic damping matrix for operating wind turbines, Mechanical Systems and Signal Processing 154 (2021) 107568.

[10]	X. Liu, C. Lu, G. Li, A. Godbole, Y. Chen, Effects of aerodynamic damping on the tower load of offshore horizontal axis wind turbines, Applied Energy 204 (2017) 1101–1114.

[11]	M.H. Hansen, K. Thomsen, P. Fuglsang, T. Knudsen, Two methods for estimating aeroelastic damping of operational wind turbine modes from experiments, Wind Energy. 9 (2006) 179–191.

[12]	R. Osgood, G. Bir, H. Mutha, B. Peeters, M. Luczak, G. Sablon, Full-scale modal wind turbine tests: comparing shaker excitation with wind excitation, in: T. Proulx (Ed.), Structural Dynamics and Renewable Energy, Volume 1, Springer New York, New York, NY, 2011: pp. 113–124.

[13]	C. Koukoura, A. Natarajan, A. Vesth, Identification of support structure damping of a full scale offshore wind turbine in normal operation, Renewable Energy 81 (2015) 882–895.

[14]	S. Oh, T. Ishihara, Structural parameter identification of a 2.4 MW bottom fixed wind turbine by excitation test using active mass damper, Wind Energy 21 (2018) 1232–1238.

[15]	M. Damgaard, L.B. Ibsen, L.V. Andersen, J.K.F. Andersen, Cross-wind modal properties of offshore wind turbines identified by full scale testing, Journal of Wind Engineering and Industrial Aerodynamics 116 (2013) 94–108.

[16]	C. Devriendt, P.J. Jordaens, G. De Sitter, P. Guillaume, Damping estimation of an offshore wind turbine on a monopile foundation, IET Renewable Power Generation 7 (2013) 401–412.

[17]	R. Shirzadeh, C. Devriendt, M.A. Bidakhvidi, P. Guillaume, Experimental and computational damping estimation of an offshore wind turbine on a monopile foundation, Journal of Wind Engineering and Industrial Aerodynamics 120 (2013) 96–106.

[18]	M. Ozbek, D.J. Rixen, Operational modal analysis of a 2.5 MW wind turbine using optical measurement techniques and strain gauges: Operational modal analysis of a 2.5 MW wind turbine, Wind Energy. 16 (2013) 367–381.

[19]	W.-H. Hu, S. Thöns, R.G. Rohrmann, S. Said, W. Rücker, Vibration-based structural health monitoring of a wind turbine system. Part I: Resonance phenomenon, Engineering Structures 89 (2015) 260–272.

[20]	X. Dong, J. Lian, M. Yang, H. Wang, Operational modal identification of offshore wind turbine structure based on modified stochastic subspace identification method considering harmonic interference, Journal of Renewable and Sustainable Energy 6 (2014) 033128.

[21]	X. Dong, J. Lian, H. Wang, T. Yu, Y. Zhao, Structural vibration monitoring and operational modal analysis of offshore wind turbine structure, Ocean Engineering 150 (2018) 280–297.

[22]	K. Dai, Y. Wang, Y. Huang, W. Zhu, Y. Xu, Development of a modified stochastic subspace identification method for rapid structural assessment of in‐service utility‐scale wind turbine towers, Wind Energy. 20 (2017) 1687–1710.

[23]	D. Tcherniak, S. Chauhan, M.H. Hansen, Applicability Limits of Operational Modal Analysis to Operational Wind Turbines, in: T. Proulx (Ed.), Structural Dynamics and Renewable Energy, Volume 1, Springer New York, New York, NY, 2011: pp. 317–327.

[24]	M. Ozbek, F. Meng, D.J. Rixen, Challenges in testing and monitoring the in-operation vibration characteristics of wind turbines, Mechanical Systems and Signal Processing 41 (2013) 649–666.

[25]	I. Bayati, M. Belloli, L. Bernini, A. Zasso, Aerodynamic design methodology for wind tunnel tests of wind turbine rotors, Journal of Wind Engineering and Industrial Aerodynamics 167 (2017) 217–227.

[26]	Y. Ma, C. Chen, T. Fan, H. Lu, J. Fang, An innovative aerodynamic design methodology of wind turbine blade models for wind tunnel real-time hybrid tests based on genetic algorithm, Ocean Engineering 257 (2022) 111724.

[27]	J. Chen, Z. Hu, D. Wan, Q. Xiao, Comparisons of the dynamical characteristics of a semi-submersible floating offshore wind turbine based on two different blade concepts, Ocean Engineering 153 (2018) 305–318.

[28]	E.-J. de Ridder, W. Otto, G.-J. Zondervan, F. Huijs, G. Vaz, Development of a Scaled-Down Floating Wind Turbine for Offshore Basin Testing, in: Volume 9A: Ocean Renewable Energy, American Society of Mechanical Engineers, San Francisco, California, USA, 2014: p. V09AT09A027.

[29]	C.L. Bottasso, F. Campagnolo, V. Petrović, Wind tunnel testing of scaled wind turbine models: Beyond aerodynamics, Journal of Wind Engineering and Industrial Aerodynamics 127 (2014) 11–28.

[30]	I. Bayati, A. Facchinetti, A. Fontanella, H. Giberti, M. Belloli, A wind tunnel/HIL setup for integrated tests of Floating Offshore Wind Turbines, J. Phys.: Conf. Ser. 1037 (2018) 052025.

[31]	F.M. Sinclair, Aerodynamic damping on offshore installations — a comparison of experimental measurements with theory, Journal of Wind Engineering and Industrial Aerodynamics 52 (1994) 321–344.

[32]	R. Fontecha, F. Kemper, M. Feldmann, On the Determination of the Aerodynamic Damping of Wind Turbines Using the Forced Oscillations Method in Wind Tunnel Experiments, Energies 12 (2019) 2452.

[33]	J. Meng, K. Dai, Z. Zhao, Z. Mao, A. Camara, S. Zhang, Z. Mei, Study on the aerodynamic damping for the seismic analysis of wind turbines in operation, Renewable Energy 159 (2020) 1224–1242.

[34]	P. Zhang, Z. He, C. Cui, L. Ren, R. Yao, Operational Modal Analysis of Offshore Wind Turbine Tower under Ambient Excitation, JMSE 10 (2022) 1963.

[35]	C. Chen, Damping in Monopile-Supported Offshore Wind Turbines, University College London (2020).

[36]	S.Y. Chen, M.S. Ju, Y.G. Tsuei, Estimation of Mass, Stiffness and Damping Matrices from Frequency Response Functions, Journal of Vibration and Acoustics 118 (1996) 78–82.

[37]	P.J. Moriarty, A.C. Hansen, AeroDyn Theory Manual, 2005.

[38]	B.J. Jonkman, Turbsim User’s Guide: Version 1.50, 2009.

[39]	R. He, H. Sun, X. Gao, H. Yang, Wind tunnel tests for wind turbines: A state-of-the-art review, Renewable and Sustainable Energy Reviews 166 (2022) 112675.

[40]	Y. Hu, Experimental study on time-varying structural performance of wind turbines based on aero-optimization model, Harbin Institute of Technology (2021).

[41]	D. Marten, QBlade Short Manual v08, TU Berlin (2014).

[42]	P.-Å. Krogstad, P.E. Eriksen, “Blind test” calculations of the performance and wake development for a model wind turbine, Renewable Energy 50 (2013) 325–333.

[43]	R.E. Fitzgerald, Wind tunnel blockage corrections for propellers, University of Maryland (2007).

[44]	J.N. Sorensen, W.Z. Shen, R. Mikkelsen, Wall Correction Model for Wind Tunnels with Open Test Section, AIAA Journal 44 (2006) 1890–1894.

[45]	J. Ryi, W. Rhee, U. Chang Hwang, J.-S. Choi, Blockage effect correction for a scaled wind turbine rotor by using wind tunnel test data, Renewable Energy 79 (2015) 227–235.

[46]	K. Thomsen, J.T. Petersen, E. Nim, S. Øye, B. Petersen, A Method for Determination of Damping for Edgewise Blade Vibrations, Wind Energy 3 (2000) 233–246.

[47]	C. Chen, P. Duffour, P. Fromme, X. Hua, Numerically efficient fatigue life prediction of offshore wind turbines using aerodynamic decoupling, Renewable Energy 178 (2021) 1421–1434.

- Appendix A

Modal aerodynamic damping matrix derivation

The equation of motion for a wind turbine system can be written as

|  | Mut+CStrucut+Kut=FFlext, | (A. 1) |
| --- | --- | --- |

where M, CStruc and K are the mass, structural damping and stiffness matrices respectively. FFlext is the external force vector, which is the aerodynamic force applied to a flexible tower and includes FTopFlext at the nodes at the tower top. and ut is the generalised displacement vector. Combining the BEM theory and tower dynamics, the aerodynamic forces applied to the flexible tower can be linearised to the sum of terms corresponding to the forces for an assumed rigid tower, plus terms proportional to the tower top linear and angular velocities[47], which can be expressed as

|  | FTopFlext=FTopRigidt-CAerouTopt, | (A. 2) |
| --- | --- | --- |

CAero has been introduced in Eq. (1). After substituting Eq. (A. 2) into Eq. (A. 1), the equation of motion changes to

|  | Mut+(CStruc+C'Aero)ut+Kut=FRigidt, | (A. 3) |
| --- | --- | --- |

where C'Aero is the aerodynamic damping matrix that contains the terms in CAero, and CStruc is the structural damping matrix. FRigidt represent the aerodynamic force vector applied to the rigid tower, including FTopRigidt at the nodes at the tower top. A partial decoupling can be obtained from the equations of motion in Eq. (A. 3) using the normal (undamped) mode shapes. The undamped mode shape matrix Ψ can be computed from the eigen analysis of the mass and stiffness matrices M and K. The displacement vector ut can then be written as the generalised coordinate vector α(t) multiplied by the mode shape matrix: ut=Ψα(t). Multiplying the transpose of the modal shape matrix, ΨT, at both sides of Eq. (A. 3), we obtain

|  | ΨTMΨαt+ΨTCStruc+CAeroΨαt+ΨTKΨαt=ΨTFRigidt. | (A. 4) |
| --- | --- | --- |

The equation above can be written in concise form:

|  | Mαt+CStruc+CAeroαt+Kαt=FRigid, | (A. 5) |
| --- | --- | --- |

where M and K are the modal mass and stiffness matrices, CStruc, and CAero are the modal structural and aerodynamic damping matrices, FRigid is the modal wind force vector. If only the first bending modes in the FA and SS directions are considered, CStruc, and CAero can be expressed by

|  | CAero+CStruc=cxx+2ζxmxkxcxycyxcyy+2ζymyky. | (A. 6) |
| --- | --- | --- |

where CAero has been shown in Eq. (2), mx, my, kx and ky are the modal masses and stiffnesses for the first FA/SS mode respectively, ζx and ζy are the structural modal damping ratios.

- Appendix B

Identification method used in Ref. [9]

Given a dynamic system with mass, stiffness and damping matrices M, K and C excited by an external force f, the equation of motion is

| Mu+Cu+Ku=f. | (B. 1) |
| --- | --- |

Rewrite this equation of motion in the frequency domain:

| (-ω2M+iωC+K)Xω=Fω. | (B. 2) |
| --- | --- |

The frequency response function matrix Hω is defined as:

| Hω=(-ω2M+iωC+K)-1. | (B. 3) |
| --- | --- |

The “normal” FRF HNω is defined with the undamped system:

| HNω=K-ω2M-1 | (B. 4) |
| --- | --- |

With the “normal” FRF, the frequency domain equation of motion can be written as

| HNω-1Xω+iωCXω=Fω, | (B. 5) |
| --- | --- |

or

| Xω+iGωXω=HNωFω, | (B. 6) |
| --- | --- |

where

| Gω=ωHNωC | (B. 7) |
| --- | --- |

Therefore, the relationship between the measured FRF Hω and the “normal” FRF HNω is

| HNω=I+iGωHω | (B. 8) |
| --- | --- |

where I is an identity matrix. Since HNω and Gω are real matrices, the imaginary part of the RHS in the above equation is zero, giving

| Gω=-im(Hω)Re(Hω)-1 | (B. 9) |
| --- | --- |

Finally, the damping matrix at any given frequency can be expressed by

| C=-1ωHNω-1Gω | (B. 10) |
| --- | --- |

It should be noted that this damping identification method requires prior knowledge of the stiffness and mass matrices.

*Corresponding author: steinchen@hnu.edu.cn