Research article for Wiley

Optimize the synthesis error in an 8-bar Peaucellier-Lipkin mechanism using an objective function maximisation approach and application to load lifting.

BODIE NGUEMIENGO Abdel Axis1 2 , MBA MBA Frédéric1 2, EPEE Alban Fabrice Lionel 1 2,  NGAYIHI ABBE Claude Valery1 2*, KOM Charles Hubert1 2

1 Laboratory of Mechanics and Materials, University of Douala, P.O.BOX: 2107 Douala, Cameroon

- National Higher Polytechnic School of Douala, University of Douala, P.O.BOX: 2107 Douala, Camerooon

* Correspondence should be addressed to ngayihiclaude@yahoo.fr

Abstract

This work deals with optimizing the synthesis error in an 8-bar Peaucellier - Lipkin mechanism, for its dimensional synthesis and applications in load-lifting machines. A new method for the formulation of the problem of maximizing the objective function is proposed and makes it possible to obtain from the PSO algorithm a minimum synthesis error emin= 9.07E-06 mm for the generation of the straight trajectory when the search interval for the lengths of the bars is 1 mm, 15 mm  and a minimum error emin= 1.47E-04 mm when the search interval is  1000 mm, 15000 mm. For 10 simulations in case 1 the average convergence time is tm = 55 s with the largest iteration at 10 (for t = 159 s); for 100 simulations in case 2, the tm = 229 s with the largest iteration at 136 (for t = 2294 s). The minimum error of case 1 is compared with the results of authors in the literature on the generation of the right trajectory because the search space is approximately equal. In the literature, emin= 0.648358 mm with the GA-DE algorithm in 2010, emin = 2.3667E-005 mm with the MKH algorithm in 2016, emin = 0.027145 mm with the SAP-TLBO algorithm in 2017, emin = 3.7E-4 with the GA algorithm in 2019. This new method brings a plus, because even when the search space is very large, the algorithm converges quickly and it allows the study to be extended to the generation of circular trajectories by just modifying the ratio between the frame bar and the crank bar. The results of the post-design FEM analysis show that for a 1.4571 steel (X6CrNiMoTi17- 12- 2) with a thickness of 50 mm and a joint with a radius of 500 mm, the mechanical device obtained can support a load of 1500 kg.

Keywords: Peaucellier - Lipkin Mechanism Synthesis, PSO, Optimisation of the synthesis error, maximisation of the objective function, application to load lifting.

- Introduction

In the dimensional synthesis of bar mechanisms, it is necessary to find the optimum bar lengths and angles to meet a specific problem. For this reason, it is advisable to start with a topological synthesis to determine the configuration of the mechanism to be used in response to the type of movement required and then to define the working space of the mechanism. An optimization algorithm can then be set up to optimize the objective function or the mechanism's synthesis error. [1-7]. In previous works, the authors approach dimensional synthesis largely through the objective function minimization approach, although they vary the optimization algorithms. [8-19].

It should also be noted that for dimensional synthesis, the authors focus much more on 4-bar mechanisms, whether for the generation of rectilinear, circular, or curvilinear motion. [19-33]; on the other hand, for applications using a perfectly rectilinear movement at the output, 8-bar mechanisms with rectilinear development offer better configurations and take up less space. In this field of study, the most commonly used optimization algorithms are the meta-heuristic algorithms, of which there are several classes: evolutionary algorithms such as the genetic algorithm (GA), differential evolution (DE), genetic programming, evolutionary programming, implicit and explicit algorithms such as particle swarm optimization (PSO), the ant colony algorithm; explicit algorithms such as the distribution estimation algorithm, evolution strategies, the direct algorithm or simulated annealing; local search algorithms such as iterative local search, stochastic local search, guided local search, GRASP, variable neighborhood search, etc. [31, 34-40].

Several works use the Peaucellier-Lipkin mechanism for applications using perfectly rectilinear motion, including the design of a walking robot in the shape of a spider. [41], the balance of the shaking force of the straight-line trajectory of the mechanisms [42] ; a scissor mechanism used to move a projector vertically [43] ;  a model of an eight-legged walking machine [44] ; A polycentric passive mechanism to improve active mediolateral balance in prosthetic walking [45] ; A new type of planar mechanism with an offset centre of gravity 2-DOF inspired by the Peaucellier-Lipkin linear linkage [46] ; Description of a propulsion unit used to guide a walking machine by recognising a trajectory bordered by three points [47] ; Non-anthropomorphic exoskeleton with legs based on eight-bar links [48];  Design of a conformal micro-tension based on the Peaucellier-Lipkin straight line mechanism [49] ; Hexapod with legs based on Peaucellier-Lipkin mechanisms: A mathematical structure used in reconfiguration for trajectory planning [50] ; Design and evaluation of a haptic interface based on the Peaucellier-Lipkin [51] ; generating movements using combinations of linear mechanisms [52] ; Dynamics of a new robotic leg based on the Peaucellier-Lipkin mechanism on linear trajectories during the transfer phase [53] ; A statically balanced shape displacement [54] ; Inherent balance of gravity in architecture in motion with new designs [55].

It should be noted that even the most recent work on the dimensional synthesis of mechanisms [56-68], minimizes the objective function, taking as the end-effector position error the average of the errors between its various desired positions and the actual positions (obtained after dimensional synthesis). Whilst minimizing the average error, there is no guarantee that the result will be optimal. Another approach is to minimize the maximum error between the desired and actual positions at each iteration of the optimization algorithm. It should be remembered that a high synthesis error can degrade the mechanism's performance, reducing its accuracy and reliability. The problem of interest therefore concerns the optimization of the synthesis error in bar mechanisms; in other words, how to minimize the maximum error (the greatest difference between the theoretical position and the actual position of the end effector at the output of the mechanism). We propose a new formulation for maximizing the objective function, with well-defined constraints for a Peaucellier mechanism, to obtain a smaller synthesis error than that found in the literature. The PSO algorithm is used to find optimal solutions to the objective function maximization problem while minimizing the synthesis error. Subsequently, this synthesis will be used to design a load-lifting mechanism to obtain a higher performance than the rudimentary methods (see Appendix 1) observed in developing countries by minimizing the lengths of the bars after dimensional synthesis. A Post-Design FEM Analysis of the lifting mechanism will be carried out to simulate the behavior of the mechanical system after its design, taking into account real constraints such as loads, displacements, and materials. In the remainder of the work, the methodology for achieving the objectives is presented, followed by the results and a discussion of the research.

- Materials and methods

2-1 Materials

All the work was carried out on a computer with a 2.4 GHz Core i5 processor, 12 GB RAM, 2 GB GPU, and a 1 TB hard disk. The Matlab software is used to implement and simulate the PSO algorithm to optimise the synthesis error; it is also used, by coupling it to Solidworks, to control the asynchronous motor designed to generate the rotational movement at the mechanism's input. Solidworks software is also used to draw the mechanism obtained in 2D and 3D before optimization and after dimensional synthesis to check that it is functional; the coupling of Solidworks and Motion Solve software enables the mechanical device to be simulated to check that the machine meets the criteria of constraints, forces and speed for lifting loads of 1500 kg.

2-2 Methods

2-2-1 Framework for the study of the Peaucellier-Lipkin mechanism

Figure 1 below shows the three configurations of the Peaucellier-Lipkin mechanism, according to the values of the lengths  and. The result is either a straight line or concave and convex arcs.

Figure 1: PL mechanism in cases 1, 2 and 3; PL: Peaucellier-Lipkin [69].

Case 1 occurs when and that  varies; point F1 traces a straight line (see Figure 1), where the PL mechanism involved corresponds to index 1.

Case 2 occurs when  and that  varies ; point F2 traces a concave arc of a circle, from the perspective of an observer positioned at A (see figure 1), the PL mechanism corresponds to index 2.

Case 3 occurs when and that  varies; point F3 describes a convex arc of a circle, when an observer is positioned at A (see Figure 1), and the PL mechanism corresponds to index 3.

Figure 2 below shows configurations of the PL mechanism in which the movement is rectilinear at the output, with variations in the distance between points A and F.

Case 4 corresponds to where K4 > K1, as shown in Figure 2 (a). In this condition, point F4 draws a straight line, as in case 1. However, the shortest distance between point A and the line drawn by point F4 is shorter than that for case 1, as shown in Figure 2 (b). It is important to note that the shortest distance ∆4

For case 5, where K5 < K1. In this condition, point F5 draws a straight line, as in case 1. However, the shortest distance between point A and the line drawn by point F5 is greater than in case 1 (see Figure 2 (c)). The lines drawn by F1 and F5 are separated by ∆5 [69].

Figure 2 : PL in cases 1, 4, and 5, where subscripts 1, 4, and 5 help to identify them. PL: Peaucellier–Lipkin [69]

2-2-2 Geometric model of the mechanism

Figure 3 below shows the geometry of the PL mechanism. The numbers 1, 2, 3, 4, 5, 6, 7, and 8 represent the numbers of the bars; the letters A, B, C, D, E, and F represent the centers of rotation of the links between the bars; and the letters A, B, C, D, E, and F represent the centers of rotation of the links between the bars.  The different angles. The circular movement of the input bar BC results in the rectilinear movement of the point F

Figure 3: Geometry of the Peaucellier-Lipkin mechanism

In this study, given that we are dealing with dimensional synthesis for the generation of the straight trajectory of the mechanism, the following relationships are verified:

(1)

In the AXY reference frame shown in Figure 3, the following relationships are verified using the closed-loop method for closed kinematic chains:

(2)

(3)

According to (2) and (3)

(4)

(5)

(6)

(7)

According to (3), (4), (5), (6)

(8)

According to (7)

(9)

The following expressions are used

(10)

The following expressions are verified

(11)

(12)

(13)

(14)

(15)

According to the geometric model (inversion principle) in the mechanism                                                                                          (16)                                                                                                    (17)

The maximum stroke is given by

(18)

From the above equations, we derive the following equations describing the trajectory of the end effector F as a function of the variation in the angle of entry q2of bar 2 and the length of the bars

(19)

2-2-3 Mechanism before optimisation (case 1)

It is important to present the mechanism before optimisation, with the precision points to be reached to generate the dimensions of the bars of the desired mechanism. Figure 4 below shows the geometry of a PL mechanism with its dimensions. The dotted line shows the precision points, which pass through the straight line.

Figure 4: Mechanism before optimisation for  a=8 mm ; b = 26 mm; c = 12mm

In this study, 19 precision points are used for dimensional synthesis. After optimisation, the distance between the straight line passing through the point at the exit of the mechanism (solid line) and the straight line (dashed line) through which the precision points pass (illustrated by 19 dotted lines) should be close to 0, as shown in Figure 4.

2-2-4 Particle swarm optimisation overview

Particle Swarm Optimisation (PSO) is an optimisation method based on stochastic search algorithms. It is inspired by the concept of self-organisation, which states that a group of relatively unintelligent individuals can have a complex overall organisation. In a PSO, there are np agents called particles. The whole population is called a swarm. Each particle moves through the search space and memorises its best position, i.e. the position with the smallest value of the objective function that respects the constraints. Each particle communicates this best position to the others and they adjust their positions and speeds according to the best global position. In this way, the particles will all tend to move towards the optimal solution to the problem. Consider an optimisation problem with minimisation of an objective function f (x). In general, the objective is to find the position x∗ such that the function f (x∗) is minimal. A PSO is performed with np particles xi, i = 1, . . . ., np. The best position of a particle at a discrete time k is defined by

(20)

The best overall position is the minimum personal minimum, i.e.

(21)

At each stage of the algorithm, the new positions of the particles are calculated by

(22)

And  vit is the velocity of particle i defined by

(23)

(24)

In this equation, c1 and c2 are the constant and positive acceleration coefficients. We recommend choosing :

(25)

and r2jt are two random numbers drawn from a uniform distribution xi0 ≈ μ 0,1. In other words, the particles interact in such a way that the distance between them is identical.

It is important to remember that this PSO topology can be adaptive depending on the problem to be solved.

2-2-5 Definitions of parameters, constraints and the problem of maximising the objective function

Or    the vector of bar lengths and the angle 2 of bar 2, required for a specific application. Each variation in this angle corresponds to a precision point.

According to (1), it can be reduced to this form

(26)

If x and y are respectively the abscissa and ordinate of the end-effector at the output of the mechanism, then for all u  U, with U the vector space of u

(27)

Let P1,P2, P3, ..., Pn be the precision points through which the mechanism's end effector must pass when the angle of the input bar varies.

is considered here as the coordinates of point P in the reference frame attached to the mechanism. The objective of the problem is to find the vector u* U such that the effector of the mechanism passes through the points pi, as i varies ,.

The positioning error of the end effector is defined in relation to a desired point by

(28)

Or er maxdu = max erdu the maximum value of the positioning error of the end effector relative to the precision points  varies from i to n.

.u* being the optimal solution to the problem, will have a maximum error of

We therefore need to find u* such that :

(29)

Subject to the following constraints:

- constraint 1 :

- constraint 2 :  b >4a2+ c2

- constraint 3 :  ; with cymax the race corresponding to max

- constraint 4 :

Let h be a function defined by    by

(30)

Posing , with ( this term ensures that the denominator is not zero in the event that the error takes the value 0 during optimisation).

The above problem can be transformed into the following maximization problem:

Find u* U such that g(u*) = max g(u) ; u  U

(31)

The PSO is therefore used to solve the problem using the algorithm described below.

2-2-6 Synthesis error optimisation algorithm

- Form a vector of particles X and randomly generate Np particles according to (26) ;

- Assign each of the generated particles as the best local position of that particle;

- Calculate the initial performance of each particle using the objective function.;

- Find the global best particle and its corresponding position.;

- Verify convergence criteria:

- If the solution has converged to the desired tolerance, terminate the algorithm.

- Otherwise, continue to step 6.

Calculate the particle velocities using the updated equations (23), (24) ;

- Update the particle positions using equation (22) ;

- Evaluate the fitness of the new particles.;

- Update the personal best positions of the particles using equation (20)

- Update the global best position and its fitness value using equation (21)

- Check if the desired precision has been reached:

- If the precision is reached, stop the algorithm.

- Otherwise, proceed to step 6.

- End.

In order to make the above algorithm explicit, the following code, illustrated in Figure 5, is proposed for optimising the synthesis error of the PL 8-bar mechanism

Figure 5 : PSO algorithm code for the dimensional synthesis of the PL mechanism

2-2-7 Dimensional summary of the mechanism for a load lifting application (case 2)

The values chosen in Figure 5 have been taken so that the error obtained can be compared with the errors obtained by authors whose work deals with dimensional synthesis and the generation of the straight trajectory of quasi-rectilinear mechanisms. [3] , [2], [12] , and mechanisms with perfectly rectilinear development [1]. However, for an application in the field of load lifting, the stroke is important. The lengths of the bars must be of the order of several metres and the search space of the algorithm is approximately 300 to 1000 times larger than what we find in the literature. Based on this application, the performance of the new approach will be assessed, in particular by looking at whether the optimisation algorithm converges quickly. For the application of lifting loads in the construction of buildings (R+5), the end effector must reach a height of 14 m, if its starting position is at ground level. The height of a standard storey is usually 2.6 m, while the height of the slab pour for an R+5 is around 13 m. In this case, Cx = 10m and Cymax = 14 m.

2-2-8 Design of the mechanical lifting device for a 1500 kg load

After the dimensional synthesis, the lengths of the bars obtained will be used to design a mechanical device, linking two Peaucellier - Lipkin mechanisms in parallel and fixed to a semi-trailer road tractor. The characteristics of the material chosen and the dimensions of all the components of the device, including the motor, must enable it to lift a load of almost 1500 kg. In this case it is a stainless steel 1.4571 (X6CrNiMoTi17- 12- 2).

Table 1 shows the dimensions such as the radius of the joints, the thickness of the chosen material and the width of the bars. As well as the maximum travel that the load must make at the mechanism exit and the abscissa of point F (considered at the end effector).

Table 1.

Data on bar dimensions

| Radius of the joints | Material thickness | Bar width | Max. stroke | xF |
| --- | --- | --- | --- | --- |
| 500 mm | 50 mm | 700 mm | 14000 mm | 10000 mm |

Figures 6 and 7 show, respectively, configurations of the PL mechanism (2D model) for given bar lengths, when θ2 = 350 and θ2 = -750 ; and figures 8 and 9 respectively show the values of θ2  for which we obtain the two limit positions (θ2 = 81.250 and  θ2 = -81.250) beyond which the mechanism presents singularities

Trajectory of point F at the exit of the PL mechanism represented with its dimensions after dimensional synthesis

Legend:

Figure 6 : mechanism at a high position above the abscissa axis with  θ = 350

Trajectory of point F at the exit of the PL mechanism represented with its dimensions after dimensional synthesis

Legend:

Figure 7 : mechanises to a low position below the x-axis with  θ = - 750

Trajectory of point F at the exit of the PL mechanism represented with its dimensions after dimensional synthesis

Legend:

Figure 8 : High limit position of the mechanism θ = 81.250

Legend:

Trajectory of point F at the exit of the PL mechanism represented with its dimensions after dimensional synthesis

Figure 9: Bottom limit position of the mechanism θ = - 81.250

After extracting the 2D model of the PL mechanism (Figures 6-9), and using the other dimensions of the mechanism mentioned in Table 1, the 3D model of the chosen mechanical lifting device was made. Figures10 and 11 show this 3D model, respectively, when the load is in high and low positions.

Figure 10 : Mechanism in a raised position, end effector above bar 1 of the frame

Figure 11 : Mechanism in a low position, end effector below bar 1

2-2-9 Post-design FEM analysis of the lifting mechanism

Post-design FEM analysis, or Finite Element Method post-design, is a powerful numerical analysis tool used in many areas of engineering, including solid and fluid mechanics, biomechanics, and structural analysis. It can be used to simulate the behavior of a system or object after it has been designed, taking account of real constraints such as loads, displacements, and materials. The Stages of a Post-Design FEM Analysis

- Model geometry:

The geometry of the 3D model is created from CAD design data. In this study, this model was made on Solidworks, and Figures 12 and 13 show respectively two 3D configurations of the PL mechanism when it is at its upper and lower limit positions. The following steps in the FEM Post-Design analysis were carried out on Motion Solve, and it is therefore recommended that the mechanism be positioned on Solidworks at a high or low limit position.

Figure 12: Mechanism with end effector in high limit position

Figure 13: Mechanism with the end effector at its lower limit position

- Mesh

The geometry of the mechanism and load is divided into a set of smaller finite elements (tetrahedra, hexahedra, etc.). Figure 14 shows an example of the mesh on the box that is to contain the load and Table 2 shows the mesh characteristics and their values.

Tableau 2.

Data for meshing the mechanism elements

| characteristics | values |
| --- | --- |
| Maximum element size | 50,88 mm |
| Minimum element size | 50,77 mm |
| Minimum number of elements in a circle | 8 |
| Element growth rate | 1,4 |
| Gauss points (Jacobian) | 16 points |

Figure 14 : mesh of the load box to be lifted

- Definition of materials

The properties of the materials used are defined (Young's modulus, Poisson's ratio, etc.) as well as the mass of the bars and joints. Table 3 shows the characteristics of the steel selected on solidworks. From the lengths of the bars obtained after dimensional synthesis and the steel chosen (see Table 3), the masses of each bar and of the joints are generated. The values obtained are given in Table 4

Tableau 3.

Material characteristics for the mechanism

Table 4 below shows the masses of the mechanism components and the lengths of the bars.

Tableau 4.

Element weights and bar lengths

| Component names | Number of elements | Mass of unit (kg) | Total weight (kg) | Bar lengths (mm) |
| --- | --- | --- | --- | --- |
| Motor shaft | 1 | 167 | 167 |  |
| Bar 2 | 2 | 96 | 192 | 3322 |
| Bar 3 | 2 | 255 | 510 | 9109 |
| Bar 4 | 2 | 255 | 510 | 9109 |
| Bar 5 | 2 | 117 | 234 | 4067 |
| Bar 6 | 2 | 117 | 234 | 4067 |
| Bar 7 | 2 | 117 | 234 | 4067 |
| Bar 8 | 2 | 117 | 234 | 4067 |
| Load | 1 | 1490 | 1490 |  |
| Load axle | 1 | 279 | 279 |  |

- Application of boundary conditions:

In the Motion Solve software, the boundary conditions (embeddings, supports, applied loads, etc.) are defined at the level of the joints, the frame and the box that is to carry the load.

- Solving the problem:

The FEM software solves the differential equations governing the model's behaviour. In this study, after drawing the 3D model of the lifting device on Solidworks, it is imported into Motion solve for numerical analysis. Figure 15 shows the complete mechanism (made up of two PL mechanisms) as well as the identification of the various elements it comprises in the Motion solve environment. Since we are dealing with two PL mechanisms mounted in parallel, the same elements are found in the second mechanism.

Bar 6

Motor shaft

Bar 3

Bar 4

Bar 2

Load axle

Load

Bar 5

Bar 8

Bar 7

Figure 15: Mechanism imported into the Motion Solve environment

- Post-processing

The results of the analysis are displayed and interpreted (displacements, stresses, strains, forces, torques, moments of the forces, etc.) see Figures 21 - 32.

2-2-10 Motor selection

Based on the post-design FEM analysis of the lifting mechanism, a choice of motor can be made. Figure 16 shows a 10,000 kW DC motor chosen for its ease of use with controllers and for its characteristics such as torque and sufficient rotation speed to set the entire mechanical device in motion. Table 5 presents data on the motor and lifting speed chosen for heavy load lifting applications. Table 6 shows the values of the motor's electrical and mechanical torques, which will be used in Matlab to control the motor.

Figure 16: A 10000 KW DC motor

Tableau 5.

Data on motor and lifting speed

| Engine power | Motor torque | Speed of rotation | Lifting speed |
| --- | --- | --- | --- |
| 10000 kw | 105 - 54610 N.m | 200 - 1750 rpm | 0,1m/s |

Tableau 6.

Values of the electrical and mechanical torques of the chosen motor

| electrical and mechanical torque | Values | Units |
| --- | --- | --- |
| Armature resistance | 0.0284 | Ohm (Ω) |
| Armature inductance | 0.35 | mH |
| Back-frequency constant | 3370/770 | V/rpm |
| Rotor inertia | 6.4 | Kg.m2 |
| Rotor damping | 0.03 | N.m/rad.s-1 |
| Initial rotor speed | 0 | rpm |

3- Results

3-1 Validation of the geometric model and dimensional synthesis Case 1

To check whether equations (42) and (43) generate a straight line, arbitrary values are assigned to the lengths of the bars and to the variation in the entry angle of  bar 2; b = 26 mm; c = 12 mm.

θ = -π3, π3  ,   xF = 33,3 mm, Cymax = 38,1 mm.

Figure 17 shows the straight line obtained for these different values. Although it is a straight line, it can be seen that the abscissa (x = 34.2 mm) and ordinate (y = 33.4911 mm) are different from the desired xF = 33,3 mm and Cymax = 38,1 mm.

In this first case, with 19 points of precision, the bar lengths will have to be optimised to obtain

XF = 10 mm and Cymax = 14 mm.

y-axis

x-axis

Figure 17: Line generated by equation (19); b = 26 mm; c = 12 mm  θ = -π3, π3

The precision points have coordinates

P1 (10, 0); P2 (10, 0.7777); P3 (10, 1.5554); P4 (10, 2.3331); P5 (10, 3.1108) ;

P6 (10, 3.8885); P7 (10, 4.6662); P8 (10, 5.4439); P9 (10, 6.2216); P10 (10, 7)

Given that the mechanism is symmetrical with respect to the axis passing through the frame and that the straight trajectory of the end-effector is perpendicular to this axis, the other precision points have the following coordinates

P11 (10, - 0.7777); P12 (10, - 1.5554); P13 (10, - 2.3331); P14 (10, - 3.1108) ;

P15 (10, - 3.8885); P16 (10, - 4.6662); P17 (10, - 5.4439); P18 (10, - 6.2216); P19 (10, - 7)

The entry angle of bar 2 varies and takes on the following values above the x-axis up to the level of the axis passing through bar 1 of the frame

θ2k=  0,  π24,  π12,  π8,  7π48,  π6,  5π24,  π4,  7π24,  π3

Similarly, below the x-axis, the angle takes on the following values:

θ2k=  - π24,  - π12,  - π8,  - 7π48,  - π6,  - 5π24,  - π4,  - 7π24,  - π3

Table 7 below gives the values of the lengths of the bars obtained after dimensional synthesis, the values of Cx, Cymax ; as well as the values of the Gbest and the synthesis error Err which is the inverse of the Gbest.

Table 7.

Values of bar lengths and data obtained after optimisation

| a (mm) | b (mm) | C (mm) | Cx (mm) | Cymax (mm) | Gbest (mm-1) | Err (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| 4.8631 | 11.5388 | 5.9902 | 10 | 14 | 3.4532*104 | 2.8959*10-5 |

Table 8 shows the values of the PSO algorithm parameters used to optimise the synthesis error of the PL mechanism.

Table 8.

Algorithm data during simulation

| NP | C1 | C2 | epsi |
| --- | --- | --- | --- |
| 50000 | 0.1 | 0.1 | 0.01 |

For case 1; 10 simulations were carried out, as shown in Table 9 below. For each simulation, the values of the simulation time, the Gbest, the synthesis error and the bar lengths are presented in order to confirm that the algorithm converges rapidly towards the optimum solution and to select the vector of solutions with the smallest sum of bar lengths; to reduce the cost of purchasing equipment

Table 9.

Some values of time, Gbest, iterations, vector of solutions and error after simulation

| Algorithm time (s) | Gbest (mm-1) | Iteration | Solution vector | Synthesis error (mm) |
| --- | --- | --- | --- | --- |
| 159,026 | 1,65E+04 | 10 | [3.1811, 9.4092, 4.9912] | 6,06E-05 |
| 20,532 | 3,45E+04 | 1 | [4.8631, 11.5388, 5.9902] | 2,90E-05 |
| 20,923 | 1,36E+04 | 1 | [3.8742, 10.3902, 5.5200] | 7,34E-05 |
| 36,485 | 3,98E+04 | 1 | [2.2183, 14.8179, 13.2362] | 2,51E-05 |
| 21,054 | 3,32E+04 | 1 | [3.2150, 12.1594, 9.1406] | 3,01E-05 |
| 80,937 | 5,55E+04 | 4 | [2.7169, 11.7063, 9.0938] | 1,80E-05 |
| 21,305 | 9,70E+04 | 1 | [1.1229, 7.3724, 5.6476] | 1,03E-05 |
| 20,9 | 1,96E+04 | 1 | [0.3342, 11.1086, 10.8037] | 5,11E-05 |
| 109,58 | 1,10E+05 | 3 | [3.8046, 14.4969, 11.5763] | 9,07E-06 |

Figure 18 shows the variation in Gbest as a function of simulation time and the number of iterations.

Figure 18: Representation of the variation in Gbest, error and error as a function of the number of iterations

According to Table 9, the vector [3.8742, 10.3902, 5.5200] (orange line) is the optimum solution in terms of bar lengths with an err = 7.34E-05 mm, but in terms of error optimisation the smallest value is that of the purple line err = 9.07E-06 mm.

Mechanism before dimensional synthesis

Mechanism obtained after dimensional synthesis

Legend:

Figure 19 shows the drawing of the mechanism after optimisation, in blue with a strong line. The segment on the right, in black with a strong line, is the one through which the precision points pass (here the points P9 and P19, which are the ends, are shown) and the mechanism before optimisation, in blue with a broken line.

Figure 19: 2D drawing of the optimal solution after dimensional synthesis

3-2 Résultat sur synthèse dimensionnelle Cas 2 (application au levage des charges)

P1 (10, 0) ; P2 (10, 777.7) ; P3 (10, 1555.4) ; P4 (10, 2333.1) ; P5 (10, 3110.8) ; P6 (10, 3888.5) ;

P7 (10, 4666.2) ; P8 (10, 5443.9) ; P9 (10, 6221.6) ; P10 (10, 7000) ; P11 (10, -777.7) ;

P12 (10, - 1555.4) ; P13 (10, - 2333.1) ; P14 (10, - 3110.8) ; P15 (10, - 3888.5) ; P16 (10, - 4666.2) ; P17 (10, - 5443.9) ; P18 (10, - 6221.6) ; P19 (10, 7000)

θ2k=  0,  π24,  π12,  π8,  7π48,  π6,  5π24,  π4,  7π24,  π3

Similarly, below the x-axis, the angle takes on the following values:

θ2k=  - π24,  - π12,  - π8,  - 7π48,  - π6,  - 5π24,  - π4,  - 7π24,  - π3

Given the application in load lifting, 100 simulations were carried out to compare the error values and see whether the algorithm converges quickly, and to choose the optimal solution vector. The results show that the vector [3321.90 mm, 9108.80 mm, 4066.60 mm] is optimal with a synthesis error of

e min = 1.4663E-4 mm and a convergence time t = 374 s. Figure 20 shows the variation in the convergence time of the algorithm and the corresponding Gbest values.  The minimum error is optimal for the largest Gbest value.

Times (s)

Figure 20: variations in the values of convergence time and Gbest

From the bar lengths obtained after dimensional synthesis, 2D (see Figures 6 - 9) and 3D (see Figures 10 - 13) models were made. The results of the post-design FEM analysis of the lifting mechanism are shown in Figures 21 - 32. Figure 21 shows the variations in applied stresses for bar 2 (considered as a flexible body during the study). It can be seen that the highest value is Max = 8.041E+01 N/m2.

Figure 22 shows the variations in the forces exerted on the axes (rotation joints) between the bars. The highest value is FMax = 6.468E+05 N.

Figure 23 shows the curves for variations in the force applied (in blue) and the coupling required (in red) to turn the motor shaft and transmit the rotational movement to bar 2 (crank).

Figure 24 shows the variation in the position of the load as a function of time, along the three axes of the reference frame (O, X, Y, Z). It can be seen that it moves mainly along (O, X) with a straight trajectory; movement along (O, Y) and (O, Z) is negligible.

Figures 25 - 29 show respectively the variations in the forces applied and the moments of the forces on bars 2 (see figure 25); bars 5 and 6 (see figure 26); bars 7 and 8 (see figure 27); bars 3 and 4 (see figure 28); the axis of the load and the load (see figure 29).

Figure 21: Variation in stresses applied to bars (considered flexible)

Figure 22 : Variation in the forces exerted on the axes (joints) between the bars

Figure 23: Curves showing the force generated by the motor and the torque

Figure 24 :  Variation in load over time

Figure 25 : Variation of the force and moment exerted on the bar 2

Figure 26: Variation of the force and moment exerted on bars 5 and 6

Figure 27 : Variation of the force and moment exerted on bars 7 and 8

Figure 28 : Variation of the force and moment exerted on bars 3 and 4

Figure 29: Variation in the force and moment of the force exerted on the load and the axle

Figures 30-32 show, respectively, the variations in displacement undergone by the box containing the load with a force of 15,000 N (see figure 30), the mechanical stresses applied to the box (see figure 31) and the deformation of the box (see figure 32).

Figure 30: result of displacement with a force of 15000 N applied in the box

Figure 31 : Result of the constraints obtained

Figure 32 : Result of the deformation of the casing

4- Analysis and discussion

Contrary to popular belief, the Peaucellier - Lipkin mechanism does not only allow a straight motion to be obtained at the output when the bar 2 at the input performs a circular motion; Figure 1 shows that it is possible to obtain a convex or concave circular motion at the output of the mechanism depending on the ratio between the fixed bar 1 and the bar 2. When L1 ˃ L2, and L1 L2, it is impossible to obtain a straight trajectory at the exit whatever the optimisation made. It is therefore necessary to situate the study and to specify that L1 = L2, which is one of the main conditions for obtaining a straight trajectory.

The kinematic modelling of the mechanism provides the equations that give the relationship between the coordinates of the end-effector at the output of the mechanism, the lengths of the bars and the angle of bar 2. Equations (1) - (19) give the relationships between the various angles and the lengths of the bars in the geometrical configuration (Figure 3) and allow us to obtain the equations describing the movements of each linkage centre (A, B, C, D, E) and consequently of all the moving bars in the mechanism.

Once the geometric model has been determined, it is necessary to check whether it can actually be used to obtain a straight line. To do this, arbitrary values for the lengths of the bars are chosen and the angle q must vary within a given range. a=8 mm ; b = 26 mm; c = 12 mm  θ = -π3, π3  . Figure 17 in Matlab and Figure 4 in Solidworks show that the curve described by the point at the output is a straight line.

Therefore, for the optimisation or dimensional synthesis problem for specific applications, an optimisation algorithm is used to obtain the optimum bar lengths and angles, while minimising the deviation between the precision points of the desired line and the points of the line produced by equation 19. The PSO algorithm is used, with precise parameter settings allowing rapid convergence. After several configurations of the PSO parameters and simulations, the values that give the best convergence for this study are c1 = 0.1; c2 = 0.1 and epsi = 0.01.

The proposed new formulation of the objective function  Fobj= h(u))emaxd u+ τ

This results in rapid convergence, with the average time over 100 simulations being

tm = 229 s; with the largest iteration at it = 136.

The minimum synthesis error emin = 9.07E-06 mm for the dimensional synthesis of case 1 with the bar length search interval 1 mm, 15 mm and for case 2 the error is emin = 3.4889E-4 mm. with the search interval for bar lengths 1000 mm, 15000 mm This algorithm can be used for dimensional synthesis applications in various engineering fields, as it offers high accuracy and converges quickly to the optimum solution.

Case 1 has practically the same research space as that found in previous works. In fact, the errors of authors whose work deals with dimensional synthesis and the generation of the straight trajectory of mechanisms with quasi-rectilinear development (4-bar mechanism) are for [3]   emin = 0.648358 mm with the GA-DE algorithm in 2010, For [2] emin = 0.027145 mm with the SAP-TLBO algorithm in 2017, For [12] emin = 2,3667E-005 mm with the MKH algorithm in 2016, and mechanisms with perfectly rectilinear development (8-bar mechanism), for [1] emin = 3,7E-4 with the GA algorithm in 2019. Each of these authors compared their results with those of previous authors and each time obtained better results.

it is noted that although the particle search space in case 2 is about 1000 times larger than that in case 1, the algorithm converges rapidly with an average time of tm = 229 s with the largest iteration at 136.

This algorithm can be used with great efficiency in mechanisms with symmetries with respect to the abscissa or ordinate axes. Figures 10 - 13 show the assembly of the mechanical device (consisting of two Peaucellier-Lipkin mechanisms powered by the same motor, the characteristics of which are given in tables 5 and 6). The lengths of the bars obtained after dimensional synthesis are supplemented with other dimensions (width of the bars, radii of the joints, etc.) to meet the requirements of the load; 15,000 kg to be lifted at a lifting speed V = 0.1 m/s.

Figures 21 - 32, obtained after post-design FEM analysis of the lifting mechanism, show that the device can be used for heavy load lifting applications. For the meshing carried out on the box with a load of 150,000 N, the maximum Von Mises stress is 1.263 E+07, well below the elastic limit of the steel chosen, 4E+08 N/m2. The maximum displacement is 1.811E-1 mm, which is insignificant, and the deformation is 3.326E-05 at the joints and also on the bars, the motor shaft and the joints. The maximum value of the greatest stress exerted on the bars is 8.24 E+01, which is negligible compared with the yield strength of the steel chosen (4E+08 N/m2 ). This means that the bars can be considered rigid. The values of the forces and moments exerted on the elements of the lifting device can be used to size the motor capable of generating sufficient torque to lift the entire device.

Conclusion

The aim of the study was to optimise the error during the dimensional synthesis of an 8-bar Peaucellier-Lipkin mechanism. To this end, the minimisation problem commonly found in the literature was transformed into an objective function maximisation problem. The kinematic model verified to obtain a straight trajectory from the end effector to the mechanism output was used to formulate this function and, using the PSO optimisation algorithm, simulations were carried out to find the smallest value of the synthesis error, which corresponds to the inverse of the largest value of the Gbest. Two cases were developed: in case 1, the particle workspace (vector of bar lengths) is reduced to the interval 1 mm, 15 mm as in the work of previous authors, and the minimum error is emin = 9.07E-06 mm; with the optimum vector of bar lengths [3.8742 mm, 10.3902 mm, 5.5200 mm]. In case 2, for an application in load-lifting machines, the working space is large, resulting in a larger error

emin = 3.4889E-4 mm, but insignificant in view of the large dimensions of the bars. All in all, the algorithm converges rapidly towards the optimum solution, in fact for an industrial application the convergence time is around 374s.

Future investigation

1- When the motor is mounted on the mechanism to make it functional and allow it to do its work, the mechanism vibrates, and this vibration can affect the precision of the rectilinear movement of the load at the mechanism's output; similarly, there is friction between the joints and the bars, which enters dissipative energy; it should also be noted that if the motor's rotation speed is not controlled, this will lead to singularities in the mechanism. For this reason we can use a PID controller to regulate the speed of rotation, the rectilinear movement of the load and reduce the effects of the mechanical phenomena mentioned above.

2- It is possible to use the dynamic model of the load-lifting device by integrating the phenomena of vibration, dissipation and friction; then to design a sliding-mode controller to regulate the speed of rotation and the rectilinear movement of the load. The errors of the two controllers will then be compared to see which is the most appropriate for this application.

3- An experimental model of the load lifting device can be developed, on a reduced scale, to compare the results with the digital model.

Credit authorship contribution statement

BODIE NGUEMIENGO Abdel Axis : Writing – review & editing, Writing – original draft, Validation, Methodology, Conceptualization. MBA MBA Frédéric : Writing – review & editing - Validation, Methodology, Conceptualization , EPEE Alban Fabrice Lionel : Writing – review & editing , NGAYIHI ABBE Claude Valery : Writing – review & editing - Validation, Methodology, Conceptualization  , KOM Charles Hubert : Writing – review & editing – Conceptualization.

Funding

There is no fund available for this paper

Declaration of competing interest

The authors declare that they have no known competing financial interests or personal relationships that could have appeared to influence the work reported in this paper.

Data availability

The data that support the findings of this study are available on request from the corresponding author.

Appendix

Appendix 1: Rudimentary method for pouring building slabs in developing countries

References

[1]	S. G. Desai, A. R. Annigeri, and A. TimmanaGouda, "Analysis of a new single degree-of-freedom eight link leg mechanism for walking machine," Mechanism and Machine Theory, vol. 140, pp. 747-764, 2019.

[2]	S. Sleesongsom and S. Bureerat, "Four-bar linkage path generation through self-adaptive population size teaching-learning based optimization," Knowledge-Based Systems, vol. 135, pp. 180-191, 2017.

[3]	W.-Y. Lin, "A GA–DE hybrid evolutionary algorithm for path synthesis of four-bar linkage," Mechanism and Machine Theory, vol. 45, no. 8, pp. 1096-1107, 2010.

[4]	H.-S. Yan and G.-J. Yan, "Integrated control and mechanism design for the variable input-speed servo four-bar linkages," Mechatronics, vol. 19, no. 2, pp. 274-285, 2009.

[5]	G. Panina and D. Siersma, "Motion planning and control of a planar polygonal linkage," Journal of Symbolic Computation, vol. 88, pp. 5-20, 2018.

[6]	R. Singh, H. Chaudhary, and A. K. Singh, "A new hybrid teaching–learning particle swarm optimization algorithm for synthesis of linkages to generate path," Sādhanā, vol. 42, no. 11, pp. 1851-1870, 2017.

[7]	E. E. Cepolina et al., "Scalable six bar linkage mechanism for re-orienting and aligning objects: Design methodology," Procedia CIRP, vol. 97, pp. 66-70, 2021.

[8]	W.-T. Lee and K. Russell, "Developments in quantitative dimensional synthesis (1970–present): four-bar path and function generation," Inverse Problems in Science and Engineering, vol. 26, no. 9, pp. 1280-1304, 2017.

[9]	A. M. Abdul Qaiyum, Hasan Zakir Jafri, "Dimensional synthesis and optimization of planar linkages for rigid body guidance: A review," Proceedings of the international conference on innovative product design et development, pp. 133-139, 2017.

[10]	B. N. Abdel Axis, N. A. Claude Valery, and K. O. M. Charles Hubert, "Dimensional Synthesis of Four-Bar Mechanisms for the Generation of Rectilinear Motion Through Analytical and Graphical Programming and Optimization of the Straight Trajectory of the Coupling Point," International Journal of Engineering Trends and Technology, vol. 72, no. 2, pp. 254-266, 2024.

[11]	A. M. Sebastian Linß, "MODEL-BASED DESIGN OF FLEXURE HINGES FOR RECTILINEAR GUIDING WITH COMPLIANT MECHANISMS IN PRECISION SYSTEMS," Mechanismentechnik in Ilmenau, Budapest und Nis, pp. 13-24, 2012.

[12]	R. R. Bulatović, G. Miodragović, and M. S. Bošković, "Modified Krill Herd (MKH) algorithm and its application in dimensional synthesis of a four-bar linkage," Mechanism and Machine Theory, vol. 95, pp. 1-21, 2016.

[13]	M. A. A.-G. a. M. M. L. Galal A. Hassaan1 "New Approach for the Synthesis of Planar 4-Bar Mechanisms for 2 CouplerPositions Generation," New York Science Journal pp. 86-90, 2012.

[14]	A. M. Khalid Nafees, "OPTIMAL DIMENSIONAL SYNTHESIS OF SIX-BAR STEPHENSON IMECHANISM FOR PATH GENERATION," International Journal of Mechanical Engineering and Technology (IJMET) , November–December 2016, ,, vol. Volume 7, Issue 6, pp. 535–546, 2016.

[15]	R. R. Bulatović and S. R. Đorðević, "Optimal synthesis of a path generator six-bar linkage," Journal of Mechanical Science and Technology, vol. 26, no. 12, pp. 4027-4040, 2013.

[16]	A. Qaiyum* and A. Mohammad, "Optimal Synthesis of six bar Mechanism using Particle Swarm Optimization," International Journal of Recent Technology and Engineering (IJRTE), vol. 8, no. 6, pp. 5287-5292, 2020.

[17]	M. E. Kutuk and L. Dulger, "Optimization with Genetic Algorithm (GA): Planar Mechanism Synthesis," Scientia Iranica, vol. 0, no. 0, pp. 0-0, 2022.

[18]	Y. Lairong, H. Jianyou, M. Cong, H. Juan, and Y. Tong, "Synthesis method based on solution regions for planar four-bar straight-line linkages," Journal of Mechanical Science and Technology, vol. 26, no. 10, pp. 3159-3167, 2012.

[19]	F. C. Jing-Shan Zhao, Zhi-Jing Feng, "Synthesis of Rectilinear Motion Generating Spatial Mechanism With Application to Automotive Suspension," Journal of Mechanical Design, vol. Vol. 130, pp. 1-8, 2008.

[20]	H. Deng, J. Ye, Y. Du, J. Zhang, M. Ma, and X. Zhong, "Bistable broadband hybrid generator for ultralow-frequency rectilinear motion," Nano Energy, vol. 65, 2019.

[21]	J. Rishmany, G. Sabiini, A. Mansour, and R. Imad, "Generating rectilinear motion using permanent magnets," Journal of Engineering and Applied Science, vol. 70, no. 1, 2023.

[22]	L. Li et al., "Generation of Efficient Rectilinear Gait Based on Dynamic Morphological Computation and Its Theoretical Analysis," IEEE Robotics and Automation Letters, vol. 6, no. 2, pp. 841-848, 2021.

[23]	S. Macfarlane and E. A. Croft, "Jerk-bounded manipulator trajectory planning: design for real-time applications," IEEE Transactions on Robotics and Automation, vol. 19, no. 1, pp. 42-52, 2003.

[24]	R. S. Sodhi and K. Russell, "Kinematic Synthesis of Planar Four-Bar Mechanisms for Multi-phase Motion Generation with Tolerances," Mechanics Based Design of Structures and Machines, vol. 32, no. 2, pp. 215-233, 2004.

[25]	Z. Duan and X. R. Li, "Modeling of target motion constrained on straight line," IEEE Transactions on Aerospace and Electronic Systems, vol. 52, no. 2, pp. 548-562, 2016.

[26]	G. Song Soh and F. Ying, "Motion Generation of Planar Six- and Eight-Bar Slider Mechanisms as Constrained Robotic Systems," Journal of Mechanisms and Robotics, vol. 7, no. 3, 2015.

[27]	J. Sun, L. Chen, and J. Chu, "Motion generation of spherical four-bar mechanism using harmonic characteristic parameters," Mechanism and Machine Theory, vol. 95, pp. 76-92, 2016.

[28]	J. Sun, P. Wang, W. Liu, and J. Chu, "Non-integer-period motion generation of a planar four-bar mechanism using wavelet series," Mechanism and Machine Theory, vol. 121, pp. 28-41, 2018.

[29]	W.-T. Lee, K. Russell, Q. Shen, and R. S. Sodhi, "On adjustable spherical four-bar motion generation for expanded prescribed positions," Mechanism and Machine Theory, vol. 44, no. 1, pp. 247-254, 2009.

[30]	Q. Shen, Y. M. Al-Smadi, K. Russell, and R. S. Sodhi, "On Planar Five-bar Motion Generation with a Driver Torque Constraint," Journal of Advanced Mechanical Design, Systems, and Manufacturing, vol. 2, no. 3, pp. 408-416, 2008.

[31]	J. Han and W. Qian, "On the solution of region-based planar four-bar motion generation," Mechanism and Machine Theory, vol. 44, no. 2, pp. 457-465, 2009.

[32]	G. R. Gogate and S. B. Matekar, "Optimum synthesis of motion generating four-bar mechanisms using alternate error functions," Mechanism and Machine Theory, vol. 54, pp. 41-61, 2012.

[33]	Y. M. Al-Smadi, K. Russell, and R. S. Sodhi, "Planar Four-Bar Motion Generation With Static Structural Conditions," Journal of Mechanisms and Robotics, vol. 1, no. 3, 2009.

[34]	A. Rodriguez-Molina, M. G. Villarreal-Cervantes, E. Mezura-Montes, and M. Aldape-Perez, "Adaptive Controller Tuning Method Based on Online Multiobjective Optimization: A Case Study of the Four-Bar Mechanism," IEEE Trans Cybern, vol. 51, no. 3, pp. 1272-1285, Mar 2021.

[35]	J. K. Grabski, M. Sopa, and A. Mrozek, "Application of the path-repairing technique and virus optimization algorithm for the dimensional synthesis of four-bar mechanisms," Archives of Civil and Mechanical Engineering, vol. 23, no. 2, 2023.

[36]	Y.-H. Kang, J.-W. Lin, and W.-C. You, "Comparative Study on the Synthesis of Path-Generating Four-Bar Linkages Using Metaheuristic Optimization Algorithms," Applied Sciences, vol. 12, no. 15, 2022.

[37]	W.-Y. Lin and K.-M. Hsiao, "Cuckoo search and teaching–learning-based optimization algorithms for optimum synthesis of path-generating four-bar mechanisms," Journal of the Chinese Institute of Engineers, vol. 40, no. 1, pp. 66-74, 2017.

[38]	A. S. J.A. Cabrera , M. Prado, "Optimal synthesis of mechanisms with genetic algorithms," Mechanism and Machine Theory   vol. 37, pp. 1165–1177, 2002.

[39]	E. Mezura-Montes, E. A. Portilla-Flores, and B. Hernández-Ocaña, "Optimum synthesis of a four-bar mechanism using the modified bacterial foraging algorithm," International Journal of Systems Science, vol. 45, no. 5, pp. 1080-1100, 2013.

[40]	A. Yildiz, "Parametric synthesis of two different trunk lid mechanisms for sedan vehicles using population-based optimisation algorithms," Mechanism and Machine Theory, vol. 156, 2021.

[41]	D. A. Núñez-Altamirano, "Kinematics and Workspace of a Reconfigurable Robotic Leg Based on the Peaucellier-Lipkin Mechanism," IEEE, pp. 339-403, 2019.

[42]	V. v. d. Wijk, "Shaking Force Balance of the Peaucellier-Lipkin Straight-Line Linkage," Springer Nature Switzerland AG vol. Mechanisms and Machine Science 71, pp. 177-184, 2019.

[43]	G. S. e. G. K. Ananthasuresh, "Minimization of Preload in Springs used in Static Balancing of Linkages under Constant Loads," Proceedings of the 1st International and 16th National Conference on Machines and Mechanisms (iNaCoMM2013), IIT Roorkee, India, , pp. 425-433, Dec 18-20 2013.

[44]	A. R. A. Shivamanappa G. Desai , A. TimmanaGouda "Analysis of a new single degree-of-freedom eight link leg mechanism for walking machine," Elsevier, vol. Mechanism and Machine Theory 140  pp. 747–764, 2019.

[45]	J. M. H. Evert S. van Hal , Klaas Postema , and Egbert Otten, "A Passive Polycentric Mechanism to Improve Active Mediolateral Balance in Prosthetic Walking," IEEE TRANSACTIONS ON NEURAL SYSTEMS AND REHABILITATION ENGINEERING, vol. VOL. 32, pp. 63-71, 2023.

[46]	J. W. Genliang Chen, Hao Wang, "A New Type of Planar 2-DOF Remote Center-of-Motion Mechanism Inspired by the Peaucellier-Lipkin Straight-line Linkage," Journal of Mechanical Design, pp. 01-13, 2018.

[47]	J.-C. I. NÚÑEZ-ALTAMIRANO Diego A, MÁRQUEZ-PÉREZ Lucia et FLORES-DÍAZ Ociel, "Description of a Propulsion Unit Used in Guiding a Walking Machine by Recognizing a Three-point Bordered Path," CHINESE JOURNAL OF MECHANICAL ENGINEERING, vol. Vol. 29, No. 6, , pp. 1157-1166, 2016.

[48]	I. J. r. C. Jorge Curiel Godoy, Lucia Marquez Perez et Leonardo Romero Mun, "Nonanthropomorphic exoskeleton with legs based on eight-bar linkages," International Journal of Advanced Robotic Systems, pp. 1–16, January-February 2018.

[49]	J. Hricko, "Design of Compliant Micro-Stage Based on Peaucellier–Lipkin Straight-Line Mechanism," 23rd International Conference on Robotics in Alpe-Adria-Danube Region , Smolenice Castle, Slovakia, pp. 01-06, September 3-5, 2014.

[50]	D. A. N.-A. Ignacio Juarez-Campos, Lucia Marquez-Perez, Leonardo Romero-Munoz and Beatriz Juarez-Campos, "Hexapod with legs based on Peaucellier–Lipkin mechanisms: A mathematical structure used in reconfiguration for path planning," International Journal of Advanced Robotic Systems, pp. 01-18, 2018.

[51]	M. K. Sanditi Khandelwal, and Abhishek Gupta, "Design and Evaluation of a Peaucellier-Lipkin Linkage Based Haptic Interface," Technologies, pp. 360-368, 2014.

[52]	C. A. P. Carl A. Nelson, "MOTION GENERATION USING COMBINATIONS OF PEAUCELLIER STRAIGHTLINE MECHANISMS," Proceedings of the ASME,  International Design Engineering Technical Conferences & Computers and Information in Engineering Conference IDETC/CIE

, Chicago, IL, USA, pp. 01-07, August 12-15, 2012.

[53]	I. J.-C. Diego Alfredo Nunez-Altamirano, Lucia Marquez-Perez, Ociel Flores-Dıaz and Leonardo Romero-Munoz, "Dynamics of a novel robotic leg based on the Peaucellier–Lipkin mechanism on linear paths during the transfer phase," Advances in Mechanical Engineering , , vol. Vol. 8(7) pp. 1–10, 2016.

[54]	C. P. L. Joseph E. Pishnery, "A statically balanced shape shifting," Proceedings of the ASME  International Design Engineering Technical Conferences & Computers and Information in Engineering Conference IDETC/CIE, Chicago, IL, USA, pp. 1-7, August 12-15 2012.

[55]	V. v. d. Wijk, "Inherent Gravity Force Balance in Moving Architecture with New Designs," Architecture, Technology and Innovation, pp. 1-9, 2020.

[56]	M. J. D. Hayes and M. Rotzoll, "Dimensional Kinematic Synthesis of Angular Velocity Function Generators," in 12th CCToMM Symposium on Mechanisms, Machines, and Mechatronics, Université Laval, Québec, QC, Canada, 2023.

[57]	I. B. Hamida, "Synthèse typologique et dimensionnelle des mécanismes à chaînes cinématiques fermées: application aux robots parallèles," Université de Poitiers; Ecole Nationale d'Ingénieurs de Sousse (Tunisie), 2021.

[58]	J.-B. Riccoboni, "Modélisation dynamique d'arborescence-application à la robotique et à l'étude du mouvement humain," Université de Poitiers, 2022.

[59]	Z. Copeland, M. Rotzoll, M. J. D. Hayes, S. Nokleby, and P. Cardou, "Concurrent type and dimensional continuous approximate function generator synthesis for all planar four-bar mechanisms," in 11th CCToMM Symposium on Mechanisms, Machines, and Mechatronics, Ontario Tech University, Oshawa, ON, Canada, 2021.

[60]	Z. A. Copeland, M. J. D. J. I. J. o. M. Hayes, and R. Systems, "Multi-modal continuous approximate synthesis of planar four-bar function generators," vol. 5, no. 3, pp. 246-269, 2023.

[61]	Z. A. Copeland and M. J. D. Hayes, "Multi-modal Continuous Approximate Algebraic Input-Output Synthesis of Planar Four-Bar Function Generators," in USCToMM Symposium on Mechanical Systems and Robotics, 2022, pp. 10-19: Springer.

[62]	N. Diab, A. J. M. Smaili, and M. Theory, "Optimum exact/approximate point synthesis of planar mechanisms," vol. 43, no. 12, pp. 1610-1624, 2008.

[63]	E. Söylemez and G. Kiper, "Four-bar function generation using Excel solver," in IFToMM World Congress on Mechanism and Machine Science, 2023, pp. 208-218: Springer.

[64]	Z. A. Copeland, "Continuous Approximate Kinematic Synthesis of Planar, Spherical, and Spatial Four-bar Function Generating Mechanisms," Carleton University, 2024.

[65]	S. Bai, J. J. M. Angeles, and M. Theory, "A unified input–output analysis of four-bar linkages," vol. 43, no. 2, pp. 240-251, 2008.

[66]	T. Xu, D. H. Myszka, and A. P. J. R. Murray, "A Bi-Invariant Approach to Approximate Motion Synthesis of Planar Four-Bar Linkage," vol. 13, no. 1, p. 13, 2024.

[67]	V. van der Wijk, "Kinetic-geometric three-position synthesis of a balanced 4R four-bar linkage," in International Symposium on Advances in Robot Kinematics, 2024, pp. 204-212: Springer.

[68]	M. Ruggiu and P. Rea, "Robotic-like formulation of the approximated body-guidance problem," in International Conference on Reliable Systems Engineering, 2023, pp. 405-417: Springer.