Wakesurfing, Wakeboarding and Waterskiing:

A Comparison of Wake Characteristics

Gregor J. Macfarlane

Australian Maritime College, University of Tasmania, Launceston, Tasmania, AUSTRALIA.

email: gregorm@amc.edu.au

Abstract

Waterskiing has been a commonplace and generally well-accepted activity on inland waterways for many decades. More recently, there has been a significant increase in wakeboarding and wakesurfing, with the latter relying heavily upon ‘enhanced’ boat wake. This has seen an increase in issues such as shoreline erosion and damage to public and private property, often resulting in additional complications for those tasked with the management of sheltered waterways. This is most prevalent in situations where lateral distance is limited, such as rivers and small lakes, where there may be insufficient distance for the larger boat-generated waves to disperse and attenuate. This has become a hot topic, with disputes occurring at many locations – for example, there are known cases in at least 20 US States.

This paper investigates the key differences in characteristics of the waves generated by typical waterski and wake boats, with and without wake enhancing devices. Measurements of the waves generated by a variety of recreational and wake boats were acquired from full-scale field trials. Results are presented graphically and compared with data from other published studies of a similar nature. It is confirmed that there are significant differences in both the height and energy of the maximum wave generated by the three different water sports (and to a lesser extent, wave period). Data is acquired at multiple locations over a relatively large lateral distance from the sailing line of the test boats which should assist regulators to identify management options for waterways with sensitive shorelines and vulnerable property.

Keywords: wakesurf, wakeboard, waterski, waterway management, bank erosion.

# Introduction

1.1	Background

Shoreline or riverbank erosion caused by vessel generated waves is a common problem in sheltered waterways that would otherwise experience minimal wave action. Erosion can of course be linked to other sources, such as wind waves, floods, regulated river flows and livestock access, however there are growing incidences where the waves from recreational boating have exacerbated erosion levels (Nanson et al. 1994; Novak et al. 2021; Fenton et al. 2023; Fleit et al. 2019; Macfarlane and Cox 2003; Maynord et al. 2008; Macfarlane et al. 2008). Towed water sports, such as waterskiing, have been a common and accepted activity for many decades, but there has been an increase in the frequency of boats specifically designed to enhance the wake for activities such as wakeboarding and wakesurfing. This latter activity requires the boat to generate a large wave that can be surfed without the aid of a tow rope.

There are clear and notable differences in several aspects of these three water sports, particularly the speed they operate, but also the characteristics of the boat used and the optimum depth of water beneath the boat, all of which can heavily influence the characteristics of the waves generated. This in turn will influence the lateral distance that may be necessary for the boat-generated waves to disperse and attenuate sufficiently to minimise the likelihood of shoreline erosion or damage to property.

Manufacturers of wake boats have sought to innovate their designs by adopting several methods to create an ‘ideal’ wave for surfing, which will typically have a long and steep wave face (the first smooth section of the wake which is generally the preferred region for surfing) leading to a plunging wave, where the crest curls over the front face and falls to the base of the wave. Commonly adopted ‘wake enhancing devices’ (WED) used to achieve ideal wakesurfing conditions include large internal (and/or inflatable) ballast tanks, hydrofoils and surf gates or plates typically located at or near the transom of the vessel to further increase the height and shape by adjusting the dynamic trim and heel of the vessel. It is standard practice to impose a notable heel angle resulting in an asymmetric wake that is surfable on just one side of the boat where the quality of wave has been maximised for surfing, as seen in the photograph provided in Figure 1.

It is worth noting that issues commonly associated with vessel-generated waves have been used to mask other community concerns such as noise (engines, music) and the loss of amenity (Macfarlane et al. 2008). Communities and governments tend to react strongly to tangible evidence such as shoreline/bank erosion, regardless of the cause, whereas noise and loss of amenity are more subjective, somewhat less tangible, and therefore less likely to attract regulation.

The Water Sport Industry Association (WSIA) in the United States actively promote three core recommendations for wake surfers to “Wake Responsibly” (WSIA, 2023). The first is to stay at least 200 feet (~60 metres) from the shoreline, docks or other structures. The appropriateness of this specific distance is regularly raised and may be considered a focus of the present study. The second rule is to play music at reasonable levels and have respect for the surroundings (ie. time of day, explicit lyrics, etc). The third rule is also quite pertinent: to minimize repetitive passes on any one portion of shoreline.

1.2	Quantifying Boat Waves

It is common to assess vessel wave wake by quantifying the height, period and energy (per unit crest width) of a single wave in the complex wave train, usually the highest (or “maximum”). Wave energy in deep water is proportional to both the square of the wave height and wave period (refer Equation 1), so any change in either height or period will result in a significant change in wave energy.

(1)

Linear wave theory is applicable where the wavelength is less than 3.5 times the water depth: a condition at which a wave's orbital velocity at depth h is only 3% of that at the surface (Lighthill, 2001). The length of the waves generated by recreational craft will typically be limited such that they can operate in fairly shallow water before their wakes become depth affected. Cox (2020) refers to this as 'practically deep'.

There are two further parameters that can identify when the characteristics of the waves generated by moving vessels may be affected by water depth: the depth Froude number (FrL=VgL) and the water depth/vessel waterline length ratio (h/L). For the former, a sub-critical (typically ‘deep’ water) condition occurs when Frh is less than approximately 0.7, which is preferred for wakesurfing. A trans-critical zone occurs between ~0.7 < Frh < 1.0 where the waves may become depth affected. Froude Depth numbers in excess of 1.0 are generally considered super-critical. However, a water depth/vessel length ratio of h/L = 1.77 identifies the point where the waves are simply not long enough to be depth affected, regardless of vessel speed (ie. there is zero super-critical effect in any condition). An effect on the waves becomes measurable around h/L = 1 and the effect can potentially be substantial when h/L is less than 0.5.

Cox and Macfarlane (2019) state whatever energy is contained in a wave system must eventually be expended at the shore. Close to the vessel, the energy is contained in only a few large waves. As lateral distance is increased, dispersion leads to the same energy shared across many more waves. This is why the most commonly adopted technique to mitigate wave wake impacts is to increase lateral distance, the total energy has not reduced, just spread across more waves. What this cannot do is reduce wave period.

When attempting to avoid or minimise erosion, the key task is to identify the threshold below which the waves are not going to alter the state of equilibrium for the specific shoreline/bank type and composition. For example, in regions with highly sensitive shorelines the conduct of experiments to quantify the turbidity (sediment movement) can identify threshold limits of wave height and wave period (Cox, 2020; Macfarlane et al. 2008). Alternatively, the threshold may be determined by quantifying the naturally occurring (prevailing) wind-generated waves, or by identifying vessels that have proven through successful operation over time that the waves they generate are acceptable for the region of concern. This is effectively what has occurred in many ‘hot spots’, for example, by setting the characteristics of the waves generated by adopting waterskiing activities as a benchmark (at a relevant lateral distance) and assessing other activities, such as wakesurfing, by identifying the lateral distance where their waves are considered approximately ‘equivalent’ in terms of erosion potential.

The existence of erosion thresholds suggests that the magnitude of energy is less important than the form in which it is delivered. Cox (2020) provides a useful analogy to describe annualised (or totalised) energy by considering the dropping of a small coin from a height of 50 mm (2 inches) every second for one year, which would be analogous to the effects of wind waves. That annualised energy as a single event would be equivalent to dropping a 1-tonne vehicle from a height of about 4.5 m (~15 feet). One could be regarded as inconsequential and the other catastrophic, yet both have the same energy. Similar applies to the use of total wave energy rather than energy of a single maximum wave, albeit to a much lesser degree.

Given these potential flaws in methods that attempt to summate or annualise wave energy, it is believed that such comparisons of boat wave energy and/or wind wave energy can be meaningless when comparing the impacts of different wave regimes. The author prefers to quantify the energy of the maximum wave as this aligns with the use of benchmarks that can compare the characteristics of discrete waves against erosion thresholds that can be quantified for specific shoreline types. One complication comes if the erosion threshold is exceeded then the height and/or energy of the maximum wave alone may not be an indication of how many waves exceed that threshold.

1.3	Relevant Studies

In an effort to better understand the waves created by wakesurfing activities and their potential to result in bank erosion or other issues, several teams have published technical reports describing full-scale experiments designed to quantify the characteristics of these waves. The following three particular studies are most the commonly cited studies:

- Goudey and Girod (2015):	Commissioned by the Water Sports Industry Association (WSIA)

- Macfarlane (2018): 		Australian Maritime College, University of Tasmania (AMC)

- Marr et al. (2022): 		St. Anthonys Falls Laboratory, University of Minnesota (SAFL)

This paper presents the most salient results from the research performed by AMC in 2018 and compares this against relevant results presented in the WSIA and SAFL reports. The author would have preferred to present all results in metric SI units, however both the WSIA and SAFL reports (and the original 2018 AMC report) present (most of) their results in imperial units, which has been adopted in this paper to allow easier and more direct comparison between all three studies. Wave heights are reported in inches, lateral distance in feet, wave energy in lb.ft/ft and vessel speed in miles per hour (mph).

Several other studies have also attempted to quantify the characteristics of the waves by recreational craft on sheltered waterways in recent years. For example, Houser et al. (2021) quantified boat-generated waves on a lake regularly used for recreational boating, but unlike the AMC, WSIA and SAFL studies, boating activity was incidental (unplanned), so none of the key information such as boat details, speed, lateral distance was recorded, just the resultant wave traces to summate cumulative wave energy. An interesting finding from their analysis suggests that up to 47% of the wave energy following a boat wake is associated with the waves reflecting from adjacent shorelines, thus the cumulative wake energy may be underestimated. A more recent study by Houser et al. (2024) includes a survey of local residents on the perception of boat wake impacts. It is felt that the measurements presented in both studies suffer from a data sampling rate that is too low (1 Hz) to accurately quantify the waves created by the type of boating activities of interest in the present study, which is confirmed in later sections (and acknowledged by the authors).

Fay et al. 2022 describes a numerical simulation of the waves and propeller wash from a wake boat. There are numerous flaws in this study, most notably the Computation Fluid Dynamics model is not calibrated or validated leaving questionable results, and there are several claims made in the abstract and conclusion that are not supported by the material presented.

# Details of Three Experimental Studies

2.1	Effect of Water Depth on Wakesurfing Waves

Before presenting details and comparing results from the three studies, it is helpful to recall that the depth of water beneath a moving vessel can have a significant effect on the characteristics of the waves generated, so vessel dynamics are defined in terms of depth Froude number. This is well reported elsewhere, for example Cox (2020). It is generally known that it is best to wakesurf in ‘deep’ water, where a depth of at least 12 feet (but deeper is better) is suggested by manufacturers of wake boats (Malibu Boats, 2024). At shallower depths, the boat will not generate a fully formed wave. This has been confirmed by a series of physical scale model experiments performed at equivalent full scale water depths of 3, 6, 9, 12 and 18 feet. Time-series data of the wake close to the boat at each water depth are shown in Figure 2, where it is clearly evident that the largest and steepest wave is found at the deepest depth/s, and this degrades with decreasing water depth.

The next three sub-sections provide a background and details for each of the three key studies identified in Section 1.3. The results from these studies are directly compared and discussed in Section 4.

2.2	WSIA (Goudey and Girod, 2015)

The WSIA were the first to recognise the paucity of reliable experimental data quantifying the characteristics of waves created by wake boats operating at wakeboarding speeds. WSIA commissioned Goudey and Girod (2015) to conduct research, primarily to achieve the following primary objectives:

- Develop methods and instrumentation to accurately measure waves generated by the passage of a wake boat.

- Measure the wakes produced by a wake-sport boat at cruising, wakeboarding, and wakesurfing speeds and loading conditions at deep and shallow test sites.

- Determine the wave energy from these wakes and how it varies with mode of operation and distance from the boat track.

- Compare the experimental findings with the wave energy associated with wind waves.

- Use the finding to assist WSIA in developing guidelines for wake boat operation to minimize any negative impacts of wake sports.

Their investigation focused on a single wake boat and two test sites, one considered “deep” where the wake boat operated in water depths around 23 to 30 feet, and the other “shallow” where the boat operated in depths around 10 feet.

The water depth/vessel length ratio for the shallow and deep sites are ~0.44 and between 1.0-1.4 respectively. From Section 1.2 we know that the boat generated waves will likely be affected at all speeds of interest at the shallow water site, especially at the higher vessel speeds. The waves created at wakesurfing speeds at the deep-water site will be largely unaffected by water depth (where Frh ~0.5-0.6), while the situation becomes more complex at the higher wakeboarding speeds where Frh ~1.0-1.2, and up to Frh ~1.5 for cruising speeds. As a result, the vessel wake characteristics from this study will likely be influenced, to varying degrees, by the water depth in all but one case: wakesurfing speeds at the “deep” site. This is not unusual, nor does this make the data invalid, it just needs to be acknowledged that this may lead to additional data scatter due as small variations in speed and/or depth affecting the waves created.

The WSIA study compares the measured wave characteristics of a single wake boat against wind waves predicted for various combinations of wind velocity and fetch. The WSIA report includes some valuable data, however the report itself may be tainted by bias, as a number of the conclusions are not supported by the data presented. It appears that the consulting engineers commissioned by WSIA performed the study, but the WSIA may have made their own interpretation of the results from which they based their recommendations – most notably that wakesurf activities stay at least 200 feet from the shoreline, docks or other structures. The following statement was available on the WSIA website: “In 2015, Ocean Engineer and Naval Architect Clifford Goudey scientifically studied and collected wave energy data on the characteristics of boat wakes. WSIA analyzed the data to develop recommendations to protect shorelines, docks and other personal property.” (WSIA, 2023).

2.3	SAFL (Marr et al. 2022)

Also motivated by a need to better understand the characteristics of wakes and waves produced by recreational boats common on lakes and rivers, the SAFL performed full-scale trials on four different boats, where two were typical recreational boats (i.e., non-wakesurf) that are commonly used for tubing, waterskiing, wakeboarding) and two boats designed specifically for the sport of wakesurfing. For these latter two boats tests were performed with and without WEDs. This allowed a direct comparison of measured wave characteristics of the more traditional non-wakesurf boats against ballasted and un-ballasted wake boats. No attempt was made to compare against wind-generated waves.

2.4	AMC (Macfarlane, 2018)

The origins of the present paper stem from a series of full-scale trials performed by the author in 2018. The need for an independent study comparing the wave characteristics for a wider range of recreational and sport boats was recognised. The study tested boating activities covering waterskiing, wakeboarding, tubing, wakesurfing and fishing. Some results were provided in an unpublished but freely available report (Macfarlane, 2018). The experiments were conducted in collaboration with the Oregon River Safety and Preservation Alliance (ORSPA, 2024) who organised the test vessels and provided valuable assistance setting up equipment. All test data were acquired and analysed solely by the author, who contributed time and equipment pro bono. The brief 2018 unpublished report provided details of the experiments and presented limited results, but deliberately refrained from making any conclusions or guidelines for wake boat operation.

This paper presents the most salient data from the full AMC study including data that is not included in the 2018 report and directly compares this with data published in both the WSIA and SAFL reports.

2.5	Common Traits Between the Three Studies

All three studies adopted test craft which were considered representative of those typically used for either waterskiing, wakeboarding and/or wakesurfing (and other recreational activities such as tubing and fishing). They also provided details of each craft, most importantly the total displacement of each vessel, which at times included significant ballast and other WEDs deployed.

All three studies also covered the two speed zones of most interest; wakesurfing (10 – 12 mph) and wakeboarding/tubing (20 – 24 mph). The AMC study also included a higher speed range commonly used by experienced water-skiers (30 – 32 mph).

As previously noted, of particular interest to this study is the effect that lateral distance has on the characteristics of waves created by various recreational/sport boats. In order to cover a wide range of lateral distances in their respective field trials, all three studies adopted a similar approach whereby one or more wave sensors were located at fixed location/s, while the (straight) track line of the test vessel was set at multiple lateral distances. This provided the desired result of acquiring data at multiple locations, but also introduces additional variables that may lead to further scatter in the experimental results (refer Section 4.5).

Each report presented a small sample of time series data and confirmed the equipment and sample rate/s used when acquiring time-series wave measurements. These are summarised below:

- WSIA – 30 Hz capacitance 2-wire wave probe and pressure sensor

- SAFL – 10 Hz for pressure transducers; 4 Hz for Acoustic Doppler Current Profiler (ADCP)

- AMC – 200 Hz capacitance 2-wire wave probe

Note that the sample rate of 4 Hz for the SAFL ADCPs is considered low when attempting to quantify wind and boat waves with periods in the range of 1.2 to 2.5 s.

Conveniently, all three studies recognise the relevance of both wave height and wave energy in such studies, and all three present the height of the maximum wave from each complete wave train. However, both WSIA and SAFL present the total wave energy in the wave train. For reasons previously noted in Section 1.2, the author believes the energy of the maximum wave to be a better indicator of the potential to cause bank erosion, due to the ability to compare this against appropriate erosion thresholds for different shoreline types. To obtain the energy in the maximum wave, both the height and period must be known – unfortunately both WSIA and SAFL fail to present wave period data. As a solution, in this paper AMC wave period data are used to determine average periods for each of the three speed zones of interest and this is used to calculate indicative values of energy for the maximum waves presented in the WSIA and SAFL reports.

# Methodology

## Full Scale Trials, Test Site and Instrumentation

The full-scale trials for the AMC 2018 study were conducted on a selected section of the Willamette River near Coalca Landing, Oregon City, Oregon, USA as indicated in Figure 3. The site provided a straight reach with a roughly constant ‘deep’ water depth beneath the test vessel in the region of 40 feet. The water depth at the probe was confirmed as 15 feet. Several images related to the test site are shown in Figure 3: the ‘zero’ starting point of the white line shown in the aerial image indicates the approximate location of the wave sensor. The test vessels ran track paths perpendicular to the while line at each of the four nominal lateral distances of 100, 200, 300 and 400 feet from the wave sensor. Buoys were deployed at appropriate locations to guide the boat operator to maintain a consistent distance/track path.

It is important to select a test site where the wave probe will not be subjected to boat-generated waves that reflect off the surrounding shore or any bluff structure as these reflected waves may contaminate the traces and lead to misleading results (as noted by Houser et al. 2021). For example, gently sloping beach-type banks are less reflective than levee-type banks. The site selected had a sufficiently non-reflective shoreline, including considerable vegetation, resulting in minimal reflection.

Water surface elevation was measured using a single MK-VI salt/freshwater capacitance wave probe manufactured by Manly Hydraulics Laboratory. The signal from the wave sensor was digitised and radio telemetered to a custom data acquisition unit which was located approximately 50 to 75 feet distant on a stationary support vessel. Each run was recorded using a Dell laptop computer that was accessed by Labview acquisition software. The wave sensor was calibrated both within the AMC laboratory and checked in situ multiple times. The calibration factors compare well against those obtained within laboratory conditions prior to departing and upon return to AMC.

At the commencement of each test session the wave sensor and data acquisition equipment were set up on the test site. The wave sensor was fixed to a vertical post that was driven into the riverbed and supported by three equispaced ropes that were anchored to the riverbed to minimise any lateral movement of the wave sensor. If the support structure moves laterally during field experiments the resulting wave periods will be contaminated. Similarly, any vertical movement will result in variations in wave height. A photograph of the wave probe set-up is shown in the top-left image in Figure 3. The umbrella was deployed to minimise overheating due to direct sunlight on the yellow case that contains the wave sensor power supply and signal transmitter.

The AMC test matrix (Table 1) provides a reasonable coverage of different and relevant recreational boat types, including a variety of wake enhancing devices and range of operational speeds, but there is a key flaw that needs to be considered. In short, the wakesurfing boats on the Willamette when run at wakesurfing speeds were sometimes driven by individuals with significant waterskiing experience but potentially lacking in the nuances associated with fine-tuning the WEDs to achieve optimum waves for wakesurfing. It was observed that large waves were generated (as expected), but they were occasionally broken or spilling. It is likely that further fine tuning of the asymmetric ballasting and wedges/trim tabs by more experienced practitioner/s would likely have produced a ‘cleaner’ and unbroken wave for wakesurfing. It is suspected that these waves would likely have been higher, which was the case observed during the physical scale model experiments that produced the deep-water data shown in Figure 2.

## Test Program and Procedure

It is highly recommended that a systematic approach be adopted for any experimental campaign involving several variables. The test program undertaken in the present study involved various different recreational craft, multiple load conditions (including ballast options and number of passengers), lateral distances and boat speeds. The marine craft used in this study and their respective load conditions are summarised in Table 1. There are eight AMC cases listed in this table which form the primary part of the test program (Cases 1-5 and 12-14). Each of these eight cases were run at multiple boat speeds representing wakesurfing (10-12 mph), wakeboarding/tubing (22-24 mph) and waterskiing (30-32 mph), giving a total of 19 test conditions. The vast majority of these cases were run at each of the four lateral distances. The other cases shown in Table 1 are the tests performed by WSIA in 2015 (Cases 6-7) and SAFL in 2022 (Cases 8-11). Their published results are compared to the AMC data in Section 4.

At the commencement of each run, the test vessel was accelerated to a nominal steady speed, achieved some distance prior to being perpendicular to the wave sensor (typically at least 200 to 500 feet). Recording of the water surface elevation signal from the wave sensor was triggered manually dependent upon the lateral distance between the sailing line of the boat and the wave sensor (nominal distances were 100, 200, 300 and 400 feet). This provided a baseline measurement of the ambient conditions prior to the arrival of the wake waves at the wave sensor. The water surface elevation continued to be recorded until all significant waves generated by the test boat had passed the wave sensor (this generally lasted for approximately 60 to 120 seconds). The sample rate was set at 200 samples per second (200 Hz) which is more than adequate to clearly define the full form of each wave. Figure 3 (bottom left) shows a photograph taken during a typical run (R70) involving the 2015 Ski Nautique 200 at a speed of 12 mph and lateral distance of 100 feet (Case #13). Yellow marker buoys were used to guide the boat skipper to the desired lateral distance (these buoys were located using a hand-held GPS).

At the end of each run the test vessel paused until the waves generated had dissipated and conditions were considered calm enough for the next test run. The vessel then sailed past the wave sensor in the opposite direction. Approximately 220 individual runs were performed.

Each test run has been individually analysed within an Excel macro worksheet, which imports the data file created during each test run and from the discrete samples collected plots a wave elevation time history. The macro then determines the characteristics of height and period of the maximum wave (and any other selected waves). Other quantities, such as wavelength, celerity, energy and power for the maximum wave can then be readily computed.

# Results and Discussion

4.1	Comparison of Wave Characteristics

As previously noted, the large number of variables involved can lead to a huge amount of data to process, so a logical and considered approach is often necessary to achieve meaningful outcomes when comparing and presenting the characteristics of the waves generated by multiple vessels. This is the primary reason why it is recommended that the present study focusses on the maximum height and energy from each wave trace/run.

Typical wave elevation time series records from four individual runs are plotted in Figure 4. These wave profiles are all for Case #2 (refer Table 1): 2014 Nautique G21 with ballast and nominal forward speed of 10 mph; the four records represent each of the lateral distances, starting with the closest of 100 ft, then 200 ft, 300 ft and 400 ft. The primary purposes of this figure are to provide examples of the time series wave traces and illustrate how dispersion over increasing lateral distance leads to a reduction in wave height and an increase in the number of waves. Note that the start/arrival times for each of the four records, relative to each other, are not displayed correctly in this figure (separated for clarity).

The Excel macro used to analyse each run determines the start of each successive wave by the change in wave elevation above the still water level from positive to negative (or vice versa) – this is the definition of a zero-crossing point. The maximum wave height is defined as being the single greatest distance from a trough to a successive crest (or crest to trough) recorded anywhere within the sample. The period of the maximum wave is obtained from the time between consecutive zero up-crossings (or down-crossings).

For two of the test wake boats, 2006 Malibu V-Ride (Case #1) and 2014 Nautique G21 (Case #2), additional runs were performed to provide a better-defined curve to investigate the effect that speed has on the height, period and energy of the maximum wave. The wave height and period for both boats as a function of vessel speed are presented in Figure 5 (note that these data are for the constant lateral distance of 100 ft). As expected, there is a very rapid increase in height of the maximum wave between 8 and 12 mph. After reaching a peak, the wave height generally reduces gradually with increasing speed, except for a bit of a ‘hump’, around 20-24 mph, which may be due to depth critical speed effects or due to the dynamics of planing craft with extremely low slenderness ratios as they transition to a semi-planing condition. More work in this area is needed.

Wave period reaches a peak around 12-14 mph, and similar to wave height, also generally reduces gradually with increasing speed. The resultant energy of the maximum wave, calculated using the wave heights and periods in Figure 5 and Equation 1, are shown in Figure 6. Not surprisingly, with the maximums of both height and period (of the maximum wave) occurring around 12 mph, there is a pronounced peak in energy at or around this speed. The general trends seen in Figures 5 and 6 were evident in the results for all boats tested by AMC.

There are three key features that can be observed from this study: (1) the relative differences in the maximum wave height, period and energy between the different water sport activities; (2) how much these alter (reduce) with increasing lateral distance, and (3) comparing the full-scale experimental data from three similar but independent studies (AMC, WSIA, SAFL). This comparison commences with Figure 7, where maximum wave height measured from all cases listed in Table 1 are presented as a function of lateral distance from the track path of the test vessels. Importantly, the data from each of the three studies are presented as separate data series for the water sport categories of Wake Boats (all at speeds around 10-12 mph); All Craft (20-24 mph); and Ski Boats (30-32 mph).

There is a lot of data in Figure 7, so the following key is designed to assist identification of each category: All data symbols from each of the three studies at wakesurfing speeds (10-12 mph) have a red central circle, while the outer ring for each study is assigned a different colour. Similarly, all craft at wakeboarding/tubing speeds (20-24 mph) have a green central triangle and the same colour for each of the three studies has been applied to the outer border. The symbol for the two ski boats travelling at the highest speed zone (30-32 mph) is a blue square (both are from the AMC study, Cases #13 and #14).

There is a stark difference between the highest waves, which are predominantly from wake boats at wakesurfing speeds (10-12 mph), and the lowest wave heights, from the ski boats at waterskiing speeds (30-32 mph). The mix of all craft (wake, ski, fishing) operating at the intermediate wakeboarding/tubing speeds (20-24 mph) generally lie between the wakesurfing and waterskiing categories. This trend generally applies at all lateral distances presented. It is worth noting that the WSIA data for the wake boat case that was run in shallow water (Case #6) lie on the lower bound of all wake boats, which is to be expected, as per the earlier discussion about the effect of shallow water and the results presented in Figure 2. Interestingly, the data for the WSIA wake boat case that was run in deep water (Case #7) lie on the upper bound of all wake boats. It is hypothesised that the WSIA deep water data may be the most representative of ‘true’ wake boat waves, assuming they engaged more experienced practitioner/s to tune the operation of their boat to produce a ‘cleaner’ unbroken wave for wakesurfing, as discussed in Section 3.1.

As previously mentioned, the WSIA or SAFL reports did not present data for the period of the maximum wave, so (almost) all wave period data presented in Figure 8 are from the AMC study. As for wave height, wave period data are distinguished for each speed category. As expected, wave period is largely unaffected by lateral distance. Ruprecht et al. (2015) presented limited wave period data for wakesurfing and wakeboarding boats which have been added to Figure 8 (listed as WRL for Water Research Laboratory). There may only be a few data points from WRL, but they are a close match to the AMC data, helping to validate these measurements. Importantly, there is a small but clear difference in the average period at each of the three key speed zones, which also agrees with the data presented in Figure 5.

As covered in Section 2, the present study prefers to compare the energy in the maximum wave (rather than total energy in the wave train), which requires both the wave height and period to be known. The average values obtained from the AMC wave period data, as stated in Figure 8, have been used with the wave height data from the WSIA and SAFL studies to calculate indicative values of energy of their maximum waves. This is plotted in Figure 9 adopting the same categories as used for wave height in Figure 7 with similar trends visible to those for wave height but here the differences are even more pronounced.

Both the WSIA and SAFL studies conducted runs where they had wave sensors close to the boat, where the lateral distance between the track path and sensor was approximately 6-14 feet. The maximum wave heights measured by these sensors ranged between 26 and 39 inches. The y-axis in Figure 7 was limited to 22 inches to improve clarity of the more relevant data at greater lateral distances. The SAFL study also measured some cases at greater distances around 600 feet (not included) where wave heights continued the declining trend. Similarly, for clarity the plot of energy data presented in Figure 9 is limited to 550 lb.ft/ft. There are four WSIA and SAFL measurements close to the boat (6-14 ft) that significantly exceed this, ranging from 850-1900 lb.ft/ft.

4.2	Benchmarks

In the original AMC Report, potential benchmarks were included in the plots for maximum wave height and energy of the maximum wave (Figures 10 and 11 in Macfarlane 2018). These values, ~7.2 inches and 43 lb.ft/ft respectively, were representative of the highest values measured at a distance of 100 ft for a 20 foot runabout/ski boat operating at wakeboarding/tubing speeds (22-24 mph), which for the local sheltered waterway was considered typical of accepted practice. These example benchmarks were included to help identify the approximate lateral distance that the waves generated by wake boats may be considered equivalent to these more ‘accepted’ water sport activities.

A similar exercise can be performed with the more comprehensive collection of relevant wave data presented in this paper. This is attempted using a slightly different approach in Figure 10, where the maximum wave height data presented in Figure 7 is repeated but with three changes:

- The data is grouped into just the speed categories (with no designation of the study or origin): all wake boats at 10-12 mph; all craft at 20-24 mph; and all ski boats at 30-32 mph,

- Power trendlines are fitted for each of these three speed zones,

- Data for the single ‘shallow’ water case are removed (Case #6, WSIA).

Similar has been done with the energy of the maximum wave data (from Figure 9) and presented in Figure 11. The limited WSIA and SAFL data very close to the boat (y = 6-14 ft) has effectively been removed from these trendlines so more weight is placed on the more relevant and distant data (80 < y < 500 ft).

Using Figures 10 and 11, an example benchmark could be defined as the value at which the trendline for the wakeboarding cases (all craft at 20-24 mph) cuts the lateral distance of 100 feet. These ‘benchmark’ values would be approximately 9 inches for wave height and 70 lb.ft/ft for energy of the maximum wave. The corresponding lateral distance that the trendline for all wake boats at wakesurfing speeds (10-12 mph) draws approximately equivalent to these ‘benchmark’ values is around 350-400 feet for wave height and approaches 500 feet for energy of the maximum wave. For this example benchmark, the WSIA recommended distance of 200 feet from the shoreline, docks or other structures would be inadequate.

4.3	Wind Waves

An alternative benchmark occasionally adopted is to compare boat-generated waves against wind-generated waves. On the surface, using local wind waves as a benchmark may make good sense as each shoreline is presumably in a state of dynamic equilibrium (ie. no substantial erosion or changes) based on the prevailing weather conditions. However, typical winding rivers naturally have very limited fetch, plus the local topography and vegetation will attenuate wind speed. As a result, it is likely that the equilibrium of very sheltered riverbanks is not impacted by wind waves much, if at all, and it may be that other riverine processes are the dominant causes of change (Cox, 2020). In the case of the Willamette River this may be more due to seasonal flows or floods rather than wind.

The WSIA acknowledges that the wind wave predictions presented in their report are applicable to more exposed waterways than typical narrow, winding rivers (often used for wakesurfing) where the naturally occurring wind wave climate is low due to significantly reduced fetch. The shortest fetch WSIA considered is 1 mile: equivalent to 5,280 feet or ~1,600 metres (Table 6, Goudey and Girod, 2015). To put this into perspective, the test section for the Willamette River (AMC) trials was approximately 1,000 ft wide by 3,000 ft long, meaning the wind direction would have to be in close alignment with significantly longer section of river for a lengthy duration to generate waves of similar energy to those given in the WSIA assessment. It is worth noting that the largest wind waves generated would expend much of their energy at river bends, not straight stretches.

4.4	Effect of Ballast on Wakesurfing Waves

Additional data presented in the SAFL report investigated both ballasted and non-ballasted conditions for two wake boats, where the results suggest the additional ballast had little effect on either the maximum wave height or total wave energy at lateral distances greater than 100 or 200 ft (refer Figures 31-36, Marr et al. 2022). This finding is unexpected as it is well known that increased displacement will lead to increased wave height/energy (see for example Macfarlane 2012). There may be several explanations for this result. One may be experimental uncertainty, which can affect all similar studies including those by WSIA and AMC – this is covered in more detail in Section 4.5. Another explanation relates to the steepness of the waves created. It is well understood that wake boats are specifically designed to significantly increase their displacement as this will increase the size of the surfing wave close to the boat (for speeds around ~10-12 mph). However, the added displacement has negligible effect on the period of this wave, thus increasing the wave steepness. When the steeper waves break, ideally just beyond the desired ‘surf zone’, they will likely lose considerably energy. It is hypothesized that the steeper ‘well-tuned’ surfing waves generated by a heavily ballasted wake boat will lose a greater level of energy through wave breaking than a lighter unballasted wake boat.

There are two data points presented in the SAFL report that provide provisional support for this hypothesis. They acquired measurements for both ballasted and non-ballasted wake boats at a distance of 5 ft from the boat and found in one case the maximum wave height was 34 inches when ballasted and 27 inches non-ballasted. The second case found a similar result with 39 and 31 inches, respectively. In both these cases there is a notable increase in maximum wave height for the ballasted boat, compared to unballasted, which is not the case with the data at greater lateral distances. The primary focus of all the WSIA, SAFL and AMC studies are on the wave characteristics at notably greater distances from the boat track, thus further research involving measurements close to the wakeboat is required to confirm this finding.

4.5	Experimental Uncertainty

As is the case for all insitu full-scale experiments, a level of scatter is present in all trials data presented in this paper. For the AMC study, considerable effort was placed on maintaining consistency during the conduct of the trials. However, the large number of variables and practical realities ensure that there will always be a greater degree of scatter in results from experiments conducted in any uncontrolled environment compared to those performed in the controlled environment provided by specialist hydrodynamic facilities, such as the AMC Model Test Basin (where the data in Figure 2 was acquired).

Guidelines for the conduct of full-scale wave wake experiments provided by Macfarlane (2002) are designed to minimise uncertainty such that reliable conclusions can be drawn from the data obtained. Included is an extensive list of variables and/or external influences which should be considered when planning and undertaking such experimental campaigns. Each of the following factors are known to adversely affect the measurements, so efforts were made to minimise them as much as practical for the experiments at the Willamette River site:

- method/s used to measure the speed of the test vessel and for it to remain as constant as practical;

- vessel track path to remain straight and at the desired lateral distance;

- minimise ambient conditions, such as wind waves by picking suitable weather window/s;

- selection of a site where the water depth is considered ‘deep’ (and approximately constant) at all lateral distances;

- use of reliable and calibrated wave sensors that are rigidly mounted in sufficiently deep water.

From the author’s experience, the factor that possibly created the largest ‘uncertainty’ in this specific study was the likelihood that the WEDs may have been insufficiently fine-tuned to achieve optimum ‘maximum’ waves during some of the wakesurfing trials, as previously noted in Section 3.1. This could potentially mean that the AMC data for wakesurfing cases is conservative (lower than what is possible).

The accuracy of the measurements of both wave height and period is estimated to be within +/-5%. This does not entirely account for variations in vessel speed, water depth, lateral distance or environmental influences such as wind waves and currents, all of which may result in increased data scatter. It is estimated that the combined effect of measurement accuracy and these uncontrolled sources of potential error may account for up to 10% variation in the experimental results. It is not the aim of studies such as this to identify the effect from small systematic changes, but to obtain a statistically robust number of data points where there are more significant differences in key variables, from which good engineering approximations can be obtained. These can then lead to the proposal of rational and justifiable management or regulatory decisions. It is believed that this study has achieved the key first step of this aim.

# Conclusions

A comparison of full-scale trials results from three separate studies have confirmed that wake boats, either ballasted or unballasted, when operating around 10-12 mph generally generate significantly higher and more energetic waves that ‘conventional’ recreational craft operating within the higher speed zone of 20 mph and above.

The data presented may assist in defining rational management options for waterways where the waves generated by sporting activities such as wakesurfing may be causing issues related to shoreline erosion, damage to maritime structures and/or presenting a danger to other users of the waterway.

The leading advocate for the towed water sports industry in the USA, the Water Sports Industry Association (WSIA), recommend wakesurfers stay at least 200 feet away from the shoreline, docks, or other structures. The appropriateness of this distance can be examined using the presented data from multiple locations from the sailing line of the test boats and compared against a suitable benchmark. It is likely that some rational benchmarks may recommend a greater distance, likely to be in excess of 350 feet.

# Acknowledgements

The full-scale experiments on the Willamette River, Oregon, were performed in collaboration with the Oregon River Safety and Preservation Alliance (ORSPA). A special mention is deserving for the tireless efforts of Steve Gregg from ORSPA. The author acknowledges the support of the Australian Research Council via Linkage Project LP150100502, which supported the development of wave measurement equipment. The experimental data on the effect of water depth on the waves from wakesurfing activities presented in Figure 2 were acquired by Robert Buchanan as part of his Honours thesis for his BEng degree in naval architecture, under the supervision of the author. Dr Gregory Cox reviewed a near-complete draft and provided valuable input.

Data Availability Statement

The data that support the findings of this study are available from the author upon reasonable request.

ORCID  https://orcid.org/0000-0002-0518-430X

# References

Cox, G.L. (2020) Vessel wave wakes: new perspectives on their generation, propagation and shoreline impacts', PhD thesis, Australian Maritime College, University of Tasmania.

Cox, G.L. and Macfarlane, G.J. (2019) The effects of boat waves on sheltered waterways – thirty years of continuous study, Proceedings of the Australasian Coasts and Ports 2019, Hobart, Australia, 10-13 September 2019.

Fay, E., Gunderson, A. and Anderson, A. (2022) Numerical Study of the Impact of Wake Surfing on Inland Bodies of Water, Journal of Water Resource and Protection, 14, 238-272, Work supported financially by the National Marine Manufacturers Association (NMMA). https://doi.org/10.4236/jwarp.2022.143012

Fenton, J.D, Buber, B., Klasz, G. and Krouzecky, N. (2023) Ship waves in rivers: Environmental criteria and analysis methods for measurements, J River Research and Applications, Volume 39, Issue 4, First published: 11 January 2023 https://doi.org/10.1002/rra.4101

Fleit, G., Baranya, S., Krámer, T., Bihs, H. and Józsa, J. (2019) A practical framework to assess the hydrodynamic impact of ship waves on river banks, River Research and Applications, First published: 16 August 2019 https://doi.org/10.1002/rra.3522

Goudey, C.A. and Girod, L.G. (2015) Characterization of Wake-Sport Wakes and their Potential Impact on Shorelines, Report prepared for Water Sport Industry Association (WSIA), 24 November 2015, https://www.wsia.net/wp-content/uploads/2020/03/WSIA_draft_report_Rev_II.pdf

Houser C., Smith A. and Lilly J. 2021. Relative importance of recreational boat wakes on an inland lake. Lake and Reservoir Management, 37:227–234, https://doi.org/10.1080/10402381.2021.1879325

Houser, C., Carswell, A., Chittle, B., Johnson, G., Caschera, A. and Smith, A. (2024) Relative energy and perceived impact of vessel-generated waves in fetch-limited environments, Lake and Reservoir Management, 40:3, 285-302, https://doi.org/10.1080/10402381.2024.2370818

Lighthill, J., Sir (2001) Waves in fluids, Cambridge, UK; New York: Cambridge University Press.

Macfarlane, G.J. (2002) The Measurement and Assessment of Sub-Critical Vessel Generated Waves, MPhil thesis, Australian Maritime College.

Macfarlane, G.J. (2012) Marine vessel wave wake: Focus on vessel operations within sheltered waterways, PhD thesis, Australian Maritime College, University of Tasmania.

Macfarlane, G.J. (2018) Wave Wake Study: HB4099 Motorboat Working Group, unpublished technical report prepared for the Oregon River Safety and Preservation Alliance, Report 18WW01, 28 August 2018 https://static1.squarespace.com/static/5a0ba0f9e5dd5bce46ef4ed2/t/5c01dec34d7a9cb0b6f25937/1543626456377/AMC+Wave+Wake+Study_HB4099+Motorboat+Working+Group+REPORT.pdf

Macfarlane, G.J. and Cox, G.L. (2003). Vessel Wash Impacts on Bank Erosion – Noosa River Between Lake Cootharaba and Lake Cooroibah – Brisbane River Kookaburra Park to the Bremer River Junction. Report Number 01/G/18. AMC Search Limited, Launceston.

Macfarlane, G.J., Cox, G.L. and Bradbury, J. (2008) Bank erosion from small craft wave wake in sheltered waterways’, RINA Transactions, Part B: Intl. Journal of Small Craft Technology, 150. 33-49 http://dx.doi.org/10.3940/rina.ijsct.2008.b2.85

Malibu Boats, 2024, https://www.malibuboats.com/what-makes-a-great-wakesurf-boat, accessed 19 November 2024.

Marr, J., Riesgraf, A., Herb, W., Lueker, M., Kozarek, J. and Hill, K. (2022) A field study of maximum wave height, total wave energy, and maximum wave power produced by four recreational boats on an inland freshwater lake, St. Anthony Falls Laboratory, SAFL Project Report No. 600, University of Minnesota, February 2022. https://hdl.handle.net/11299/226190; https://conservancy.umn.edu/items/99d10d82-eada-46ec-83ec-d97f8953d326

Maynord, S.T., Biedenharn, D.S., Fischenich, C.J. and Zufelt, J.E. (2008). Boat-wave-induced bank erosion on the Kenai River, Alaska. Engineer Research and Development Center TR-08-05, US Army Corps of Engineers, Vicksburg.

Nanson, G.C., Von Krusenstierna, A., Bryant, E.A. and Renilson, M.R. (1994). Experimental Measurements of River Bank Erosion caused by Boat-Generated Waves on the Gordon River, Tasmania, Regulated Rivers, Research and Management Vol. 9. 1-14. https://doi.org/10.1002/rrr.3450090102

Nautique Boats, https://nautique.com/models/nautique-surf-system, accessed 19 November 2024.

Novak P.A., Fairfield C.A., Miloshis M., Knight Z.C., Lindsay R. and King A.J. (2021) Bank erosion in a macrotidal tropical river: Exploring the relative impact of boat wash on riverbank erosion. River Res Applic. 2021;37:3–16. https://doi.org/10.1002/rra.3736

Oregon River Safety & Preservation Alliance (ORSPA), https://www.orspa.org/, accessed 22 November 2024.

Ruprecht, J.E., Glamore, W.C., Coghlan, I.R. and Flocard, F. (2015) Wakesurfing: Some Wakes are More Equal than Others, Australasian Coasts & Ports Conference 2015, Auckland, New Zealand, 15-18 September 2015.

Water Sport Industry Association (WSIA), “Wake Responsibly”, https://www.wsia.net/wake-responsibly/, accessed 18 January 2023.

| Case No. | Boat Category | Nominal Speed Range |  |  | Boat Description | Length Overall (ft) | Ballast Condition and/or Wake Enhancing Devices | Total Displacement (lbs) | Nominal Water Depth (ft) | Data Origin |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10-12 (mph) | 20-24 (mph) | 30-32 (mph) |  |  |  |  |  |  |
| 1 | Wake | Yes | Yes |  | 2006 Malibu V-Ride | 21' | Ballast | 5,520 | 40 | AMC |
| 2 | Wake | Yes | Yes |  | 2014 Nautique G21 | 21'6" | Ballast | 9,970 | 40 | AMC |
| 3 | Wake | Yes | Yes | Yes | 2014 Nautique G21 | 21'6" | No ballast | 7,120 | 40 | AMC |
| 4 | Wake | Yes | Yes |  | 2018 Axis T23 | 23'6" | Ballast | 7,020 | 40 | AMC |
| 5 | Wake | Yes | Yes |  | 2018 Axis T23 | 23'6" | No ballast | 6,120 | 40 | AMC |
| 6 | Wake | Yes |  |  | 2015 Nautique G23 | 23' | Ballast | 10,150 | 8-10 | WSIA |
| 7 | Wake | Yes |  |  | 2015 Nautique G23 | 23' | Ballast | 10,150 | 22-30 | WSIA |
| 8 | Wake | Yes |  |  | 2019 Malibu Wakesetter VLX | 21' | Ballast, Hydrofoil, Wake shaper | 8,630 | 15-30 | SAFL |
| 9 | Wake |  | Yes |  | 2019 Malibu Wakesetter VLX | 21' | Hydrofoil | 4,940 | 15-30 | SAFL |
| 10 | Wake | Yes |  |  | 2019 Malibu Wakesetter MXZ | 24'6" | Ballast, Hydrofoil, Wake shaper | 11,125 | 15-30 | SAFL |
| 11 | Wake |  | Yes |  | 2019 Malibu Wakesetter MXZ | 24'6" | Hydrofoil | 6,240 | 15-30 | SAFL |
| 12 | Fishing | Yes | Yes | Yes | 2004 Thunder Jet Alexis | 21' | N/A | 5,180 | 40 | AMC |
| 13 | Ski | Yes |  | Yes | 2015 Ski Nautique 200 | 20' | N/A | 3,930 | 40 | AMC |
| 14 | Ski | Yes | Yes | Yes | 2008 Reinell Ski Boat | 20' | N/A | 3,980 | 40 | AMC |
| 15 | Ski | Yes | Yes |  | 2004 Larson LXI 210 | 21' | N/A | 3,255 | 15-30 | SAFL |
| 16 | Ski | Yes |  |  | 2004 Malibu Response LX | 20' | Hydrofoil; Wake shaper | 2,780 | 15-30 | SAFL |
| 17 | Ski |  | Yes |  | 2004 Malibu Response LX | 20' | N/A | 2,780 | 15-30 | SAFL |

Table 1: Summary of test vessels and details. The eight grey shaded cases are from the test program performed by the author (AMC) on the Willamette River, 2018. Data from the AMC study are compared to data extracted from the WSIA and SAFL reports (unshaded cases). The green shaded cases indicate which of the three nominal speed ranges are included in the comparisons compared graphically in this paper.

Figure 1: Wakesurfing in the enhanced and tuned wave a short distance aft of the transom of a typical wake boat. Note the asymmetry of the wake and the long, high, clean and steep wave face (Nautique Boats, 2024)

Figure 2: Time-series plots from experiments of the wake close to a 1:6 scale model of the MasterCraft XStar wake boat to investigate the effect of water depth. The largest and steepest wave of approximately 30” is found at the deepest water depth of 18 ft. Both the height and steepness degrade with decreasing water depth. The inset (bottom right) highlights the decrease in wave height with decreasing water depth.

Figure 3: Right - Location of the test site on the Willamette River. The water depth at the wave sensor was 15’ and approximately 40’ at each of the vessel track paths (100, 200, 300, 400’ from the wave sensor at y = 0 ft). Top left - wave sensor attached to rigid vertical pole. Bottom left - test vessel passing the wave sensor.

Figure 4: Typical wave elevation time series records from four individual runs for 2014 Nautique G21 with ballast at 10 mph (Case #2). The four records are for lateral distances of 100, 200, 300 and 400 ft. Note how dispersion leads to a reduction in wave height and an increase in the number of waves with increasing lateral distance. Note that the start/arrival times for each of the four records, relative to each other, are not displayed correctly (separated for clarity).

Figure 5: Maximum wave height and period as a function of vessel speed for two wake boats: 2006 Malibu V-Ride (Case #1) and 2014 Nautique G21 (Case #2) at a lateral distance of 100 ft. Note that the peak wave height occurs around 10-12 mph and peak of the wave period around 12-14 mph.

Figure 6: Energy of the maximum wave as a function of vessel speed for two wake boats: 2006 Malibu V-Ride (Case #1) and 2014 Nautique G21 (Case #2) at a lateral distance of 100 ft. Note that the peak wave energy occurs around 12 mph    and peak of the wave period around 12-14 mph.

Figure 7: Maximum wave height as a function of lateral distance for each of the three nominal speed ranges: 10-12 mph (red), 20-24 mph (green) and 30-32 mph (blue). Where appropriate, the data have been sub-categorised into each of the three studies (AMC, WSIA, SAFL).

Figure 8: Period of the maximum wave as a function of lateral distance for each of the three nominal speed ranges: 10-12 mph (red), 20-24 mph (green) and 30-32 mph (blue). Also shown are the average values for each speed range, which were used to estimate the energy in the maximum waves for the WSIA and SAFL studies. A few additional points from WRL (Ruprecht et al., 2015) are added to help validate the AMC data.

Figure 9: Energy of the maximum wave as a function of lateral distance for each of the three nominal speed ranges: 10-12 mph (red), 20-24 mph (green) and 30-32 mph (blue). Where appropriate, the data have been sub-categorised into each of the three studies (AMC, WSIA, SAFL).

Figure 10: Maximum wave height as a function of lateral distance for each of the three nominal speed ranges (with no study designation): 10-12 mph (red), 20-24 mph (green) and 30-32 mph (blue). Power trendlines are included to assist interpretation: wakesurfing (10-12 mph) wave height data are approximately equivalent to wakeboarding data at a lateral distance of 100 ft (20-24 mph) between a lateral distance of 350 and 400 ft.

Figure 11: Energy of the maximum wave as a function of lateral distance for each of the three nominal speed ranges (with no study designation): 10-12 mph (red), 20-24 mph (green) and 30-32 mph (blue). Power trendlines are included to assist interpretation: wakesurfing (10-12 mph) wave energy data are approximately equivalent to wakeboarding data at a lateral distance of 100 ft (20-24 mph) approaches a lateral distance of 500 ft.